# 3GPP TSG-RAN WG4 Meeting #112	R4-2412531

Maastricht, NETHERLANDS, 19th – 23th August, 2024

Agenda Item:	8.22.4.2

Source: 	Samsung

Title: 	RRM impact for Rel-19 LP-WUS/WUR

Document for:	Discussion

- Introduction

During RAN#103 meeting, a revised WID[1] was approved on low-power wake-up signal and receiver for NR (LP-WUS/WUR). In RAN4#111 meeting, RAN4 achieved some agreements in WF[2].

Issue 1-1-2: Cases/states to be considered for RRM relaxation Agreement: Discuss the RAN4 requirements first for the following case #1, and FFS for case #2 to #5. RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #1 Fully offloading case; Off; Off: FFS the condition and the details; ON. RAN4 to further discuss case #2 to #4: RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #2 Relaxed case a; On with relaxation measurement; Off; ON. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON. #4 Relaxed case c; Off; On, FFS the condition and the details; ON. - Sub-topic 1-4 Simulation work and assumptions Issue 1-4-2: LP-SS periodicity for evaluation Agreement: - As starting point for RAN4 simulation purpose, uses 320 ms as the LP-SS periodicity in IDLE/Inactive mode, and other values based on RAN1/2 discussion are not precluded. - Not consider the MR DRX configuration in the simulation. Issue 1-4-5: Simulation assumptions Agreement: First start the simulation for FR1, and FFS for FR2 pending on the RF progress. Recommendations: Companies are encouraged to provide simulation assumptions based on framework in the appendix.

In this contribution, our views on LP-WUS/WUR and related analysis of RRM requirements are provided.

- Discussion

  - General aspect

Issue 1-1-2: Cases/states to be considered for RRM relaxation

In last RAN4 meeting, RAN4 agreed at least case 1 (full offloading case) need to considered for specify RRM relaxtion requirements.

Several aspects need to be considered together including LR measurement performance, MR serving cell measurement relaxation, MR neighbouring cell measurement relaxation and corresponding side condition. In our view, RAN4 shall further discuss the applicable side condition /scenario with corresponding assumption on MR serving cell measurement, MR neighbouring cell measurement and LP-WUR measurement as a package. In our view, several states may be considered for MR RRM relaxation and LP-WUR RRM measurement which as summarized in Table 1 as example for further discussion (RRM measurement with high priority can be considered separately):

Table 1 States of MR RRM relaxation and LP-WUR RRM measurement

| RRM measurement state index | MR serving cell measurement | MR neighbouring cell measurement | LR measurement | Applicable scenario and side condition |
| --- | --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off | ON | - MR enters into ultra-deep sleep state w/o paging monitoring and RRM measurement - LP-WUR enable for LP-WUS monitoring and serving cell RRM measurement. - LP-WUR can maintain good measurement performance and UE placed in good condition i.e., cell centre |
| #2 Relaxed case a | On with relaxation factor >8 | Off | ON | - MR still needs to perform RRM measurement with longer period - LP-WUS monitoring and partial RRM measurement rely on LP-WUR. LP-WUR can maintain good measurement performance with assistant measurement from MR. - UE placed in good condition w/o triggering neighbouring cell measurement |
| #3 Relaxed case b | On with relaxation factor >8 | On with relaxation factor >3 | On | - MR still needs to perform RRM measurement on serving cell and neighboring cell - LP-WUR can monitor LP-WUS, and maintain good measurement performance with assistant measurement from MR - UE with good condition e.g., fulfil existing Relaxed measurement criterion including low-mobility and not-at-cell edge |
| # Fall back to normal mode | On | On | Off | - MR follows normal RRM measurement - Exit from LP-WUS RRM measurement - Relationship with LP-WUS monitoring exit/entrance condition |

Proposal 1: In additional to case 1, RAN4 shall consider case 2 (relaxed case a) and case 3 (relaxed case b) at least.

- RRM measurement with high priority can be considered separately

Issue 1-1-3: Core requirements to be specified for MR RRM relaxation

According to WID[1], RAN4 needs to specify MR serving cell RRM measurement relaxation in RRC_IDLE/INACTIVE state.  Related objective is provided as follows:

Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

For MR in RRC_IDLE/INACTIVE state, MR is required to wake-up to perform serving cell measurement at least per one or two DRX cycles by current requirements. There is observation [3] that power saving gain can be achieved, if sufficient MR RRM measurement relaxation in time domain is applied e.g., relaxation factor larger than 8. According to the observation, OOK-based LP-WUS is more robust to frequency error than OFDM-based signal. So, in our view, there can be a room for serving cell relaxation in case LP-WUR is enabled, since time/frequency tolerance of LP-WUS is higher than the existing signals. Therefore, first we think RAN4 needs to specify whether serving cell measurement relaxation is feasible in case LP-WUR is enabled. Then, we can discuss whether to introduce serving cell measurement relaxation.

For intra, inter, and inter-RAT cell measurement relaxation by MR, we think existing RRM measurement requirement relaxation under low mobility or not-at-cell-edge scenario can be considered as starting point. For further relaxation, we think it is beneficial for reducing power consumption, however, unless clear scenario is defined, there can be negative impact on proper cell- reselection. Therefore, it needs further evaluation whether it is feasible.

Proposal 2: RAN4 needs to further evaluate serving cell measurement relaxation and neighbouring cell measurement relaxation by MR

Issue 1-1-5: Criteria (entry/exit conditions) for LP-WUR measurement / Issue 1-1-6: Criteria (entry/exit conditions) for MR RRM measurement relaxation

Regarding the criteria for LP-WUR measurement and MR RRM measurement relaxation, we think RAN4 needs to discuss the criteria related to measurement. Although RAN2 is the main group for design of criteria, traditionally RAN4 was involved in the designing criteria related to measurement. Otherwise, RAN2 may not find technical guidance for determining the criterion. Therefore, we think that RAN4 shall lead the discussion regarding criterion or scenario (entry/exit condition) for measurement.

##### RRM measurement relaxation and offloading in RRC_IDLE INACTIVE RAN2#126 For serving cell measurement offloading (i.e., serving cell measurement fully offloaded to LR and no serving cell measurement via MR is required), RAN2 should focus on specifying the offloading criterion for serving cell for UEs supporting LP-WUS, and assume that RAN4 will define the measurement offloading requirements for serving cell. RAN2 understand that the RRM measurement of the neighboring cell can only be performed by MR. Can discuss again if RAN1 inform us otherwise. RAN2 will further discuss the neighbor cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’.

Proposal 3: RAN4 shall be involved on criterion and scenario (entry/exit condition) design for LP-WUR measurement and MR RRM measurement relaxation together with  RAN1 and RAN2.

- RAN4 shall further discuss the applicable side condition /scenario with corresponding assumption on MR serving cell measurement, MR neighbouring cell measurement and LP-WUR measurement as a package

Issue 1-1-7: Criteria (entry/exit conditions) for LP-WUS monitoring

follows:

Working Assumption (RAN1#116bis) From RAN1 perspective, for the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode, - The UE may start LP-WUS monitoring if - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB, and/or - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied - If UE starts LP-WUS monitoring, it may stop the legacy PO monitoring before UE receives LP-WUS indicating wake-up - The UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB, and/or - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied - FFS the serving cell measurement metrics - The entry/exit thresholds can be configured separately for different types of LR - It is left to RAN2 discussion whether the threshold(s) are always configured by the gNB. - Note: This may be revisited based on the RAN2/RAN4 discussion.

RAN2#126 RAN2 will further discuss the details about LP-WUS monitoring entry/exit conditions based on RAN1’s existing working assumptions. The LP-WUS related configuration in SIB at least include the following information for IDLE/INACTIVE: - - LP-SS configuration - - LP-WUS configuration - - Entry/exit condition for LP-WUS monitoring (FFS if it always configured) Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed. Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS. RAN2 understand that if UE is configured with CN-based LP-WUS subgrouping, it is up to CN to assign the LP-WUS subgroup ID to the UE. RAN2 assume the maximum number of subgroups that can be configured for LP-WUS subgrouping is no less than 8. From RAN2 perspective, no new procedure is introduced for SI reception/updates.

RAN1’s view for the entry condition is that the signal quality of LP-WUS can be evaluated by MR through comparing the serving cell measurement and entry thresholds. The exit condition involves comparing the serving cell measurement performed by the LP-WUR and exit threshold, at least for the device supporting LP-WUR measurement. In general, above conditions are similar to the method of existing threshold comparison methods, which can be determined based on pre-configured criteria with SS-RSRP/SS-RSRQ. For MR, such metrics are sufficient for the threshold; however, for LP-WUS monitoring by LP-WUR, LP-WUR shall detect LP-WUS and wake up MR for paging when LP-WUS indicates “on” state. UE needs to guarantee LP-WUS can be reliably received and detected by LP-WUR with corresponding target miss detection rate on LP-WUS. A metric similar to RLM can be considered as shown in Table 2.

Table 2 Miss detection rate on LP-WUS

| Miss_erro_rate_Entrance | Miss_erro_rate_Exit |
| --- | --- |
| Th1 i.e. 1% | Th2 i.e. 10% |

Proposal 4:  For Entry/exit condition for LP-WUS monitoring, the performance of miss detection rate on LP-WUS can be considered as side condition in addition to RSRP and RSRQ

Furthermore, another issue needs to be further discussed on entry/exit condition is that depending on UE architecture, MR and LP-WUR measurement performance can be different i.e., antenna gain, IL and NF. Calibration between LP-WUR measurement and MR measurement required to have unified exit and entrance condition. In order to define such unified condition, it is worth noting that the RSRP measurement results difference between MR and LP-WUR should be stable and comparable. One solution is to consider the antenna sharing architecture between LP-WUR and MR to guarantee better accuracy for RRM measurement offloading that can be discussed in UE RF.

Observation 1: LP-WUS entrance condition is specified based on MR measurement and exit condition is specified base LR measurement.  There is potential mismatch between MR and LR measurement pending on UE architecture.

Proposal 5: Calibration between MR measurement and LR measurement maybe required which also pending on the progress of UE RF session on LP-WUR RF architecture, NF assumption.

  - Sub-topic 1-2 LP-WUR requirements at RRC_IDLE/INACTIVE state

Issue 1-2-1: Methodology on specifying LP-WUR RRM requirements at Idle/Inactive mode

Regarding target SNR to achieve coverage as PUSCH MSG3 following agreement reached in RAN1

RAN1#117 Agreement For evaluation of LP-WUS and LP-SS design to achieve coverage of PUSCH for message3 from RAN1 perspective, at least the following SNR values should be considered: - SNR=-3dB for NF of LR = NF of MR+ 8dB - SNR= -0.5dB for NF of LR = NF of MR+ 5dB - SNR=2dB for NF of LR = NF of MR+ 2dB - Note 1: The NF of MR is assumed as 7dB

In the evaluation work, measurement accuracy, target SNR and measurement samples are major factors which interacted with each other. RAN4 can follow RAN1 to fix target SNR first and then to evaluate suitable measurement period and measurement accuracy.

Proposal 6: Follow RAN1 conclusion on target SINR condition, and further evaluate measurement accuracy and measurement period.

- Target SINR for OOK based LR: -3dB

- Target SINR for OFDM based on LR:  -0.5dB and/or 2dB

Issue 1-2-4: Measurement metrics

RAN1#116bis Agreement From RAN1 perspective, at least the following metrics can be supported for RRM serving cell measurement performed by OOK-based receiver based on LP-SS: - LP-RSRP - LP-RSRP is the linear average of received power of LP-SS in OOK ON symbols. - FFS: How to determine the received power of LP-SS in OOK ON symbols - LP-RSRQ - LP-RSRQ = LP-RSRP/LP-RSSI - For the definition of LP-RSSI for determination of LP-RSRQ, further consider the following options: - Option 1: LP-RSSI is the linear average of total received power in all LP-SS OOK symbols. - Option 2: LP-RSSI is the linear average of total received power in LP-SS OOK OFF symbols. - Option 3: LP-RSSI is the linear average of total received power in LP-SS OOK ON symbols. - FFS: LP-SINR, Power ratio of OOK-ON symbol and OOK-OFF symbol Note: RAN1 will send an LS to RAN2 and RAN4 on the measurement metrics that can be supported from RAN1 perspective, to facilitate RAN2/RAN4 discussions. The exact metrics for OOK-based receiver to be used and defined in the specifications depend on the outcome of [RAN1]/RAN2/RAN4 discussions. Conclusion LP-SINR is not considered further as a metric for RRM serving cell measurement RAN1#117 Agreement For LP-SS based LP-RSRQ, LP-RSRP and LP-RSSI are measured within the same bandwidth. Agreement For LP-RSSI definition for LP-RSRQ, the following is agreed - Option 1: LP-RSSI is the linear average of total received power in ON and OFF LP-SS OOK symbols. Note: Above does not constrain LP-SS sequence design for OOK Working Assumption From RAN1 perspective, for the RRM measurement metrics based on SSS for OFDM-based LP-WUR, use the same definition of SS-RSRP and SS-RSRQ for LP-SSS-RSRP and LP-SSS-RSRQ, respectively. - Above is applicable for both time-domain processing or frequency-domain processing - Above does not imply that RAN1 will introduce LP-SSS-RSRP and LP-SSS-RSRQ in the specifications

Based on above RAN1 design on RRM measurement metric for LP-WUR, we can conclude that.

Proposal 7: RAN4 need to specify RRM measurement requirements following metrics:

- OOK based on LP-WUR: LP-RSRP and LP-RSRQ.

- OFDM based on LP-WUR: SS-RSRP and SS-RSRP

Issue 1-2-5: Time/frequency sync

RAN1#117 Agreement The LP-WUS and LP-SS design assumes the residual frequency error after frequency error correction without considering impact of drift, is up to X ppm for OOK-based LP-WUR. - FFS X which is no larger than 20ppm - Initial frequency error assumption: up to company report For the overlaid OFDM sequence design of LP-WUS, it is assumed that the residual frequency error for OFDM-based LP-WUR after frequency error correction without considering impact of drift is not larger than Y. - FFS Y which is no larger than 20ppm and lower than X - Initial frequency error assumption: up to company report

We suggest to consider 20ppm frequency error as worst case for RAN4 evaluation work.

Proposal 8: 20 ppm frequency error can be considered for OOK based on LR for initial RAN4 evaluation work.

Issue 1-2-7: Other considerations

Once LP-WUR detected LP-WUS indicating paging indication, then UE needs to wake up MR receiver and monitor paging on paging occasion.

The maximum interruption/processing time between UE being ready to receive Paging/PDCCH by MR and LP-WUS received “on” indication by LP-WUR consist of following factor (time offset between LP-WUS/paging indication to MR ready for Paging monitoring):

“LP-WUS detection processing time + MR wake-up time + MR sync time”

- MR wake-up time depending UE implementation can be 400 ms ~1s

- MR sync time pending on whether UE can utilize LR sync time/frequency information as starting point for MR i.e., if MR and LR sharing same of RF components e.g.  LO, and/or antenna then UE can utilize LR sync information for MR receiver:

  - For UE with shared architecture between LR and MR, LR sync info can be used

    - ~1~2 SSB required for MR sync time

  - Other wise

    - ~10# SSB required for MR sync time

RAN4 may need to specify the maximum interruption time between LP-WUS/LO to PO, which the UE expects to be ready for MR in Paging monitoring. And NW can configure PO offset to LO no less than above processing time/interruption time specified by RAN4.

Proposal 9: New requirements need to be specified for the processing/interruption time between LP-WUS received paging indication to MR ready for Paging monitoring.

  - Sub-topic 1-3 MR RRM relaxation

Issue 1-3-1: MR RRM relaxation for serving cell/neighbour cell

RAN2 is still on discussion for the neighbour cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’.

In R16, relaxation factor 8 was introduced in RAN4 for UE inter-frequency/intra-frequency measurement when UE meet ‘not at cell edge’ criteria and or ‘low mobility’.

According to LP-WUR SI discussion, LP-WUS can achieve power saving gain when relaxation factor larger than 8.

Proposal 10: Relaxation factors within the range from 8 to 16 as the starting point for the relaxation factor for the MR RRM relaxation.

  - Sub-topic 1-5 LP-WUR CONNECTED mode

As indicated in the WI objective:

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Check in RAN#105 for potential TU adjustment in RAN2 - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR

During CONNECTED mode, UE RRM/RLM/BFD are performed by MR, these requirements are not applicable for LP_WUR during RRC CONNECTED mode. And timing requirements are also not applicable given LP-WUR don’t support UL transmission or CA/DC.

Observation 2: No impact is observed   for RRC_CONNECTED state mobility, timing, signalling characteristic, and measurement procedure requirements.

The details of LP-WUS procedure to trigger PDCCH monitoring are still on discussion in RAN1 with several alternatives. Also the activation/deactivation of LP-WUS monitoring is still on discussion in RAN1 and RAN2.

Proposal 11: FFS whether need to introduce LP-WUS monitoring activation and deactivation delay requirements pending on RAN2/RAN1 progress.

Paging reception

When LP-WUR is enabled, LP-WUR can monitor LP-WUS to wake up MR for triggering PDCCH/paging monitoring. RAN1/2 discussion is still on going. It’s FFS whether exiting RAN4 requirements for “Maximum interruption in paging reception” can be reused or not.

Proposal 12: Paging reception requirement impact can be discussed after further input from RAN2 and RAN1.

- Conclusion

In this contribution, our view on RRM requirement is provided.

General aspect

Proposal 1: In additional to case 1, RAN4 shall consider case 2 (relaxed case a) and case 3 (relaxed case b) at least.

- RRM measurement with high priority can be considered separately

Proposal 2: RAN4 needs to further evaluate serving cell measurement relaxation and neighbouring cell measurement relaxation by MR

Proposal 3: RAN4 shall be involved on criterion and scenario (entry/exit condition) design for LP-WUR measurement and MR RRM measurement relaxation together with RAN1 and RAN2.

- RAN4 shall further discuss the applicable side condition /scenario with corresponding assumption on MR serving cell measurement, MR neighboring cell measurement and LP-WUR measurement as a package

Proposal 4:  For Entry/exit condition for LP-WUS monitoring, the performance of miss detection rate on LP-WUS can be considered as side condition in addition to RSRP and RSRQ.

Observation 1: LP-WUS entrance condition is specified based on MR measurement and exit condition is specified base LR measurement.  There is potential mismatch between MR and LR measurement pending on UE architecture.

Proposal 5: Calibration between MR measurement and LR measurement maybe required which also pending on the progress of UE RF session on LP-WUR RF architecture, NF assumption.

Sub-topic 1-2 LP-WUR requirements at RRC_IDLE/INACTIVE state

Proposal 6: Follow RAN1 conclusion on target SINR condition, and further evaluate measurement accuracy and measurement period.

- Target SINR for OOK based LR: -3dB

- Target SINR for OFDM based on LR:  -0.5dB and/or 2dB

Proposal 7: RAN4 need to specify RRM measurement requirements following metrics:

- OOK based on LP-WUR: LP-RSRP and LP-RSRQ.

- OFDM based on LP-WUR: SS-RSRP and SS-RSRP

Proposal 8: 20 ppm frequency error can be considered for OOK based on LR for initial RAN4 evaluation work.

Proposal 9: New requirements need to be specified for the processing/interruption time between LP-WUS received paging indication to MR ready for Paging monitoring.

Sub-topic 1-3 MR RRM relaxation

Proposal 10: Relaxation factors within the range from 8 to 16 as the starting point for the relaxation factor for the MR RRM relaxation.

Sub-topic 1-5 LP-WUR CONNECTED mode

Observation 2: No impact is observed   for RRC_CONNECTED state mobility, timing, signalling characteristic, and measurement procedure requirements.

Proposal 11: FFS whether need to introduce LP-WUS monitoring activation and deactivation delay requirements pending on RAN2/RAN1 progress.

Proposal 12: Paging reception requirement impact can be discussed after further input from RAN2 and RAN1.

# References

RP-240801 Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo, NTT DOCOMO, Ericsson, MediaTek, Samsung, Sony

R4-2410296	WF for RRM core requirements for LP-WUS/WUR, vivo

3GPP TR38.869: “Study on low-power wake up signal and receiver for NR”

R4-2406366, “Work plan on RRM for LP-WUR WI”, vivo