# 3GPP TSG-RAN WG4 Meeting #112	R4-2412532

Maastricht, NETHERLANDS, 19th – 23th August, 2024

Agenda Item:	8.8.4

Source: 	Samsung

Title: 	NTN testing for NGSO

Document for:	Discussion

1		Introduction

For Rel-19 Enhanced requirements and conductive test methodology for NR NTN and IoT NTN WI, NGSO testing is one of major objective with detailed WID as following:

NTN testing for NGSO - Specify the TE-emulated channel model with varying Doppler and delay shifts for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands - Match the satellite motion trajectory based on the ephemeris for NGSO (Non-Geostationary Orbit) scenarios - Inform RAN5 to assist specifying RF frequency error tests, if needed - Checking if the existing frequency error requirement, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE RRM performance test of uplink timing for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Checking if the existing RRM uplink timing requirements, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE demodulation performance requirement(s) of PDSCH for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Minimize the number of demodulation performance requirements - Specify the applicability of the tests under the TE-emulated channel model

In this contribution, we bring brief analysis on NGSO testing.

2		Discussion

Timeline

According to the WID, NTN testing consists of several parts:

- Introducing TE-emulated channel model with varying Doppler and delay shifts for NGSO

- Specify corresponding UE demodulation requirements with identified channel model

- Update RRM uplink timing test case for NGSO with identified channel model

- Update RAN5 RF frequency tests if needed based on identified channel model

Clearly UE demodulation requirements and update on UE frequency error test cases and RRM UL timing test cases are pending on channel model discussion status. Without sufficient progress on channel model, it’s hard to have concrete discussion on the impact to UE demodulation requirements and RF/RRM test cases.

Proposal 1: Focus on channel model discussion in initial stage for NGSO testing

- Once sufficient progress made for channel model introduction, RAN4 can start the discussion on the impact to UE demodulation, RF/RRM test cases

RF and RRM core requirements impact

Rel-17 RAN4 RF/RRM/Demod requirements are specified based on the assumption with UE pre-compensation on doppler shift and time delay. From RAN4 core and performance requirements perspective, these requirements can be applicable for both non-zero/zero doppler/delay conditions; and which also irrespective to constant or time varied delay/ Doppler condition which related to Satellite motion.

Observation 1: RAN4 Rel-17 IoT over NR and NR NTN requirements (RRM, RF and performance) are specified based on the assumption that UE shall be capable of  pre-compensation of doppler shift and time delay for Satellite access link; and requirements agonistic to test conditions of doppler and time delay due to Satellite motion.

As captured in the WID, RF and RRM core requirements shall be maintained regardless channel condition. Meanwhile introducing variable time/frequency to model NGSO motion trajectory will bring challenge for testing and UE to pass corresponding RF (frequency error) and UL timing RRM test cases.

Proposal 2: RAN4 shall consider additional TT and/or MU for RF (frequency error) and UL timing RRM test cases if variable channel model introduced without change on the existing core requirements.

NGSO channel modelling

In Rel-17 maintenance phase, companies raised the demand to verify UE behaviour of tracking/compensating frequency shift and time shift due to NGSO mobility.  Due to time limitation, RAN4 specified new test cases with fixed time shift and doppler shift (corresponding ranges were agreed in RAN4 and recommended to RAN5) in Rel-17 and it can be considered single snap shot pending on relative location between satellite and UE. This simplified test set-up has limitation which can’t model satellite motion trajectory with time varied doppler shift and time shift.

To model satellite motion trajectory with time varied doppler shift and time shift existing model from TR 38.811 5.3.4 can be considered as starting point with further simplifications and down selection on the parameters.

- UE mobility: fixed UE without mobility

- Target frequency range: 2GHz

- Satellite type:

-	LEO- 600 km

-	LEO- 1200 km

Proposal 3: Existing channel model from TR 38.811 5.3.4 can be considered as starting point with further simplifications and down selection on the parameters.

- UE mobility: fixed UE without mobility

- Target frequency range: 2GHz

- Satellite type:

-	LEO- 600 km

-	LEO- 1200 km

3		Conclusion

In this contribution, we bring brief analysis on NGSO testing.

Proposal 1: Focus on channel model discussion in initial stage for NGSO testing

- Once sufficient progress made for channel model introduction, RAN4 can start the discussion on the impact to UE demodulation, RF/RRM test cases

Proposal 2: RAN4 shall consider additional TT and/or MU for RF (frequency error) and UL timing RRM test cases if variable channel model introduced without change on the existing core requirements.

Proposal 3: Existing channel model from TR 38.811 5.3.4 can be considered as starting point with further simplifications and down selection on the parameters.

- UE mobility: fixed UE without mobility

- Target frequency range: 2GHz

- Satellite type:

-	LEO- 600 km

-	LEO- 1200 km

Reference

[1] RP-241281, Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN, Samsung

[2] TR 38.811