# 3GPP TSG-RAN WG4 Meeting #112	R4-2412533

Maastricht, NETHERLANDS, 19th – 23th August, 2024

Agenda Item:	8.19.3

Source: 	Samsung

Title: 	Initial view on Rel-19 Duplex Evo WI RRM impact

Document for:	Discussion

1		Introduction

Revised WID on Rel-19 Duplex Evo WI was approved in RAN#104 on RAN4 related objectives as copied below

The objectives are as follows: - For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Note: Without dedicated optimization for dynamic/flexible TDD. - Specify BS RF requirements for SBFD operation at gNB [RAN4] - Specify applicable RRM core requirements for CLI handling mechanisms [RAN4] - Specify other RRM core requirements for SBFD operation, if identified [RAN4]

In this contribution, initial view on RAN4 RRM impact was provided for Rel-19 Duplex Evo WI.

2		Overall RAN4 RRM impact

To enable SBFD operation in DL BWP, several enhancements on different areas are on discussion in RAN1 following the WID objectives as captured in above.

2.1 Semi-static indication of time/frequency location of SBFD-subband

RAN1 is introducing the semi-static indication of SBFD sub-band location of time/frequency domain on DL BWP part.

There is no dedicated RRM requirement required for above objective and RAN4. And for specific RRM requirements introduced in SBFD slots/symbols, RAN4 needs to follow the design from RAN1 for SBFD configuration and indication.

Proposal 1: No new RRM requirement required for objective “Semi-static indication of time/frequency location of SBFD-sub band”

2.2 UE transmission, reception and measurement behavior and procedures

This objective can be further categorized with following areas:

- RAN1  is on discussion to specify the procedure and UE behaviour for  DL reception and UL transmission scheduling  across SBFD symbols/slot and non-SBFD symbols/slot including FH and repetition schemes.

We didn’t observe any impact on RRM requirement for DL/UL transmission scheduling across SBFD symbols/slot and non-SBFD symbols/slot as this is major related to demodulation requirements from RAN4 perspective.

- RAN1 is on discussion to specify CSI-RS configuration and CSI reporting for SBFD symbols/slot and non-SBFD symbols/slot.

This part may have impact on RAN4 L1 RRM measurement requirements as separate CSI reports maybe configured for SBFD symbols and non-SBFD symbols.

- RAN1 is on discussion on collision handling.

RAN1 conclusion is to reuse existing collision handling principles i.e. principles i.e. SSB is prioritized over than configured/scheduled UL transmission. There is no RAN4 RRM impact foreseen on the collision handling.

- RAN1 is on discussion on separate UL PC and/or spatial relation for SBFD slots and non-SBFD slots e.g. reusing multi-TRP repetition scheme framework (detailed framework will be decided by August RAN1 meeting)

FFS on RAN4 RRM impact for UL PC and or spatial relation update e.g.,unified TCI state framework and /or UL spatial info related framework.

Proposal 2: For objective “UE transmission, reception and measurement behavior and procedures”, following potential RRM impact foreseen:

- L1 RRM measurement may need to be updated to following RAN1 update on CSI report configuration between SBFD slots and non-SBFD slots

- FFS on RAN4 RRM impact for UL PC and or spatial relation update e.g., unified TCI state framework and /or UL spatial info related framework.

2.3 SBFD random access operation

RAN1 is introducing new RO configurations and resource mapping for SBFD slots and no RAN2 RACH procedure update is foreseen.

Proposal 3: No RAN4 RRM requirements impact foreseen on “SBFD random access operation”, some update on RAN4 PRACH configuration maybe required pending on detailed RAN1 specification update.

2.4a gNB-to-gNB CLI handling

For gNB to gNB CLI schemes, it can be categorized as following:

- Spatial schemes (FR1 gNB Tx beam nulling. FR2 gNB beam paring)

- RAN1 specify the required information exchange between gNBs e.g., measurement resource configuration

- Power domain schemes

- RAN1 concluded: UL Tx power control-based schemes are not considered in the down-selection of gNB-to-gNB CLI handling and UE-to-UE CLI handling schemes.

- gNB-to-gNB CLI measurement

- RAN1 specify the required information exchange between gNB

- RAN1 is discussion to support non-transparent UL resource muting for gNB-to-gNB CLI measurement and/or interference covariance matrix measurement

- Time/frequency coordination

- RAN1 specify the required information exchange between gNBs e.g., time/frequency location

Above enhancement is major related to RAN3 information exchange between gNBs and there are no new gNB RRM measurement quantities introduced for gNB-to-gNB CLI measurement. Also there is no RRM impact for UL non-transparent muting which related to UE RF and UL demodulation.

Proposal 4: There is no RRM impact on gNB to gNB CLI handing.

2.4b UE-to-UE CLI handling

Similar as gNB side, UE to CLI schemes can be categorized as following:

- Spatial schemes for FR2

- RAN1 is on discussion to specify Rx beam configuration for UE-to-UE CLI measurement  with separate optional UE capability

- Power domain schemes

- UL Tx power control based schemes are not considered in the down-selection of gNB-to-gNB CLI handling and UE-to-UE CLI handling schemes

- UE-to-UE CLI measurement

- RAN1 is introducing L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework

- Measurement resource : Periodic, semi-persistent, or aperiodic i.e. SRS-RSRP resource or CLI-RSSI resources

- Measurement reporting: at least aperiodic reporting, new reporting quantities e.g. L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices; CSI framework update (timeline and priority rules)

- Measurement method still FFS with candidate options

- Time/frequency coordination

- RAN1 specify the required information exchange between gNBs e.g. time/frequency location

According to updated WID and above RAN1 status summary, clearly RAN4 need to specify L1 CLI measurement requirements. Existing R16 CLI measurement requirements and Rel-15 L1-RSRP measurement requirements can be considered as starting point.

Proposal 5: RAN4 need to specify L1 based CLI measurement requirements

- Leverage existing Rel-16 L3 CLI measurement requirements framework as much as possible with necessary update.

3  L1 CLI measurement

As summarized in section 2.4b, new L1 CLI measurement requirement need to be specified for SBFD UE to UE CLI operation.

In following section, we analyse the difference between R16 L3 CLI measurement and L1 CLI measurement which is on going in RAN1.

3.1 Side condition

In Rel-16 L3 CLI measurement, following side condition specified for L3-CLI-SRS-RSRP in TS 38.133 10.1.22

The SRS-RSRP measurement reported by the UE shall fulfil the accuracy requirements defined in Table 10.1.22.1.1-1 for FR1 and Table 10.1.22.1.1-2 for FR2, provided that the following conditions are met. The accuracy requirements in this clause are derived based on AWGN radio propagation conditions. - Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled. - Conditions for SRS-RSRP measurements are fulfilled according to Annex B.2.z for a corresponding Band for each relevant SRS resource configured for measurement. - The time difference between UE’s DL reference timing in the serving cell and SRS arrival time is no larger than Terror_SRS_RSRP, where - Terror_SRS_RSRP = TC × NTA_offset + 4.67us for FR1 - Terror_SRS_RSRP = TC × NTA_offset + 3.67us for FR2 - NTA_offset is defined in Table 7.1.2-2 - TC is 0.509ns - The number of SRS ports in the SRS resource configured for measurement is 1, - The number of symbols in the SRS resource configured for measurement is 1, - The number of repetitions in the SRS resource configured for measurement is 1, - Frequency hopping, sequence group hopping or sequence hopping is disabled in the SRS resource configured for measurement, - The bandwidth of the SRS resource is 48 PRBs. - One of the following conditions is met - There is no other SRS resource with the same root sequence and on the same symbol and with same comb as the relevant SRS resource. - If multiple SRS resources are on the same symbol and with same comb, the distance between cyclic shifts of any two resources is no less than 6 if transmissionComb = n4, and no less than 4 if transmissionComb = n2.

During R16 L3 CLI measurement discussion, timing offset is critical issue due to timing difference between victim UE DL timing and SRS arrival timing from neighbour cell in victim UE side as show in below figure:

In Rel-16 CLI following agreement reached in the end:

- R16 CLI value: TCLI = Tc*NTA_offset + Tother

  - Tother = Tpropagation_delay + Tcell_phase_synchronization

  - Tother = 4.67usec for FR1 and 3.67usec for FR2 with (ISD =500m for FR1 and 200m for FR2 assumed)

And in section 9.7.1 following specification on UE measurement time window assumption is specified:

When the UE measures SRS-RSRP and CLI-RSSI, a constant offset relative to the downlink reference timing in the serving cell shall be applied. The constant offset value is derived by UE implementation and shall be at least Tc*NTA_offset.

Based on above agreements, residual time error is up to 4.67 us for FR1 and 3.67 us for FR2. And cell_phase_synchronization is 3us which is dominant factor on this residual time error.   For L1 CLI measurement, this time offset and residual time error maybe optimized with following reason:

- Rel-16 L3 CLI introduced for SRS transmitted from neighbor cell for dynamic TDD scenario (different gNBs have different DL-UL TDD pattern)

- Rel-19 SBFD operation with both UL sub-band and DL sub-band in DL BWP from SBFD gNB side, L1 CLI SRS-RSRP is target for intra-cell scenario which no cell phase error need to be considered

- If inter-cell L1 CLI measurement considered in Rel-19, cell phase error between cells/gNBs can be optimized under co-location and well synced scenario with info exchange between gNBs.

Observation 1: Time difference and residual timing error can be optimized for Rel-19 CLI SRS-RSRP measurement:

- No cell phase error for intra-cell L1 CLI measurement (target scenario for SBFD operation)

- Cell phase error can be optimized for inter-cell LI CLI measurement (if considered in Rel-19) e.g., co-located scenario

Proposal 6: For Rel-19 L1 CLI SRS-RSRP measurement, the time difference between UE’s DL reference timing in the serving cell and SRS arrival time shall be further discussed.

For other side condition, same assumption as L3 CLI measurement can be assumed as starting which pending on further agreement from RAN1.

Proposal 7: Reusing Rel 16 L3 CLI SRS-RSRP measurement assumption as starting point for following side conditions:

- SRS port: 1

- SRS bandwidth: 48 RBs

- SRS symbol: 1

- No SRS repetitions

- Frequency hopping, sequence group hopping or sequence hopping is disabled

- SNR side condition: 1 dB

3.2 Measurement period and measurement accuracy

As mentioned in section 3.1, R16 L3 CLI SRS-RSRP measurement is specified with 3 samples and separate accuracy requirements pending on SCS due to large residual timing error impact.

For Rel-19 L1 CLI measurement, one of major benefits compared to L3 CLI is short measurement delay by using CSI framework and timeline with minimum as 1ms.

Pending on whether residual time error can be optimized or not, RAN4 shall further evaluate L1 SRS-RSRP measurement accuracy and measurement delay considering both single shot and multiple shot measurement with different time error.

Proposal 8: RAN4 can evaluate L1 SRS-RSRP measurement performance considering both single shot and multiple shots measurements if different side condition identified e.g.

- For intra-cell L1 CLI SRS-RSRP measurement: Tother = 1.67usec for FR1 and 0.67usec for FR2 can be considered.

- For inter-cell L1 CLI SRS-RSRP measurement: Reduced values compared R16 CLI assumption can be considered.

- 3.3 Measurement reporting

Unlike R16 L3 CLI measurement, Rel-19 L1 CLI measurement use CSI reporting framework and at least A-P reporting supported.

Proposal 9: Focus on Aperiodic CSI reporting mode for Rel-19 CLI measurement, for other reporting modes hold on the discussion pending on further progress in RAN1.

- 3.4  Measurement and scheduling restriction

In Rel-16 L3 CLI,  FR2 QCL assumption leave to UE implementation with latest received PDSCH/coreset QCL.

R16 L3 CLI measurement:  Depending on the UE capability, UE is not expected to receive PDCCH/PDSCH on SRS-RSRP/CLI-RSSI measured OFDM symbols, and on Y/X data symbol before SRS-RSRP measured symbol in intra-band CA.

- Y = 1 : 15kHz/30kHz SCS for FR1 and 60kHz SCS for FR2

- Y = 2 : 60kHz SCS for FR1 and 120kHz SCS for FR2

For Rel-19 CLI, configuration/determination of ‘typeD’ QCL assumptions will be specified for the CLI measurement resource.  UE measurement method is still on discussion in RAN1 with following candidate

Agreement For SBFD aware UEs, CLI measurements is performed within the active DL BWP and the following can be considered - Method#1: UE measures RSSI within DL subband - Method#2: UE measures RSRP of aggressor UE within UL subband - Method#3: UE measures RSSI within UL subband - Method#4: UE measures RSSI within guard band, if guard band exists Note: If DL subband, UL subband or guard band is outside the active DL BWP, the above methods does not apply. Note: Method#4 does not imply that guard band is explicitly configured.

Proposal 10: UE measurement behaviour and corresponding measurement restriction and/or scheduling availability pending on further progress from RAN1.

4 Conclusion

In this contribution, initial view on RAN4 RRM impact was provided for Rel-19 Duplex Evo WI.

Overall RRM scope

Proposal 1: No new RRM requirement required for objective “Semi-static indication of time/frequency location of SBFD-sub band”

Proposal 2: For objective “UE transmission, reception and measurement behavior and procedures”, following potential RRM impact foreseen:

- L1 RRM measurement may need to be updated to following RAN1 update on CSI report configuration between SBFD slots and non-SBFD slots

- FFS on RAN4 RRM impact for UL PC and or spatial relation update e.g., unified TCI state framework and /or UL spatial info related framework.

Proposal 3: No RAN4 RRM requirements impact foreseen on “SBFD random access operation”, some update on RAN4 PRACH configuration maybe required pending on detailed RAN1 specification update.

Proposal 4: There is no RRM impact on gNB to gNB CLI handing.

Proposal 5: RAN4 need to specify L1 based CLI measurement requirements

- Leverage existing Rel-16 L3 CLI measurement requirements framework as much as possible with necessary update.

L1 CLI measurement

Observation 1: Time difference and residual timing error can be optimized for Rel-19 CLI SRS-RSRP measurement:

- No cell phase error for intra-cell L1 CLI measurement (target scenario for SBFD operation)

- Cell phase error can be optimized for inter-cell LI CLI measurement (if considered in Rel-19) e.g., co-located scenario

Proposal 6: For Rel-19 L1 CLI SRS-RSRP measurement, the time difference between UE’s DL reference timing in the serving cell and SRS arrival time shall be further discussed.

Proposal 7: Reusing Rel 16 L3 CLI SRS-RSRP measurement assumption as starting point for following side conditions:

- SRS port: 1

- SRS bandwidth: 48 RBs

- SRS symbol: 1

- No SRS repetitions

- Frequency hopping, sequence group hopping or sequence hopping is disabled

- SNR side condition: 1 dB

Proposal 8: RAN4 can evaluate L1 SRS-RSRP measurement performance considering both single shot and multiple shots measurements if different side condition identified e.g. residual timing error

- For intra-cell L1 CLI SRS-RSRP measurement: Tother = 1.67usec for FR1 and 0.67usec for FR2 can be considered.

- For inter-cell L1 CLI SRS-RSRP measurement: Reduced values compared R16 CLI assumption can be considered.

Proposal 9: Focus on Aperiodic CSI reporting mode for Rel-19 CLI measurement, for other reporting modes hold on the discussion pending on further progress in RAN1.

Proposal 10: UE measurement behaviour and corresponding measurement restriction and/or scheduling availability is pending on further progress from RAN1.

Reference

[1] RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, HiSilicon