# 3GPP TSG-RAN WG4 Meeting #112	R4-2412535

Maastricht, NETHERLANDS, 19th – 23th August, 2024

Agenda Item:	8.14.2

Source: 	Samsung

Title: 	Initial view on spatial channel modelling SI

Document for:	Discussion

1		Introduction

For Rel-19 Spatial channel modelling SI, the detailed SID as following:

The study item includes the following objectives: - Study practical spatial channel modelling methodology for both SU- and MU-MIMO demodulation requirements and CSI reporting requirements: - Identify the limitations of the current (i.e. up to and including Release 18) channel models and corresponding scenarios and how they relate to UE MIMO performance - Consider both Clustered Delay Line (CDL)-based and TDL-based channel modelling approaches - For CDL-based channel modelling, use the tuned repeatable spatial channel model of TR38.827 as the starting point and identify any necessary changes. - Study and verify test methodology feasibility including test complexity and achievable results uncertainty. The test complexity shall not be significantly increased. - The methodology shall include both FR1 (conducted) and FR2 (wireless cable), with first priority for FR1.

In this contribution, we bring brief analysis on spatial channel modelling.

2		Discussion

In the beginning of NR Rel-15, RAN4 has discussed whether consider CDL channel model or TDL channel model for RAN4 performance requirements. In the end, TDL channel model + MIMO correlation matric together with BF modelling was agreed to be continued used for NR demodulation requirements.

Table from R4-1801768

|  | NR TDL Channel Model | NR CDL Channel Model | Comments |
| --- | --- | --- | --- |
| Tx/Rx beamfoming impact | Model does not account the Tx/Rx beamforming. | Suitable for explicit modelling of Tx Beamforming and antenna geometry. Rx beamforming impact cannot be modelled. | Baseband only test methodology isolates the UE antenna impact. Baseband only test setup assumes beamlock and omni directional antenna at UE. Under practical conditions Tx/Rx beamforming will be applied and effective number of taps (and delay spread) might be reduced. Both models may need to be adjusted to be more close to the practical propagation conditions. |
| MIMO correlation modelling | Suitable for modelling configurable Tx/Rx antenna correlation | Tx/Rx antenna correlation is based on antenna geometry and spacing between elements. Effectively model will allow modelling of low Rx correlation only. | TDL channels model antenna correlation and would be the only option to consider baseband verification with different Rx correlation levels. |
| Power delay profile | PDP with multiple taps and low power for paths at larger delays. | PDP with multiple taps and low power for clusters at larger delays | Large number of taps may cause unnecessary TE complexity. For both models need to apply some filtering or thresholding to simplify model by reducing number low power paths/ clusters. |
| Delay spread | Suitable for setting a desired delay spread based on test conditions, SCS | Suitable for setting a desired delay spread based on test conditions, SCS | Both TDL and CDL models are well suited |

Both CDL and TDL channel model have pros and cons in terms of deployment scenario match, test complexity, feasibility of introducing UE performance requirements.

Proposal 1: The study on spatial channel model shall take following aspects into consideration:

- Deployment scenario match

- Test complexity

- Feasibility of introducing RAN4 performance requirements (stability, repeatability, alignability)

- Limitation of the current channel models and impact to UE receiver BB processing

For target scenario and test cases, we suggest to consider following case as example cases for study spatial channel modelling:

- SU-MIMO: 8Rx with 6-layer transmission test case

- MU-MIMO: R16 Type-II codebook  PMI test case

The detailed scenario assumption and parameters need to be further discussed.

Proposal 2: Using following test cases as example cases for further study:

- SU-MIMO: 8Rx with 6-layer transmission test case

- MU-MIMO: R16 Type-II codebook PMI test case

In SID, it’s mentioned ”CDL channel model use the tuned repeatable spatial channel model of TR38.827 as the starting point and identify any necessary changes”. The channel model of TR 38.827 was used to introduce MIMO OTA requirements with major difference summarized below:

- Requirement methodology: MIMO OTA requirements were specified based on measurement data campaign from commercial devices. RAN4 demodulation requirements are derived by simulation.  During Rel-18 MIMO OTA, companies tried to use simulation for developing FR2 MIMO OTA requirements, it’s failed in the end due to difficult to align the results among companies.

- Test complexity and RAN4 work effort:  Under each release, only limited requirements will be specified for MIMO OTA (1 or 2 cases) due to test complexity and work effort on developing requirements. Under each release, RAN4 developed hundreds of demodulation requirements for different features by comparable link level approach.

- Test environment: MIMO OTA was verified under OTA chamber to verify whole UE receiver performance including BB and RF front end especially antennal performance. For demodulation, we focus on to verify UE BB performance with cable or virtual cable which not consider UE antenna performance.

With above comparison between MIMO OTA and demodulation requirements, it’s questionable whether spatial channel model of TR38.827 can be used for demodulation requirements.

Proposal 3: RAN4 needs to further evaluate the feasibility of using the channel model of TR 38.827 for demodulation requirements.

3		Conclusion

In this contribution, we bring brief analysis on spatial channel modelling.

Proposal 1: The study on spatial channel model shall take following aspects into consideration:

- Deployment scenario match

- Test complexity

- Feasibility of introducing RAN4 performance requirements (stability, repeatability, alignability)

- Limitation of the current channel models and impact to UE receiver BB processing

Proposal 2: Using following test cases as example cases for further study:

- SU-MIMO: 8Rx with 6-layer transmission test case

- MU-MIMO: R16 Type-II codebook  PMI test case

Proposal 3: RAN4 needs to further evaluate the feasibility of using the channel model of TR 38.827 for demodulation requirements.

Reference

[1] RP-241610, New SID: Study on spatial channel model for demodulation performance requirements, Nokia

[2] R4-1801768, Discussion on Propagation Channel models for demodulation, Intel