3GPP TSG-RAN WG4 Meeting #112	   R4-2412540

Maastricht, Netherlands, 19th – 23rd August, 2024

Source:	Huawei, Hisilicon

Title:	Discussion on Rel-19 simultaneous Rx-Tx issues

Agenda item:			7.11.2

Document for:	Approval

# 1. Introduction

In RAN#104, the new work item[1] for simultaneous Rx-Tx capability and related band combinations was approved to further analyze the leftover issues from Rel-18 and study the simultaneous Rx-Tx requirements for combos.

This paper is to discuss the simplification on some of notes for simultaneous Rx-Tx in inter-band CA operating bands.

# 2. Discussion

Simultaneous Rx-Tx capability for inter-band CA, SUL and EN-DC band combinations were introduced from Rel-15. According to the description of the capability in TS38.306, it is conditional mandatory and the condition is described in the field, i.e. indicated in the RAN4 spec which combinations should mandatorily support simultaneous Rx/Tx. For the combinations without such indication, the capability is optional, i.e. for UE supporting simultaneous Rx/Tx, the capability should be reported, otherwise, the capability is absent or not reported. The capability is important for the network scheduling flexibility, and highly related to implementation, it should be stated clearly in spec and reported accurately.

In TS38.101-1 v18.6.0, Table 5.2A.2.1-1: Inter-band CA operating bands involving FR1 (two bands), the non-simultaneous Rx-Tx operation NOTE 5 and NOTE 7 are both related to band n77. For NOTE 5, if a UE supports band n77, the wide band filter applies, and band n78 share the same filter with n77.

For Note 7, the simultaneous Rx-Tx operation of n77-n79 cannot be supported, due to the wide band filter applied to band n77, and the narrow separation between band n77 and n79.

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 8) |
| --- | --- | --- |
| CA_n77-n787 | n77, n78 |  |
| CA_n77-n797 | n77, n79 |  |
| CA_n78-n795 | n78, n79 |  |
| NOTE 5: For UEs supporting band n77, the minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n78-n79 NR carriers. This restriction applies also for these carriers when applicable NR CA configuration is part of a higher order configuration. NOTE 7: The minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n77-n78 or n77-n79 NR carriers. This restriction applies also for these carriers when applicable NR CA configuration is part of a higher order configuration. NOTE 9: Only applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. Same restrictions are applied when applicable NR CA configuration is part of a higher order configurations. |  |  |

Observation 1: The reasons of non-simultaneous Rx-Tx operation for CA_n77-n79 and CA_n78-n79 illustrated in NOTE 5 and NOTE 7 are same.

Proposal 1: NOTE 5 and NOTE 7 can be merged for CA_n77-n79 and CA_n78-n79.

But for n77-n78, the reason that simultaneous Rx-Tx operation cannot be supported is that the frequency range of n78 is the subset of the frequency range of n77. In TS38.306, the capability of simultaneous Rx-Tx does not apply to the frequency range overlapping case.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| simultaneousRxTxInterBandCA Indicates whether the UE supports simultaneous transmission and reception in TDD-TDD and TDD-FDD inter-band NR CA. If this field is included in ca-ParametersNR-ForDC, it indicates the UE supports simultaneous transmission and reception between any UL/DL band pair within a cell group and across MCG and SCG in TDD-TDD and TDD-FDD inter-band NR-DC. It is mandatory for certain TDD-FDD and TDD-TDD band combinations defined in TS 38.101-1 [2], TS 38.101-2 [3] and TS 38.101-3 [4]. This capability does not apply to the following components within TDD-TDD and TDD-FDD inter-band NR-CA or NR-DC combinations: - Intra-band NR-CA or NR-DC component - Inter-band NR-CA or NR-DC component where the frequency range of one TDD band is a subset of the frequency range of the other NR TDD band (as specified in TS 38.101-1 [2]). | BC | CY | N/A | N/A |

Observation 2: The simultaneous Rx-Tx capability must not be reported for CA_n77-n78, as limited in RAN2 spec that the capability does not apply to the inter-band CA where the frequency range of one TDD band is a subset of the frequency range of the other NR TDD band.

Proposal 2: NOTE 9 should be applied to CA_n77-n78 in Table 5.2A.2.1-1.

Proposal 3: we propose the following change of Note for simultaneous Rx-Tx operations.

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 8) |
| --- | --- | --- |
| CA_n77-n789 | n77, n78 |  |
| CA_n77-n795 | n77, n79 |  |
| CA_n78-n795 | n78, n79 |  |
| NOTE 5: For UEs supporting band n77, the minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n78-n79 or n77-n79 NR carriers. This restriction applies also for these carriers when applicable NR CA configuration is part of a higher order configuration. NOTE 7: void. NOTE 9: Only applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. Same restrictions are applied when applicable NR CA configuration is part of a higher order configurations. |  |  |

# 3. Conclusions

This contribution presents the following observations and proposal:

Observation 1: The reasons of non-simultaneous Rx-Tx operation for CA_n77-n79 and CA_n78-n79 illustrated in NOTE 5 and NOTE 7 are same.

Observation 2: The simultaneous Rx-Tx capability must not be reported for CA_n77-n78, as limited in RAN2 spec that the capability does not apply to the inter-band CA where the frequency range of one TDD band is a subset of the frequency range of the other NR TDD band.

Proposal 1: NOTE 5 and NOTE 7 can be merged for CA_n77-n79 and CA_n78-n79.

Proposal 2: NOTE 9 should be applied to CA_n77-n78 in Table 5.2A.2.1-1.

Proposal 3: we propose the following change of Note for simultaneous Rx-Tx operations.

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 8) |
| --- | --- | --- |
| CA_n77-n789 | n77, n78 |  |
| CA_n77-n795 | n77, n79 |  |
| CA_n78-n795 | n78, n79 |  |
| NOTE 5: For UEs supporting band n77, the minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n78-n79 or n77-n79 NR carriers. This restriction applies also for these carriers when applicable NR CA configuration is part of a higher order configuration. NOTE 7: void. NOTE 9: Only applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. Same restrictions are applied when applicable NR CA configuration is part of a higher order configurations. |  |  |

# 4. Reference

[1] RP-241682, New WID Simultaneous RxTx band combinations for NR CA/DC, NR SUL and LTE/NR DC in Rel-19, RAN#104