3GPP TSG-RAN WG4 Meeting #112	R4-2412549

Maastricht, Netherlands, 19 August – 23 August 2024

Agenda Item:	5.28.2.1

Source:	Ericsson

Title:	UE demodulation and CSI reporting requirements for RedCap enhancements

Document for:	Discussion

# 1	Introduction

RAN4#111 completed the UE demodulation performance part for RedCap enhancements (eRedCap) [1], and agreed with the big CR of eRedCap UE demodulation and CSI reporting requirements [2].

Although RAN4 have completed the performance part discussion, this contribution discusses whether the Rel-17 RedCap CQI fading tests can be applicable for Rel-18 eRedCap UE.

# 2	CQI reporting test in fading condition

## 2.1	Background

RAN4 has agreed with the BWP/PDSCH/CSI-RS PRB size configuration for eRedCap CSI reporting tests as shown in Table 1 [3]. Based on the configuration, RAN4 discussed whether the Rel-17 RedCap CSI reporting requirements (e.g., SNR test points) can be applicable for Rel-18 eRedCap UEs.

Table 1	BWP/PDSCH/CSI-RS PRB size for eRedCap CSI reporting requirements.

|  | CQI reporting tests (static/fading) | PMI reporting test |
| --- | --- | --- |
| FDD/HD-FDD CBW/SCS=10MHz/15kHz | BWP: 52 RBs CSI-RS: 24RBs (PRB 0 to 23) PDSCH:15 RBs (PRB 0 to 14)) | BWP: 52 RBs CSI-RS: 28RBs (PRB 0 to 27) PDSCH: 25RBs (PRB 0 to 24) |
| TDD CBW/SCS=20MHz/30kHz | BWP: 51 RBs CSI-RS: 24RBs (PRB 0 to 23) PDSCH: 7RBs (PRB 0 to 6) | BWP: 51 RBs CSI-RS: 24RBs (PRB 0 to 23) PDSCH: 12RBs (PRB 0 to 11) |
| Note: BWP size is same as the transmission bandwidth in TS38.101-1 5.3.2. |  |  |

In RAN4#111, only two companies provided the simulation results for CQI reporting test in fading condition. One company argued the mismatch between the CSI-RS BW and PDSCH BW gives negative impact to the following CQI results (e.g., 7RBs for PDSCH vs. 24RBs for CSI-RS in TDD SCS=30kHz scenario), and therefore they proposed not to apply the CQI reporting test in fading condition to eRedCap UE [4]. On the other hand, another company showed their simulation results gives the same CQI reporting performance as Rel-17, and they proposed to apply the CQI reporting test in fading condition to eRedCap UE. Since only two companies provide the results, RAN4#111 was not able to conclude whether the CQI reporting test in fading condition is applicable for eRedCap UE. Note the mismatch comes from the eRedCap UE capability that it can receive CSI-RS with up to 20MHz, but PDSCH with up to 5MHz.

## 2.2	Simulation results

CQI reporting test in fading condition [3] - Interested companies are encouraged to provide the simulation results whether to apply the same test points and requirements as Rel-17 RedCap CQI reporting requirements in fading condition. - SNR=9/10dB and SNR=15/16dB for FDD/HD-FDD/TDD with 1Rx. - SNR=6/7dB for FDD/HD-FDD/TDD with 2Rx. - Requirements: - A CQI index not in the set {median CQI - 1, median CQI, median CQI + 1} shall be reported at least α% of the time where α = 20%. - Throughput ratio of follow CQI and fixed median CQI shall be ≥ γ, where γ = 1.05 - Average BLER with follow CQI shall be greater than or equal to 0.02.

### 2.2.1 TDD SCS=30kHz

Table 2 and Table 3 show our simulation results of the TDD SCS=30kHz CQI fading tests for 1Rx and 2Rx, respectively. The rows highlighted in yellow corresponding to the test points set for the Rel-17 RedCap CQI reporting test. For each test, we have configured CSI-RS with 24 and 8 RBs. Please note that CSI-RS BW = 8 RB is only the evaluation purpose to study the impact of mismatch between CSI-RS BW and PDSCH BW.

Table 2	Simulation results for TDD SCS=30kHz CQI reporting test in fading condition (1Rx).

|  | CSI-RS BW = 8 RBs (for evaluation) |  |  |  | CSI-RS BW = 24 RBs |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SNR | Median CQI | BLER | Throughput ratio (gamma) | Alpha | Median CQI | BLER | Throughput ratio (gamma) | Alpha |
| 4 | 6 | 0.21 | 1.25 | 0.58 | 6 | 0.28 | 1.23 | 0.53 |
| 5 | 7 | 0.21 | 1.27 | 0.55 | 7 | 0.28 | 1.23 | 0.56 |
| 6 | 7 | 0.20 | 1.35 | 0.58 | 7 | 0.28 | 1.27 | 0.57 |
| 7 | 8 | 0.19 | 1.25 | 0.59 | 8 | 0.28 | 1.16 | 0.54 |
| 8 | 8 | 0.19 | 1.28 | 0.61 | 8 | 0.28 | 1.18 | 0.55 |
| 9 | 9 | 0.19 | 1.24 | 0.58 | 9 | 0.28 | 1.14 | 0.54 |
| 10 | 9 | 0.20 | 1.25 | 0.60 | 9 | 0.28 | 1.16 | 0.56 |
| 11 | 10 | 0.19 | 1.33 | 0.62 | 10 | 0.28 | 1.21 | 0.54 |
| 12 | 10 | 0.20 | 1.32 | 0.56 | 10 | 0.28 | 1.21 | 0.53 |
| 13 | 11 | 0.19 | 1.27 | 0.58 | 11 | 0.27 | 1.17 | 0.52 |
| 14 | 11 | 0.19 | 1.26 | 0.61 | 11 | 0.27 | 1.16 | 0.53 |
| 15 | 12 | 0.19 | 1.26 | 0.59 | 12 | 0.26 | 1.15 | 0.53 |
| 16 | 12 | 0.19 | 1.23 | 0.54 | 12 | 0.25 | 1.14 | 0.52 |
| 17 | 13 | 0.17 | 1.24 | 0.55 | 13 | 0.24 | 1.14 | 0.50 |
| 18 | 13 | 0.16 | 1.22 | 0.37 | 13 | 0.23 | 1.11 | 0.41 |

Table 3	Simulation results for TDD SCS=30kHz CQI reporting test in fading condition (2Rx).

|  | CSI-RS BW = 8 RBs (for evaluation) |  |  |  | CSI-RS BW = 24 RBs |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SNR | Median CQI | BLER | Throughput ratio (gamma) | Alpha | Median CQI | BLER | Throughput ratio (gamma) | Alpha |
| 4 | 8 | 0.19 | 1.32 | 0.60 | 8 | 0.26 | 1.20 | 0.54 |
| 5 | 9 | 0.19 | 1.31 | 0.57 | 9 | 0.27 | 1.16 | 0.54 |
| 6 | 9 | 0.19 | 1.30 | 0.59 | 9 | 0.27 | 1.17 | 0.54 |
| 7 | 10 | 0.19 | 1.39 | 0.60 | 10 | 0.27 | 1.24 | 0.53 |
| 8 | 11 | 0.20 | 1.36 | 0.57 | 10 | 0.27 | 1.24 | 0.53 |
| 9 | 11 | 0.19 | 1.33 | 0.57 | 11 | 0.26 | 1.21 | 0.52 |
| 10 | 12 | 0.20 | 1.35 | 0.57 | 12 | 0.25 | 1.25 | 0.53 |
| 11 | 12 | 0.20 | 1.29 | 0.58 | 12 | 0.25 | 1.21 | 0.52 |
| 12 | 13 | 0.19 | 1.32 | 0.55 | 12 | 0.24 | 1.18 | 0.51 |
| 13 | 13 | 0.19 | 1.25 | 0.56 | 13 | 0.22 | 1.19 | 0.50 |
| 14 | 14 | 0.17 | 1.32 | 0.32 | 14 | 0.20 | 1.26 | 0.35 |
| 15 | 14 | 0.17 | 1.24 | 0.26 | 14 | 0.20 | 1.18 | 0.28 |
| 16 | 15 | 0.15 | 1.34 | 0.32 | 14 | 0.18 | 1.14 | 0.36 |
| 17 | 15 | 0.13 | 1.27 | 0.26 | 15 | 0.16 | 1.21 | 0.29 |
| 18 | 15 | 0.12 | 1.20 | 0.21 | 15 | 0.14 | 1.16 | 0.23 |

The simulation results show that the reported median CQI indexes are same regardless of CSI-RS BWs. But PDSCH BLER and throughput ratio between the follow CQI and fixed median CQI are different. Figure 1 shows the throughput ratio of follow CQI and fixed median CQI with different CSI-RS BW configuration. It is seen that the CSI-RS BW=8 RB results in larger throughput gain compared with 24 RB. We think it is because UE uses the same RBs for the CQI estimation and PDSCH transmission. However, we should also point out, the throughput ratio values are high enough compared with Rel-17 requirements, i.e., γ=1.05, even though CSI-RS BW = 24 RB.

Figure 1	Throughput ratio of follow CQI and fixed median CQI.

Observation 1: Throughput ratio of follow CQI and fixed median CQI is degraded due to the mismatch between CSI-RS BW and PDSCH BW.

Observation 2: Throughput ratio of follow CQI and fixed median CQI is high enough compared with the Rel-17 RedCap CQI fading test, i.e., γ=1.05, even if CSI-RS BW is 24RB.

### 2.2.2 FDD SCS=15kHz

Table 4 shows the simulation results for CQI reporting test in fading condition for FDD/HD-FDD SCS=15kHz, where we set the CSI-RS BW to 24 RB. It is also seen that we can keep the same requirements (i.e., α = 20%, γ=1.05, and BLER=0.02), as well as TDD SCS=30kHz.

Table 4	Simulation results for FDD/HD-FDD SCS=15kHz CQI reporting test in fading condition (CSI-RS BW = 24RBs).

|  | 1Rx |  |  |  | 2Rx |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SNR | Median CQI | BLER | Throughput ratio (gamma) | Alpha | Median CQI | BLER | Throughput ratio (gamma) | Alpha |
| 4 | 6 | 0.23 | 1.22 | 0.58 | 9 | 0.26 | 1.15 | 0.53 |
| 5 | 7 | 0.24 | 1.24 | 0.59 | 9 | 0.26 | 1.11 | 0.55 |
| 6 | 7 | 0.24 | 1.24 | 0.62 | 10 | 0.26 | 1.23 | 0.53 |
| 7 | 8 | 0.24 | 1.18 | 0.58 | 10 | 0.27 | 1.18 | 0.55 |
| 8 | 9 | 0.25 | 1.14 | 0.58 | 11 | 0.27 | 1.18 | 0.53 |
| 9 | 9 | 0.24 | 1.15 | 0.59 | 11 | 0.28 | 1.11 | 0.55 |
| 10 | 10 | 0.25 | 1.23 | 0.57 | 12 | 0.27 | 1.13 | 0.53 |
| 11 | 10 | 0.25 | 1.21 | 0.59 | 12 | 0.28 | 1.08 | 0.55 |
| 12 | 11 | 0.25 | 1.18 | 0.56 | 13 | 0.27 | 1.18 | 0.50 |
| 13 | 11 | 0.25 | 1.16 | 0.58 | 13 | 0.26 | 1.11 | 0.52 |
| 14 | 12 | 0.25 | 1.16 | 0.55 | 14 | 0.25 | 1.20 | 0.33 |
| 15 | 12 | 0.25 | 1.11 | 0.58 | 14 | 0.23 | 1.14 | 0.27 |
| 16 | 13 | 0.24 | 1.20 | 0.51 | 15 | 0.21 | 1.60 | 0.21 |
| 17 | 13 | 0.23 | 1.15 | 0.54 | 15 | 0.19 | 1.47 | 0.27 |
| 18 | 14 | 0.22 | 1.21 | 0.56 | 15 | 0.17 | 1.37 | 0.21 |

# 3	Summary

Observation 1: Throughput ratio of follow CQI and fixed median CQI is degraded due to the mismatch between CSI-RS BW and PDSCH BW.

Observation 2: Throughput ratio of follow CQI and fixed median CQI is high enough compared with the Rel-17 RedCap CQI fading test, i.e., γ=1.05, even if CSI-RS BW is 24RB.

With the observations, we propose to apply Rel-17 RedCap CQI fading test to Rel-18.

Proposal 1: Apply the Rel-17 RedCap CQI reporting test in fading condition to Rel-18 eRedCap:

- A CQI index not in the set {median CQI - 1, median CQI, median CQI + 1} shall be reported at least α% of the time, where α = 20%.

- Throughput ratio of follow CQI and fixed median CQI shall be ≥ γ, where γ = 1.05

- Average BLER with follow CQI shall be greater than or equal to 0.02.

Proposal 2: Set SNR=9/10dB and SNR=15/16dB for CQI reporting test in fading condition for FDD/HD-FDD/TDD with 1Rx, same as Rel-17 RedCap CQI reporting test in fading condition.

Proposal 3: Set SNR=6/7dB for CQI reporting test in fading condition for FDD/HD-FDD/TDD with 2Rx, same as Rel-17 RedCap CQI reporting test in fading condition.

# References

- RP-241124, “Status report for WI Perf. part: Enhanced support of reduced capability NR devices”, Ericsson.

- R4-2408778, “Big CR for 38.101-4: Introduction of eRedCap UE demodulation and CSI reporting requirements”, Ericsson.

- R4-2406052, “Way Forward for [110bis][330] NR_redcap_enh_demod”, Ericsson.

- R4-2410123, “Topic summary for [111][330] NR_redcap_enh_demod”, Moderator (Ericsson).