3GPP TSG-RAN WG4 Meeting #112	R4-2412552

Maastricht, Netherlands, 19 August – 23 August 2024

Agenda Item:	8.8.4

Source:	Ericsson, Anritsu Corporation

Title:	TE-emulated channel model for NTN UE demodulation requirements

Document for:	Discussion

# 1	Introduction

RAN#104 agreed with the new Rel-19 RAN4-led WI enhanced requirements and test methodology for NR and IoT NTN“ [1], where one of the objectives is the NTN testing for NGSO as follows:

Specify the TE-emulated channel model with varying Doppler and delay shifts for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands - Match the satellite motion trajectory based on the ephemeris for NGSO (Non-Geostationary Orbit) scenarios - Inform RAN5 to assist specifying RF frequency error tests, if needed - Checking if the existing frequency error requirement, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE RRM performance test of uplink timing for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Checking if the existing RRM uplink timing requirements, can be met under the TE-emulated channel model without intention of modifying the existing core requirement Specify UE demodulation performance requirement(s) of PDSCH for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Minimize the number of demodulation performance requirements Specify the applicability of the tests under the TE-emulated channel model

This contribution mainly discusses the TE-emulated channel models which affect to RF, RRM and demodulation performance parts.

# 2	TE-emulated channel model for NTN

## 2.1	Background

RAN4 UE performance test configurations for NTN/IoT-NTN are specified with the assumption that UE has already performed the pre-compensation based on the ephemeris information signaled by the network. This means, for example, the UL frequency error requirement becomes same as Rel-15 terrestrial UE, i.e., within +/- 0.1ppm. Similarly, the UE demodulation requirements for NTN/IoT-NTN are also specified with the assumption UE has already performed the DL pre-compensation. This means the Doppler frequency applied to the channel models are derived only from the UE mobility, i.e., 200Hz used with NTN-TDLA100 and NTN-TDLC5.

Observation 1: RAN4 performance test is defined based on the assumption UE has already performed the pre-compensation based on the ephemeris information signaled from the network.

RAN5 conformance test environment for NTN test cases, specified in TS 38.508 clause 5.6.2.1, uses the ephemeris configuration:

- corresponding to zero Doppler and constant delay for the type of satellite under test, keeping such information constant throughout the duration of each measurement in the test.

For example, according to TS 38.521-5, the Doppler shift is set to 0.013 ppm and one-way delay is set to 2.03ms for LEO-600 scenario (Table 5.6.2.1-2), and this configuration is used for the conformance tests of UE demodulation requirements and RF requirements except for UL frequency error test. For the UL frequency error tests, for example, the Doppler shift is set to 22.65ppm and the one-way delay is set to 6.44ms for the maximum positive Doppler scenario (Table 6.4.1_2.4.3-1).

Observation 2: RAN5 NGSO conformance test procedure sets the constant Doppler shift and delay with the range:

- One-way delay between UE and satellite to be between 2 ms (lowest value for LEO orbit 600 km) and 6.67 ms (highest value for LEO orbit 1200 km)

- Doppler (frequency) shift up to 24 ppm for NGSO (LEO-600).

The test methods above have already confirmed in RAN#101 [2] as follows:

The condition for testing the frequency error requirements to verify NGSO UE pre-compensation functionality for Doppler shift, which will be implemented in the RAN5 NTN specifications - For example, setting an artificial fixed Doppler shift, which is randomly selected out of a range of Doppler shift values decided by RAN4, as the condition in a test The condition for RRM uplink timing test cases to verify NGSO UE delay pre-compensation functionality, which can be implemented in RAN5 NTN specifications - For example, setting an artificial fixed delay, which is randomly selected out of a range of delay values decided by RAN4, as the condition in a test - Note: For all the other NTN RRM test cases (except NGSO scenario in UE timing TCs), zero doppler shift and zero timing shift are assumed to be configured for RAN5 conformance testing

## 2.2	Varying frequency/delay shift model

### 2.2.1	Deployment scenario

According to the objective in WID, the TE-emulated channel model should consider varying Doppler (frequency) and delay shifts. In Rel-16 HST WI, RAN4 has already used the varying Doppler shift and delay for HST-SFN channel model as specified in TS 38.101-4 B.3.2.

The difference between NGSO scenario and HST scenario is that HST scenario is based on the single path channel (AWGN + Doppler shift), although NGSO scenario assumes multi-path fading channel such as NTN-TDLA100-200. Although it is up to RAN5 conformance test setup, we think the system simulator needs to apply the variable frequency shift and delay shift before the signals are fed into the fading simulator as shown in Figure 1.

Figure 1	Example of test equipment connection for GNSO UE tests.

To derive the variable frequency/delay shifts, we consider a simple NGSO moving scenario as illustrated in Figure 2. The description of parameters and the example of values for LEO-600 are shown in Table 1. According to the deployment parameters, it is possible to derive several examples of frequency/time shift models. In the following subsections, we show examples with E=30, 60, and 80 degrees.

Figure 2	Deployment model for NGSO.

Table 1	Example of deployment parameters for LEO-600.

| Parameters | Description | Typical values for LEO-600 (TS 38.821 Table 4.2-2, Table 6.1.2-1, Table 7.1-1) |
| --- | --- | --- |
| hNGSO | Altitude of satellite | 600 km |
| vNGSO | Relative speed of satellite with respect to earth | 7.56 km/s |
| EAOS | Elevation angle of the acquisition of satellite (AOS) | 30 degrees |
| ELOS | Elevation angle of the loss of satellite (LOS) | 30 degrees |
| dh,AOS | Horizontal distance between satellite and UE at the AOS | hNGSO/tan⁡(EAOS) |
| dh,LOS | Horizontal distance between satellite and UE at the LOS | hNGSO/tan⁡(ELOS) |
| dAOS | Distance between satellite and UE at the AOS | dh,AOS/cosEAOS |
| dLOS | Distance between satellite and UE at the LOS | dh,LOS/cosELOS |

#### 2.2.1.1	Example 1: E=30 degrees

Figure 3 shows the distance between the satellite and UE with time t, where Figure 3 (a) is the distance between satellite and UE, i.e., d(t), and Figure 3 (b) is the horizonal distance between satellite and UE, dh(t). Figure 4 illustrates the relation between d(t) and dh(t). As shown in the figure, we assume E=EAOS=ELOS, for simplicity. The maximum horizontal distance (2dh,max) corresponds to the footprint size. In the case of E=30 degrees, the footprint size is around 2,000 km. Considering the LEO-600 satellite speed of 7.56km/s, the coverage time (UE is served by one satellite) is around 275 seconds.

| (a) Distance between satellite and UE. | (b) Horizontal distance between satellite and UE |
| --- | --- |

Figure 3	Distance between satellite and UE for 30 degrees.

Figure 4	Illustration of d(t) and dh(t).

In this scenario, the propagation delay and Doppler shift is shown in Figure 5.

Propagation delay, T(t), is derived from the formula:

dht=modvNGSO⋅t, 2dh,max-dh,max

Tt=dtc=dh(t)2+hNGSO2c

where

- dh(t) is the horizontal distance between the satellite and UE at time t,

- d(t) is the distance between the satellite and UE at time t,

- dh,max is the maximum horizontal distance between the satellite and UE, i.e., hNGSO/tan⁡(E),

- c is the speed of light.

Like the HST channel model, the Doppler shift, F(t), is derived from the formula:

Ft=-fddhtd(t)=-fddh(t)dht2+hNGSO2

where

- fd is the maximum Doppler shift.

In this model, it is assumed that fd is 48kHz according to TR 38.821 Table 4.2-2, where the maximum Doppler shift is 24ppm with the carrier frequency of 2.0 GHz.

| (a) Propagation delay | (b) Doppler shift |
| --- | --- |

Figure 5	Propagation delay and Doppler shift for the case E=30 degrees.

#### 2.2.1.2	Example 2: E=60 degrees

Figure 6 shows the case E=60 degrees. In this case, the footprint size is around 700km, and the coverage time is about 90 seconds.

| (a) Propagation delay | (b) Doppler shift |
| --- | --- |

Figure 6	Propagation delay and Doppler shift for the case E=60 degrees.

#### 2.2.1.3	Example 3: E=80 degrees

Figure 7 shows the case E=80 degrees. In this case, the footprint size is around 210km, and the coverage time is about 28 seconds.

| (a) Propagation delay | (b) Doppler shift |
| --- | --- |

Figure 7	Propagation delay and Doppler shift for the case E=80 degrees.

## 2.3	Discussion

Table 2 summarizes the ranges of Doppler shift and propagation delay according to the elevation angles of the acquisition and loss of the satellites (E=EAOS=ELOS). The range of propagation delay shift and Doppler shift becomes small as the elevation angles become large, since the footprint size becomes small.

Table 2	Ranges of Doppler shift and propagation delay.

| Parameters | E=30degrees | E=60degrees | E=80degrees |
| --- | --- | --- | --- |
| Footprint size | 2,078 km | 692 km | 211 km |
| Coverage time | 275 seconds | 91 seconds | 28 seconds |
| Range of Doppler shift | +/- 41,568 Hz | +/- 23,998 Hz | +/- 8,330 Hz |
| Range of propagation delay | 2 ms to 4 ms | 2 ms to 2.31 ms | 2 ms to 2.03 ms |

The objective of this WI mentions that the TE-emulated channel mode should match the satellite motion trajectory. If we assume E=30 degrees with LEO-600, we can emulate the worst-case scenario, i.e., Doppler shift changes between -42kHz to +42kHz, however it requires to run the test for 27500 radio frames (275 seconds), to cover one satellite coverage time. We are not sure it is realistic configuration considering the DUT test time. On the other hand, larger elevation angle can shorten the satellite coverage time, but the Doppler shift range and delay shift become small.

RAN4 also need to discuss how to apply the varying Doppler (frequency) and delay shifts to the UE performance requirements specified in TS 38.101-5/133 and TS 36.102/133. For example, RAN4 has discussed the varying Doppler and delay shift models for IoT-NTN SAN demodulation requirements, and it was concluded that the model is applicable only for conformance tests. This means the SAN demodulation conformance test specification in TS 36.181 applies the varying shift models. If we apply this methodology, since UE conformance test requirements are specified by RAN5, RAN4 does not change any performance core requirements, but RAN4 may specify the varying Doppler (frequency) and delay shift models in specified in TS 38.101-5/133 and TS 36.102/133 so that RAN5 can refer to.

# 3	Summary

We propose RAN4 first agree with the varying Doppler and delay shift models. Then RAN4 discuss the suitable parameters for TE-emulated channel model considering the realistic satellite motion trajectory and test time. After that, RAN4 need to discuss how to apply the models.

Observation 1: RAN4 performance test is defined based on the assumption UE has already performed the pre-compensation based on the ephemeris information signaled from the network.

Observation 2: RAN5 NGSO conformance test procedure sets the constant Doppler shift and delay with the range:

- One-way delay between UE and satellite to be between 2 ms (lowest value for LEO orbit 600 km) and 6.67 ms (highest value for LEO orbit 1200 km)

- Doppler (frequency) shift up to 24 ppm for NGSO (LEO-600).

Proposal 1: Assume the varying Doppler (frequency) and delay shifts are applied before the fading simulator.

Proposal 2: Use the following model to derive the TE-emulated channel for NTN:

dht=modvNGSO⋅t, 2dh,max-dh,max

Tt=dtc=dh(t)2+hNGSO2c

Ft=-fddhtd(t)=-fddh(t)dht2+hNGSO2

where

- Tt: Propagation delay of the signal from the satellite at time t,

- F(t): Doppler (frequency) shift of the signal from the satellite at time t.

- dh(t): the horizontal distance between the satellite and UE at time t,

- d(t): the distance between the satellite and UE at time t,

- dh,max: the maximum horizontal distance between the satellite and UE, i.e., hNGSO/tan⁡(E),

- c: the speed of light,

- fd: Maximum Doppler shift (Hz)

Proposal 3: RAN4 further discuss the suitable parameters for TE-emulated channel model, e.g., height of satellite, speed of satellite, considering realistic satellite motion trajectory and test time.

Proposal 4: RAN4 discuss how to apply the varying Doppler and delay shifts to the UE performance requirements.

# References

- RP-240857, “New WID: Enhanced requirements and test methodology for NR and IoT NTN”, RAN4 chair (Huawei).

- RP-232682, “Summary of offline discussions for NTN testing”, RAN WG4 Chair, RAN WG5 Chair.

- TR 38.821, “Solutions for NR to support Non-Terrestrial Networks (NTN)”, V16.2.0, 3GPP.

# Appendix

## A.1	SIB19 NTN Ephemeris Information for zero Doppler conditions specified in TS 38.508

Table 5.6.2.1-1: SIB19- NR NTN Ephemeris information for GSO satellites (zero Doppler conditions)

| Derivation Path: Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -16944523 |  |  |
| positionY-r17 | 27362209 |  |  |
| positionZ-r17 | 4019393 |  |  |
| velocityVX-r17 | -336 |  |  |
| velocityVY-r17 | -208 |  |  |
| velocityVZ-r17 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 35.04 degrees, one-way delay equal to 127.33 ms and Doppler equal to 4.89E-06 ppm |  |  |  |

Table 5.6.2.1-2: SIB19- NR NTN Ephemeris information for NGSO (LEO-600) satellites (zero Doppler conditions)

| Derivation Path: Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2550081 |  |  |
| positionY-r17 | 4151097 |  |  |
| positionZ-r17 | 2263034 |  |  |
| velocityVX-r17 | 21970 |  |  |
| velocityVY-r17 | -48164 |  |  |
| velocityVZ-r17 | 114109 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 89.61 degrees, one-way delay equal to 2.03 ms and Doppler equal to 0.013 ppm |  |  |  |

Table 5.6.2.1-3: SIB19- NR NTN Ephemeris information for NGSO (LEO1200) satellites (zero Doppler conditions)

| Derivation Path: Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2771542 |  |  |
| positionY-r17 | 4509243 |  |  |
| positionZ-r17 | 2455572 |  |  |
| velocityVX-r17 | 20837 |  |  |
| velocityVY-r17 | -46176 |  |  |
| velocityVZ-r17 | 109528 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 89.54 degrees, one-way delay equal to 4.04 ms and Doppler equal to 3.6E-3 ppm |  |  |  |

## A.2	SIB19 NTN Ephemeris Information for zero Doppler conditions for UL frequency error test specified in TS 38.521-5

Table 6.4.1_2.4.3-1: SIB19 for RF tests with NR NTN Ephemeris Information for NGSO (LEO-600) satellites (maximum positive Doppler)

| Derivation Path: TS 38.508-1, Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EphemerisInfo-r17 ::= CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2717617 |  |  |
| positionY-r17 | 4550419 |  |  |
| positionZ-r17 | 852799 |  |  |
| velocityVX-r17 | 6164 |  |  |
| velocityVY-r17 | -19424 |  |  |
| velocityVZ-r17 | 124281 |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 10 degrees, one-way delay equal to 6.44 ms and Doppler equal to 22.65 ppm. |  |  |  |

Table 6.4.1_2.4.3-2: SIB19 for RF tests with NR NTN Ephemeris Information for NGSO (LEO-600) satellites (maximum negative Doppler)

| Derivation Path: TS 38.508-1, Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EphemerisInfo-r17 ::= CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2199272 |  |  |
| positionY-r17 | 3404229 |  |  |
| positionZ-r17 | 3535794 |  |  |
| velocityVX-r17 | 35394 |  |  |
| velocityVY-r17 | -74414 |  |  |
| velocityVZ-r17 | 94682 |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 169.97 degrees, one-way delay equal to 6.60 ms and Doppler equal to -22.62 ppm. |  |  |  |

Table 6.4.1_2.4.3-3: SIB19 for RF tests with NR NTN Ephemeris Information for NGSO (LEO-600)  satellites (maximum positive Doppler/2)

| Derivation Path: TS 38.508-1, Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EphemerisInfo-r17 ::= CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2592823 |  |  |
| positionY-r17 | 4245650 |  |  |
| positionZ-r17 | 2024520 |  |  |
| velocityVX-r17 | 19359 |  |  |
| velocityVY-r17 | -43278 |  |  |
| velocityVZ-r17 | 116553 |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 60.25 degrees, one-way delay equal to 2.30 ms and Doppler equal to 11.29 ppm. |  |  |  |

## A.3	TR 38.821 Table 4.2-2: Reference scenario parameters

| Scenarios | GEO based non-terrestrial access network (Scenario A and B) | LEO based non-terrestrial access network (Scenario C & D) |
| --- | --- | --- |
| Orbit type | notional station keeping position fixed in terms of elevation/azimuth with respect to a given earth point | circular orbiting around the earth |
| Altitude | 35,786 km | 600 km 1,200 km |
| Spectrum (service link) | <6 GHz (e.g. 2 GHz) >6 GHz (e.g. DL 20 GHz, UL 30 GHz) |  |
| Max channel bandwidth capability (service link) | 30 MHz for band < 6 GHz 1 GHz for band > 6 GHz |  |
| Payload | Scenario A: Transparent (including radio frequency function only) Scenario B: regenerative (including all or part of RAN functions) | Scenario C: Transparent (including radio frequency function only) Scenario D: Regenerative (including all or part of RAN functions) |
| Inter-Satellite link | No | Scenario C: No Scenario D: Yes/No (Both cases are possible.) |
| Earth-fixed beams | Yes | Scenario C1: Yes (steerable beams), see note 1 Scenario C2: No (the beams move with the satellite) Scenario D 1: Yes (steerable beams), see note 1 Scenario D 2: No (the beams move with the satellite) |
| Max beam foot print size (edge to edge) regardless of the elevation angle | 3500 km (Note 5) | 1000 km |
| Min Elevation angle for both sat-gateway and user equipment | 10° for service link and 10° for feeder link | 10° for service link and 10° for feeder link |
| Max distance between satellite and user equipment at min elevation angle | 40,581 km | 1,932 km (600 km altitude) 3,131 km (1,200 km altitude) |
| Max Round Trip Delay (propagation delay only) | Scenario A: 541.46 ms (service and feeder links) Scenario B: 270.73 ms (service link only) | Scenario C: (transparent payload: service and feeder links) 25.77 ms (600km) 41.77 ms (1200km) Scenario D: (regenerative payload: service link only) 12.89 ms (600km) 20.89 ms (1200km) |
| Max differential delay within a cell (Note 6) | 10.3 ms | 3.12 ms and 3.18 ms for respectively 600km and 1200km |
| Max Doppler shift (earth fixed user equipment) | 0.93 ppm | 24 ppm (600km) 21ppm(1200km) |
| Max Doppler shift variation (earth fixed user equipment) | 0.000 045 ppm/s | 0.27ppm/s (600km) 0.13ppm/s(1200km) |
| User equipment motion on the earth | 1200 km/h (e.g. aircraft) | 500 km/h (e.g. high speed train) Possibly 1200 km/h (e.g. aircraft) |
| User equipment antenna types | Omnidirectional antenna (linear polarisation), assuming 0 dBi Directive antenna (up to 60 cm equivalent aperture diameter in circular polarisation) |  |
| User equipment Tx power | Omnidirectional antenna: UE power class 3 with up to 200 mW Directive antenna: up to 20 W |  |
| User equipment Noise figure | Omnidirectional antenna: 7 dB Directive antenna: 1.2 dB |  |
| Service link | 3GPP defined New Radio |  |
| Feeder link | 3GPP or non-3GPP defined Radio interface | 3GPP or non-3GPP defined Radio interface |
| NOTE 1: Each satellite has the capability to steer beams towards fixed points on earth using beamforming techniques. This is applicable for a period of time corresponding to the visibility time of the satellite NOTE 2: Max delay variation within a beam (earth fixed user equipment) is calculated based on Min Elevation angle for both gateway and user equipment NOTE 3: Max differential delay within a beam is calculated based on Max beam foot print diameter at nadir NOTE 4: Speed of light used for delay calculation is 299792458 m/s. NOTE 5: The Maximum beam foot print size for GEO is based on current state of the art GEO High Throughput systems, assuming either spot beams at the edge of coverage (low elevation). NOTE 6: The maximum differential delay at cell level has been computed considering the one at beam level for largest beam size. It does not preclude that cell may include more than one beam when beam size are small or medium size. However the cumulated differential delay of all beams within a cell will not exceed the maximum differential delay at cell level in the table above. |  |  |

## A.4	TR 38.821 Table 7.1-1: NTN scenarios versus delay constraints

| NTN scenarios | A | B | C1 | C2 | D1 | D2 |
| --- | --- | --- | --- | --- | --- | --- |
|  | GEO transparent payload | GEO regenerative payload | LEO transparent payload |  | LEO regenerative payload |  |
| Satellite altitude | 35786 km |  | 600 km |  |  |  |
| Relative speed of Satellite with respect to earth | negligible |  | 7.56 km per second |  |  |  |
| Min elevation for both feeder and service links | 10° for service link and 10° for feeder link |  |  |  |  |  |
| Typical Min / Max NTN beam foot print diameter (note 1) | 100 km / 3500 km |  | 50 km / 1000 km |  |  |  |
| Maximum propagation delay contribution to the Round Trip Delay on the radio interface between the gNB and the UE | 541.46 ms (Worst case) | 270.73 ms | 25.77 ms |  | 12.89 ms |  |
| Minimum propagation delay contribution to the Round Trip Delay on the radio interface between the gNB and the UE | 477.48 ms | 238.74 ms | 8 ms |  | 4 ms |  |
| Maximum Round Trip Delay variation as seen by the UE (note 2) | Negligible |  | Up to +/- 93.0 µs/sec (Worst case) |  | Up to +/- 47.6 µs/sec |  |
| NOTE 1: The beam foot print diameter are indicative. The diameter depends on the orbit, earth latitude, antenna design, and radio resource management strategy in a given system. NOTE 2: The delay variation measures how fast the round trip delay (function of UE-satellite-NTN gateway distance) varies over time. The worst case is when the satellite moves towards/away from the UE at 10⁰ elevation angle, assuming the UE speed is 1200 km/h. It is expressed in µs/s and is negligible for GEO scenario. NOTE 3: Void NOTE 4: Speed of light used for delay calculation is 299792458 m/s. |  |  |  |  |  |  |