3GPP TSG-RAN WG4 Meeting #112                                   R4-2412553

Maastricht, Netherlands, August 19 – August 23, 2024

Source: 	MediaTek Inc.

Title: 	Discussion on power domain enhancement for NR single carrier

Agenda Item:	8.1.1.2.1

Document for:	Approval

- Introduction

In RAN#111, the following was agreed in [1] as shown below, in addition to simulation assumptions to use for further evaluations. This document provides further evaluations to support closure of the open points.

.

- Extended UE channel bandwidth further study

The definition of Inner RB allocation and Outer RB allocations in TS38.101-1 show that in theory the NRB of 20MHz channel bandwidth is covered by the maximum Inner RB allocation of 40MHz channel bandwidth, and therefore extending the channel bandwidth by ½ bandwidth symmetrically would enable NRB to be equivalent to an Inner RB allocation from MPR perspective. Simulation analysis is shown in section 3.

We have also further studied the different FFS aspects highlighted in [1] in the sub-sections below.

2.1 Extended UE channel bandwidth exceeding BS CBW edge

Exceeding the BS CBW edge with the extended UE channel bandwidth may not be acceptable on assumption that the spectrum outside of the BS CBW is owned by another operator. One could argue that system level simulations for inter-operator coexistence may need to be provided, and this may be too much for the current WI, also taking into account the scenario prioritisation agreed in RAN4#110bis. Therefore, we would suggest that we focus on the case where extended UE CBW is confined within the BS CBW.

We do believe however that an asymmetrical extended CBW scenario can be considered to support this case. This scenario is depicted in the figure below. This was defined as “one-sided emissions relaxation” in MediaTek documents [2] and [3] submitted to RAN4#110bis and RAN4#111.

Figure 1: Asymmetrical extended CBW

Two aspects to consider for asymmetric channel bandwidths are:

- Based on existing Inner RB allocation definitions, it would not be feasible for a full RB allocation of the CBW to be equivalent to an Inner RB allocation of extended CBW.

- The centre location of the CBW would not be at the centre of the extended CBW, and this may lead to some differences in the leakage profile of the UE transmission and achievable MPR reduction for a given RB allocation.

Simulation analysis on this scenario is provided in section 3, and demonstrates that, for an RB allocation < NRB of the CBW, MPR reduction can still be applied.

Proposal 1: Specify the case where extended CBW does not exceed the BS CBW edge.

Proposal 2: For the case where a UE CBW is allocated at the edge of the BS CBW, agree to consider further an “Asymmetrical extended CBW” to allow outer RB allocation MPR reduction applicability for a UE CBW allocated at the edge of the BS CBW.

2.2 Applicability of IBE in region between UE CBW and ext-CBW

As highlighted in MediaTek’s analysis [2] and [3] in previous meetings, the UE CBW = BS CBW for LTE, and therefore IBE was always applicable for UE Tx emissions within a BS CBW, and ACLR/SEM was applicable for emissions outside of the BS CBW. For NR, we defined the same approach for an eMBB UE that operates UE CBW = BS CBW. However, for UE CBW < BS CBW that does not apply today and we see no reason from a specification perspective why it could not apply – especially for a RedCap UE where in high FR1 bands UE CBW < BS CBW may frequently apply.

From a device implementation perspective, if the ACLR/SEM restrictions leading to Outer RB allocation MPR are removed, there should be no problems to fulfil IBE requirements if those restrictions are removed and MPR requirements for Inner RB allocations apply instead, because the UE is required to achieve IBE and EVM requirements today for Inner RB allocations.

It is acknowledged that the inter-RB interference within the BS CBW may be slightly higher from applying IBE rather than applying ACLR/SEM, but this is a trade-off that operators may make in their networks. Therefore, we believe it would be reasonable for operators to have full control over whether the “extended CBW” is permitted to be used, via network signalling to the UE, whereby e.g. a network may permit the usage of extended CBW if it is interested to enable the MPR reduction feature within the cell.

Proposal 3: Agree that IBE shall apply in the region between UE CBW and extended CBW. Also agree to allow operators to be given control via network signalling to the UE of whether usage of extended UE CBW is permitted within the cell.

2.3 Starting point of ΔfOOB for UE ACLR/SEM

In our view, the ext-CBW could be equal to the BS CBW, or not. However, our analysis shows that ext-CBW = 2 x UE CBW is sufficient to allow NRB to be equivalent to an inner RB allocation, and an Asymmetrical ext-CBW can also apply when there is not sufficient gap to the BS CBW edge. Therefore, extending beyond those options does not seem necessary for Rel-19.

Furthermore, defining different rules per BS CBW would seem to further complicate requirements and rules for the UE, and may be unnecessary. We also see the main applicability of this feature in general for RedCap UE operating 20MHz CBW, so this UE would not ordinarily have those rules naturally in place for all CBWs up to 100MHz.

Also, we propose to apply ACLR and SEM from the same point. We would not like to apply ACLR from edge of ext-CBW and SEM from edge of UE CBW, as this would complicate further the definitions of Inner and Outer RB allocations, with increased impact from SEM likely leading to more restrictions on narrow allocations near the edge of the UE CBW, which may put more restrictions on the Inner RB allocation applicability for this.

Proposal 4: Agree that ΔfOOB shall be the same for ACLR and SEM when extended UE CBW applies.

Proposal 5: Agree that requirements for ACLR and SEM apply from edge of extended CBW.

2.4 Integral region of OOBE – based on UE CBW or ext-CBW

We provide our analysis of the pros and cons in the table below. We have also performed simulations to identify whether there would be any issue to apply OOBE based on the UE CBW, starting from edge of extended CBW.

Table 1: Analysis of OOBE integration options

|  | OOBE over UE CBW | OOBE over ext-CBW |
| --- | --- | --- |
| Specification impact | More complex: Would need to define the “shifted edge” in OOBE section. Mapping needed to higher channel BW size for MPR rules. | Simpler: Can re-apply MPR formulas and refer to defined channel BW sizes and corresponding OOBE requirements. |
| Impact to other requirements | If SE shifted with OOBE region, then it can still fulfil ITU requirements for 250% of UE CBW. Similar IBE requirements impact for both options, i.e. extending IBE applicability outside of UE CBW. | SE for 250% of UE CBW would not be achievable if applied from edge of ext-CBW OOBE region. No issue if the 250% applied to ext-CBW instead. Similar IBE requirements impact for both options, i.e. extending IBE applicability outside of UE CBW. |
| Impact on MPR reduction | Our simulations suggest that Inner RB Allocation MPR can apply for all RB allocations inside 20MHz channel | From MPR formulas it can be derived that Inner RB Allocation MPR can apply for all RB allocations inside 20MHz channel |

2.5 Spurious Emission FOOB boundary

The Spurious Emission boundary is currently specified to be at the end of the OOBE boundary, and 5MHz away from the ACLR region. As stated in MediaTek documents [2] and [3] in the last 2 meetings, ITU allows the Spurious Emission to apply 250% of the channel BW from the centre of the channel. 3GPP requirements are more stringent but driven by legacy UMTS rather than regulation. Therefore, shifting the boundary compared to current 3GPP FOOB is feasible.

We do believe that from specification perspective it would be more reasonable to shift the FOOB if we apply an extended CBW. Also from a minimum requirements perspective, if we do not shift the SE boundary but we shift the OOBE, then there would be a point where the two requirements overlap and the Spurious Emission requirement would be more stringent than ACLR/SEM.

Proposal 6: Agree to shift the FOOB boundary to the edge of the OOBE for extended CBW.

- 3. Simulation analysis

The following table provides the outcome of our simulation analysis, using the simulation assumptions agreed in the last meeting in [1], and also evaluated the CP-OFDM and DFT-s-OFDM waveform with 15kHz SCS.

The results are provided assuming SEM/ACLR according to 20MHz CBW, with PC3 requirements assumed. We evaluated the following cases:

- 20MHz with “Symmetrical extended CBW” of 40MHz (see figure 2a)

- 20MHz with “Asymmetrical extended CBW” of 30MHz (see figure 2b)

Figure 2a: Symmetrical extended CBW evaluation scenario

Figure 2b: Asymmetrical extended CBW evaluation scenario

For Asymmetrical extended CBW, the following specific aspects are highlighted:

- The LO was allocated at the centre of the 20MHz CBW, i.e. 10MHz from left edge of 30MHz ext-CBW.

- The maximum evaluated size of LCRB and corresponding RBstart was selected to ensure that the number of non-allocated RBs (LCRB/2) inside the channel provided sufficient guardband to adhere to existing requirements at the left edge of the channel – values for DFTS and CP-OFDM are different due to restricted RB allocation sizes for DFTS-OFDM.

- We only modelled QPSK so far, but plan to evaluate 16QAM for the next meeting.

Table 2a: Evaluation results for Symmetric extended UE CBW

| Waveform, Mod, Tx power | RB allocation (RBstart, LCRB) | ACLR Pass/fail | SEM Pass/fail |
| --- | --- | --- | --- |
| DFT-s-OFDM, QPSK, 23dBm | {0, 100}, {0, 75}, {0, 50}, {0, 25}, {0, 1} | ALL PASS | ALL PASS |
| CP-OFDM, QPSK, 21.5dBm | {0, 106}, {0, 75}, {0, 50}, {0, 25}, {0, 1} | ALL PASS | ALL PASS |
| DFT-s-OFDM, 16QAM, 22dBm | {0, 100}, {0, 75}, {0, 50}, {0, 25}, {0, 1} | ALL PASS | ALL PASS |
| CP-OFDM, 16QAM, 21dBm | {0, 106}, {0, 75}, {0, 50}, {0, 25}, {0, 1} | ALL PASS | ALL PASS |

Table 2b: Evaluation results for Asymmetrical extended UE CBW

| Waveform, Mod, Tx power | RB allocation (RBstart, LCRB) | ACLR Pass/fail | SEM Pass/fail |
| --- | --- | --- | --- |
| DFT-s-OFDM, QPSK, 23dBm | {25, 50}, {105, 1} | ALL PASS | ALL PASS |
| CP-OFDM, QPSK, 21.5dBm | {35, 71}, {105, 1} | ALL PASS | ALL PASS |

Observation 1: All modelled RB allocations pass the test. These test points covered the most restrictive cases. We believe that this is sufficient to demonstrate that the requirements would be passed for all RB allocations within NRB of the actual UE CBW.

While not all aspects are modelled, we believed it can also be considered that:

- If the UE fulfils the requirements for OOBE integrated over actual UE CBW, then for OOBE integrated over Extended CBW (40MHz and 30MHz respectively) the requirements would also be fulfilled.

- For 30kHz SCS, the same outcome should also apply scaled for the corresponding NRB size adjustment within the UE CBW.

Observation 2: An extended CBW is shown to be feasible for reducing MPR for Outer RB allocation MPR for PC3, for Symmetrical Extended UE CBW (for QPSK and 16QAM) and also appears to be feasible for Asymmetrical extended UE CBW (for QPSK, with 16QAM FFS).

- 4. Conclusion

In this contribution, we have presented our perspectives on power domain enhancement for NR single carrier, and have provided the following observations and proposals:

Proposal 1: Specify the case where extended CBW does not exceed the BS CBW edge.

Proposal 2: For the case where a UE CBW is allocated at the edge of the BS CBW, agree to consider further an “Asymmetrical extended CBW” to allow outer RB allocation MPR reduction applicability for a UE CBW allocated at the edge of the BS CBW.

Proposal 3: Agree that IBE shall apply in the region between UE CBW and extended CBW. Also agree to allow operators to be given control via network signalling to the UE of whether usage of extended UE CBW is permitted within the cell.

Proposal 4: Agree that ΔfOOB shall be the same for ACLR and SEM when extended UE CBW applies.

Proposal 5: Agree that requirements for ACLR and SEM apply from edge of extended CBW.

Proposal 6: Agree to shift the FOOB boundary to the edge of the OOBE for extended CBW.

Observation 1: All modelled RB allocations pass the test. These test points covered the most restrictive cases. We believe that this is sufficient to demonstrate that the requirements would be passed for all RB allocations within NRB of the actual UE CBW.

Observation 2: An extended CBW is shown to be feasible for reducing MPR for Outer RB allocation MPR for PC3, for Symmetrical Extended UE CBW (for QPSK and 16QAM) and also appears to be feasible for Asymmetrical extended UE CBW (for QPSK, with 16QAM FFS).

- 5. References

- RP-2410749, “WF on power domain enhancements”, Huawei.

- R4-2405619, “Discussion on power domain enhancement for NR single carrier”, MediaTek (RAN4#110bis)

- R4-2407910, “Discussion on power domain enhancement for NR single carrier”, MediaTek (RAN4#111)