3GPP TSG-RAN4 Meeting # 112	R4-2412555

# Maastricht, Netherlands, 19th – 23rd August, 2024

Title:	Discussion on co-existence study scenarios and assumptions

Source:	Samsung

Agenda item:	8.8.2.1

Document for:	Approval

1	Introduction

In the RAN#103 meeting, a new WID on enhanced requirements and test methodology for NR and IoT NTN was approved [1] and it was revised in RAN #104 meeting [2]. In [2], the objective includes the co-existence study for NR and IoT NTN high power UE, and the related content is captured below:

High power UE (HPUE) for NTN - Conduct the coexistence analysis based on TR 38.863 for the above example bands and corresponding power classes - Evaluate the impact on the uplink performance of TN (Terrestrial network) - Example bands include bands n256 and 256, n255 and 255 - PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS - For NR-NTN and IoT-NTN: coexistence studies shall be carried out under the same conditions, including geographical separation between NTN UE and Terrestrial Networks (TN), as the co-existence studies in Rel-17 and Rel-18

In this paper, the scenarios and assumptions which are different to previous studies (e.g. TR 38.863) are discussed, and the corresponding proposals are provided to progress the work.

2	Discussion

In this section, the the previous co-ex works in Rel-17 NR-NTN and Rel-18 IoT-NTN are discussed, and the suggested scenarios and assumptions are proposed.

In the WID[2], it clearly stated that the co-existence for this High power UE (HPUE) for NTN should be conducted based on TR 38.863 [3] for the example bands and power classes. Hence, to not duplicate the work and also to save the efforts, it is suggested to adopt the previous work as much as possible.

Proposal 1: According to the WID, it is proposed to adopt the assumptions, topologies, parameters and models from TR 38.863 as baseline for HPUE NR -NTN co-existence studies. The changes to this baseline can be made for the purpose of studying new power classes.

The Rel-18 IoT-NTN studies was carried out later, but also based on TR 38.863, and its agreed assumptions were documented in the approved WF R4-2217473 [4]. The differences from this WF [4] and TR [3] is basically the IoT system parameters, ACIR modelling for narrorwer bandwith, etc.

Proposal 2: It is proposed to adopt the assumptions, topologies, parameters and models from WF-2217473, whcih is also based on TR 38.863, as baseline for HPUE IoT-NTN co-existence studies. The changes to this baseline can be made for the purpose of studying new power classes.

2.1 Scenarios

In Rel-17 and Rel-18, the co-existence studies were conducted between NR-NTN or IoT-NTN with NR traditional terrestrial network (TN) for the following scenarios. And from the co-ex results, the RF requirements were derived from the worst scenarios as indicated below.

Table 1. Summary of Rel-17/Rel-18 NR/IoT NTN co-ex studies

| No. | Combination | Aggressor | Victim | Rel-17 NR-NTN | Rel-18 IoT-NTN |
| --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | TN DL | NTN DL | NR-NTN UE ACS | IoT-NTN UE ACS |
| 2 | TN with NTN | TN UL | NTN UL |  |  |
| 3 | TN with NTN | NTN DL | TN DL | NR-NTN SAN ACLR |  |
| 4 | TN with NTN | NTN UL | TN UL | NR-NTN UE ACLR | IoT NTN UE ACLR |
| 5 | TN with NTN | NTN UL | TN DL |  |  |
| 6 | TN with NTN | TN DL | NTN UL | NR-NTN SAN ACS |  |

In the current scenarios and WID, considering the changes of this WI to the previous studies is only the transmitting power from NTN UL, we should narrow down the studied scenarios which are not impacted by our target. Scenario 1 and 3 are completely not related to NTN UL, which should be down scoped.

Observation 1: Introducing HPUE would not impact the co-ex study results for scenario 1 and 3. Hence these scenarios are out of scope of this WI.

And scenario 2, 3 and 6 which is targeted to define the SAN ACLR or ACS, which was not one of the objectives as agreed in this [3]. And considering introducing the HPUE should not bring changes to the current network deployment, which is also the traditional way, we think the scenario 2, 3 and 6 can be down scoped.

Observation 2: The WID is not intended to change any SAN requirements, by introducing HPUE. Hence, the scenario 2, 3 and 6 are out of scope of this WI.

Among scenario 4 and 5, it can be observed that when NTN UL is the aggressor, the TN UL (i.e. the TN NR gNB) as victim is more sensitive or requires more ACLR compared to TN DL (i.e. TN NR UE). And this observation applies to both NR-NTN and IoT-NTN studies.

Moreover, in both Rel-17 and Rel-18 studies, scenario 4 has worst results when TN gNB using AAS assumptions and NTN SAN using GEO assumptions.

Observation 3: In Rel-17 NR-NTN and Rel-18 IoT-NTN studies, when considering NTN UE Tx as aggressor, the Scenario 4 (NTN UL interfering TN UL) with TN gNB assuming AAS antenna and NTN SAN assuming GEO is the most critical scenario.

Proposal 3: It is proposed that scenario 1, 2, 3 and 6 to be down scoped for this HPUE coex studies for both NR-NTN and IoT-NTN.

Proposal 4: It is proposed to focus co-ex studies for Scenario 4, with NTN UL communicating with GEO as aggressor, and interfering TN UL with AAS gNB for both HPUE NR-NTN and IoT-NTN co-existence studies. And scenario 5 can be deprioritized unless companies later identified even worse findings for co-ex.

2.2 Deployment

Regarding the Scenario 4, the deployment was previously defined in TR 38.863 and adopted for both NR-NTN and IoT-NTN studies as following. And the option 2 described below was finally used to derive conclusion for scenario 4 and the suggested ACLR requirements.

Table 2. Network and UE deployment [3]

| No. | Combination | Aggressor | Victim | Which NTN cell/UE to observe? | Which TN/UE to observe? | Which TN cells in a TN to observe? |
| --- | --- | --- | --- | --- | --- | --- |
| 4 | TN with NTN satellite | NTN UL | TN UL | NTN cell: Nadir point. NTN UE: NTN UEs dropped at the edge of TN clusters | TN randomly placed in this NTN beam | Option 1: All active TN clusters which has the NTN UE(s) at its edge. Option 2: Only the TN sectors which have NTN UE(s) at their edges. Option 1 is the baseline and it is not precluded companies can follow Option 2 to bring results |

Hence, for the HPUE NTN co-ex studies, despite reusing the agreed deployment assumptions from TR 38.863, we propose to start with previous baseline option 1 to proceed the work.

Proposal 5: It is propose to begin co-ex studies with TR 38.863 baseline option 1, which is to observe all active TN clusters which has the NTN UE(s) at its edge, and rest unchanged for deployment assumptions.

2.3 System parameters

In Rel-17 NR-NTN studies, TR 38.863 documented two sets of Satellite parameters, which were originally from TR 38.811. But then in later discussions, the study results and conclusions were focusing on Set-1 of those two sets.

Similarly, in Rel-18 IoT-NTN studies, the co-ex study also focused on Set-1 satellite parameters.

Observation 4: For both Rel-17 NR-NTN and Rel-18 IoT-NTN co-ex studies, Set-1 satellite parameters were focused.

Proposal 6: It is proposed to focus on Set-1 satellite parameters for both NR-NTN and IoT-NTN HPUE coex studies. And set-2 parameters can be deprioritized unless companies later identified even worse findings for co-ex.

2.3.1 IoT NTN system parameters

In Rel-18 IoT-NTN studies, the WF [4] had documented the agreed changes to TR [3], especially for BW, SCS, and number of UL UE to fit the IoT system.

Proposal 7: For IoT-NTN HPUE co-ex, following WF R4-2217473, to use 180kHz as bandwidth assumption.

Proposal 8: For IoT-NTN HPUE co-ex, following WF R4-2217473, to start with following 3 options for UL UE number and SCS combinations.

- Option 1: 9 UL UE with 18kHz SCS single tone

- Option 2: 18 UL UE with 3.75kHz SCS single tone;

- Option 3: 36 UL UE with 3.75kHz SCS single tone;

In a separate paper to this meeting R4-2412556, we shared our preliminary results for the above three options for Scenario 4. And the observation is that the Option 3 created the worst interference from IoT-NTN to NR, and then it’s Option 2 and Option 1.

Observation 5: From the preliminary results in R4-2412556, the 36 UL UE with 3.75kHz SCS single tone will create worst interference from IoT-NTN to NR TN.

2.3.2 High power UE (HPUE)

The major differences introduced to [3] and [4] from this WI is to study the NTN UE with different power class, i.e. the HPUE. By introducing HPUE in co-ex study, by our review, we find it will impact two parts in system parameters:

- UL Tx power

- UL power control

For UL Tx power, it is quite straightforward, the assumptions require update for PC 2, PC 1.5 and PC1. Based on the definition from TS 38.101-1, the max Tx power can be assumed as 23/26/29/31 dBm for PC 3/2/1.5/1. And based on the TS 38.101-1, the min Tx power is not specified as a function of power class, hence we suggest it to be unchanged for different power classes.

Proposal 9: It is proposed to update UE max Tx power assumptions in [3] and [4] for NR-NTN and IoT-NTN with 23/26/29/31 dBm for PC 3/2/1.5/1 respectively. And keep the UE min Tx power as -40 dBm for all power classes.

And with the updates for max power, the uplink power control equation need to be updated slightly. For the equation adopted in [3] and [4] as below, the Rmin would need to be updated as following:

TN UL Transmit Power Control For uplink scenario, TPC model specified in Section 9.1 TR 36.942[8] is applied for TN with following parameters. where: Pmax = 23dBm, Rmin = -63/-66/-69/-71 dB for PC3/2/1.5/1 respectively, CLx-ile and γ are set as following: - CLx-ile = 88 + 10*log10 (200/X) + 11 – Y, where X is UL transmission BW (MHz) and Y is the BS noise figure - γ = 1For uplink scenario,

Proposal 10: It is propose to update the uplink power control parameters accordingly, i.e. to update the Rmin in [3] and [4] to “Rmin = -63/-66/-69/-71 dB for PC3/2/1.5/1 respectively”.

2.4 Others

For the rest assumptions documented in [3] and [4], as we proposed in our Proposal 1 and 2, we haven’t identified the necessity to introduce any changes for HPUE co-ex studies. Hence, from our proposal 1 and 2, we suggest to start with re-using the assumptions for NR-NTN and IoT-NTN from the content in [3] and [4] for the following sections respectively, unless any updates would be agreed later:

- antenna modelling,

- ACIR model, propagation model,

- received power model,

- performance metrics,

- throughput and SNR mapping

- simulation procedure

- methods and principle to process results

2.5 Summary

For the purpose to better document the above mentioned baseline assumptions, as requested in WID, a separate paper R4-2412557 was provided for information to list all the required information for HPUE co-ex studies.

Proposal 11: It is propose to take this R4-2412557 as an informational running document to update and capture latest agreed simulation assumptions in each meeting.

3	Conclusion

For the co-existence study scenarios and assumptions of enhanced requirements and test methodology for NR and IoT NTN, we propose the following.

Proposal 1: According to the WID, it is proposed to adopt the assumptions, topologies, parameters and models from TR 38.863 as baseline for HPUE NR -NTN co-existence studies. The changes to this baseline can be made for the purpose of studying new power classes.

Proposal 2: It is proposed to adopt the assumptions, topologies, parameters and models from WF-2217473, whcih is also based on TR 38.863, as baseline for HPUE IoT-NTN co-existence studies. The changes to this baseline can be made for the purpose of studying new power classes.

Observation 1: Introducing HPUE would not impact the co-ex study results for scenario 1 and 3. Hence these scenarios are out of scope of this WI.

Observation 2: The WID is not intended to change any SAN requirements, by introducing HPUE. Hence, the scenario 2, 3 and 6 are out of scope of this WI.

Observation 3: In Rel-17 NR-NTN and Rel-18 IoT-NTN studies, when considering NTN UE Tx as aggressor, the Scenario 4 (NTN UL interfering TN UL) with TN gNB assuming AAS antenna and NTN SAN assuming GEO is the most critical scenario.

Proposal 3: It is proposed that scenario 1, 2, 3 and 6 to be down scoped for this HPUE coex studies for both NR-NTN and IoT-NTN.

Proposal 4: It is proposed to focus co-ex studies for Scenario 4, with NTN UL communicating with GEO as aggressor, and interfering TN UL with AAS gNB for both HPUE NR-NTN and IoT-NTN co-existence studies. And scenario 5 can be deprioritized unless companies later identified even worse findings for co-ex.

Proposal 5: It is propose to begin co-ex studies with TR 38.863 baseline option 1, which is to observe all active TN clusters which has the NTN UE(s) at its edge, and rest unchanged for deployment assumptions.

Observation 4: For both Rel-17 NR-NTN and Rel-18 IoT-NTN co-ex studies, Set-1 satellite parameters were focused.

Proposal 6: It is proposed to focus on Set-1 satellite parameters for both NR-NTN and IoT-NTN HPUE coex studies. And set-2 parameters can be deprioritized unless companies later identified even worse findings for co-ex.

Proposal 7: For IoT-NTN HPUE co-ex, following WF R4-2217473, to use 180kHz as bandwidth assumption.

Proposal 8: For IoT-NTN HPUE co-ex, following WF R4-2217473, to start with following 3 options for UL UE number and SCS combinations.

- Option 1: 9 UL UE with 18kHz SCS single tone

- Option 2: 18 UL UE with 3.75kHz SCS single tone;

- Option 3: 36 UL UE with 3.75kHz SCS single tone;

Observation 5: From the preliminary results in R4-2412556, the 36 UL UE with 3.75kHz SCS single tone will create worst interference from IoT-NTN to NR TN.

Proposal 9: It is proposed to update UE max Tx power assumptions in [3] and [4] for NR-NTN and IoT-NTN with 23/26/29/31 dBm for PC 3/2/1.5/1 respectively. And keep the UE min Tx power as -40 dBm for all power classes.

Proposal 10: It is propose to update the uplink power control parameters accordingly, i.e. to update the Rmin in [3] and [4] to “Rmin = -63/-66/-69/-71 dB for PC3/2/1.5/1 respectively”.

Proposal 11: It is propose to take this R4-2412557 as an informational running document to update and capture latest agreed simulation assumptions in each meeting.

4	Reference

[1] RP-240857, New WID on Enhanced requirements and test methodology for NR and IoT NTN, Samsung, Xiaomi, RAN #103.

[2] RP-241281, Revised WID on Enhanced requirements and test methodology for NR and IoT NTN, Samsung, Xiaomi, RAN #104.

[3] TR 38.863

[4] R4-2217473, Way Forward on coexistence study for IoT over NTN, MediaTek

[5] TS 38.101-1