3GPP TSG-RAN4 Meeting # 112	R4-2412556

# Maastricht, Netherlands, 19th – 23rd August, 2024

Title:	Preliminary HPUE NR-NTN and IoT-NTN co-ex results

Source:	Samsung

Agenda item:	8.8.2.1

Document for:	Information

1	Introduction

In the RAN#103 meeting, a new WID on enhanced requirements and test methodology for NR and IoT NTN was approved [1] and it was revised in RAN #104 meeting [2]. In [2], the objective includes the co-existence study for NR and IoT NTN high power UE, and the related content is captured below:

High power UE (HPUE) for NTN - Conduct the coexistence analysis based on TR 38.863 for the above example bands and corresponding power classes - Evaluate the impact on the uplink performance of TN (Terrestrial network) - Example bands include bands n256 and 256, n255 and 255 - PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS - For NR-NTN and IoT-NTN: coexistence studies shall be carried out under the same conditions, including geographical separation between NTN UE and Terrestrial Networks (TN), as the co-existence studies in Rel-17 and Rel-18

In this paper, we share the preliminary results from our side for NR-NTN and IoT-NTN based on Rel-17 NR-NTN and Rel-18 IoT-NTN assumptions.

2	Preliminary results

In this section, we share our preliminary results for Scenario 4, and assuming NTN UE connecting with GEO SAN, and TN gNB with AAS antenna.

2.1 NR-NTN HPUE with NR-TN

In this section, we share the preliminary results between NR-NTN HPUE, with power class 3, 2, 1.5 and 1, and NR-TN. The scenario is NTN UL interfering TN UL, which is NTN HPUE transmitting as aggressor and TN gNB receiving as victim. The throughput loss of victim NR-TN gNB with regards to the different aggressor NR-NTN HPUE ACLR is shown in the table below.

Table 1. Throughput loss w.r.t. different ACLR for PC 3/2/1.5/1 and Scenario 4

|  | Average Throughput Loss (%) |  |  |  |  | Cell-edge Throughput Loss (%) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ACLR | 20 | 25 | 30 | 35 | 40 | 20 | 25 | 30 | 35 | 40 |
| PC3 | 1.91 | 0.97 | 0.49 | 0.25 | 1.31 | 9.31 | 4.84 | 2.53 | 1.23 | 0.67 |
| PC2 | 2.79 | 1.44 | 0.73 | 0.37 | 0.20 | 13.49 | 6.67 | 3.48 | 1.75 | 1.00 |
| PC1.5 | 4.07 | 2.16 | 1.11 | 0.57 | 0.30 | 20.88 | 10.22 | 5.24 | 2.71 | 1.49 |
| PC1 | 5.23 | 2.83 | 1.48 | 0.77 | 0.42 | 26.86 | 13.81 | 7.11 | 3.80 | 2.18 |

Observation 1: The results from PC3 aligned with the Rel-17 NR-NTN discussions, which somehow can indicate that the co-ex platform is aligned with the previous Rel-17 NR-NTN studies.

Observation 2: The results for PC 2/1.5/1 will need to further review with the final agreed assumptions and also collaborate with other companies results to determine required ACLR for different HPUE power classes.

2.2 IoT-NTN HPUE with NR-TN

In this section, we share the preliminary results between IoT -NTN HPUE, with power class 3, 2 and 1, and NR-TN. The scenario is NTN UL interfering TN UL, which is NTN HPUE transmitting as aggressor and TN gNB receiving as victim. The throughput loss of victim NR-TN gNB with regards to the different aggressor NR-NTN HPUE ACLR is shown in the table below.

Table 2-1. Throughput loss w.r.t. different ACLR for PC 3/2/1 and Scenario 4

| 36 x 3.75kHz | Average Throughput Loss (%) |  |  |  |  | Cell-edge Throughput Loss (%) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ACLR | 20 | 25 | 30 | 35 | 40 | 20 | 25 | 30 | 35 | 40 |
| PC3 | 1.21 | 0.63 | 0.36 | 0.24 | 0.19 | 6.10 | 3.13 | 1.47 | 0.72 | 0.44 |
| PC2 | 1.73 | 0.91 | 0.52 | 0.35 | 0.29 | 8.33 | 4.60 | 2.48 | 1.44 | 1.02 |
| PC1.5 | 2.78 | 1.49 | 0.87 | 0.59 | 0.49 | 13.54 | 7.51 | 4.32 | 2.86 | 2.25 |

Table 2-2. Throughput loss w.r.t. different ACLR for PC 3/2/1 and Scenario 4

| 9 x 15kHz | Average Throughput Loss (%) |  |  |  |  | Cell-edge Throughput Loss (%) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ACLR | 20 | 25 | 30 | 35 | 40 | 20 | 25 | 30 | 35 | 40 |
| PC3 | 0.32 | 0.17 | 0.09 | 0.06 | 0.05 | 1.55 | 0.83 | 0.37 | 0.18 | 0.11 |
| PC2 | 0.50 | 0.26 | 0.15 | 0.10 | 0.08 | 2.34 | 1.25 | 0.71 | 0.40 | 0.28 |
| PC1.5 | 0.93 | 0.49 | 0.28 | 0.19 | 0.16 | 4.58 | 2.42 | 1.47 | 1.03 | 0.84 |

Observation 3: The results from PC3 aligned with the Rel-18 IoT-NTN discussions, which somehow can indicate that the co-ex platform is aligned with the previous Rel-17 NR-NTN studies.

Observation 4: 36 UL IoT NTN UE with SCS 3.75kHz results in worst interference to NR TN network, which is aligned with the findings from Rel-18 IoT-NTN studies.