3GPP TSG-RAN4 Meeting # 112	R4-2412558

# Maastricht, Netherlands, 19th – 23rd August, 2024

Title:	Discussion on NTN HPUE Tx requirements

Source:	Samsung

Agenda item:	8.8.2.2

Document for:	Approval

1	Introduction

In the RAN#103 meeting, a new WID on enhanced requirements and test methodology for NR and IoT NTN was approved [1] and it was revised in RAN #104 meeting [2]. In [2], the objective include to specify HPUE for NR-NTN and IoT-NTN for single uplink (UL) carrier, and the related content is captured below:

High power UE (HPUE) for NTN - Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study. - Support other power classes based on the co-existence and feasibility study - Non-hand held UE is prioritized - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands - Necessary Tx requirements, at least including - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary

In this paper, we share our initial views and observations to the agreed objectives on Tx requirements aspects.

2	Power classes

In the objectives of the WID, the following power classes were considered:

PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS

The WID includes the power classes 2, 1.5 and 1 for NR-NTN and IoT-NTN, however the PC 1.5 was specified for NR-NTN only. And the co-existence studies were proposed by the WID before specifying any of the above-mentioned power classes.

Observation 1: For NR-NTN, the power class 2, 1.5 and 1 are considered at initial stage. For IoT-NTN, the power class 2 and 1 are considered in initial stage.

Observation 2: The consideration of the power classes depend on the outcome of co-existence studies.

Proposal 1: The power classes 2, 1.5 and 1 for NR-NTN, power classes 2 and 1 for IoT-NTN are all considered for RF requirement discussion at starting point, and it will depend on the outcomes of co-existence studies.

3	Handheld and non-Handheld

In the objectives of the WID, following content contioned the handheld and non-handheld requirements.

- Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study. - Support other power classes based on the co-existence and feasibility study - Non-hand held UE is prioritized - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands - Necessary Tx requirements, at least including - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE

With regards to the different considerations for handheld and non-handheld UE in terms of RF requirements, the WID mentioned the feasibility study and SAR (Specific absorption rate) solutions can be different for handheld UE and non-handheld UE. And the SAR solution is specified only for handheld UE other than non-handheld UE according to WID.

Proposal 2: The RF requirements could be different for handheld UE and non-handheld UE. The discussion can start with one set of requirement, and whether two sets of requirements will be needed can depend on the co-existence and feasibility studies later.

Observation 3: SAR solution is to be specified only for handheld UE. However, this depends on whether the handheld UE and non-handheld UE eventually need two sets of RF requirements.

In current 3GPP spec, the P-MPR has been introduced and widely adopted for the UE to solve the SAR issues world wide, especially for the FDD bands as the example bands listed for this WI. Hence, in this WI, the P-MPR can be a mature and starightforward solution to meet SAR requirements.

Proposal 3: It is proposed to adopt P-MPR for SAR solution for handheld NTN HPUE as the starting point.

3	Tx requirements

In the WID, it only mentions the Tx requirements at least include SAR, MPR and A-MPR. And the major impact is by inrtoducing high power UE with new power classes to the existing NTN UE specifications.

Observation 4: Given the difference in structure and technology, the NR-NTN and IoT-NTN HPUE RF requirements are suggested to be seaprately discussed.

Observation 5: For NR-NTN HPUE, if we take the current structure of TS 38.101-5, the following Tx requirements can be impacted depending on further discussions and agreements:

| Section | Expected changes/update |
| --- | --- |
| 6.2 Transmitted power 6.2.1 UE maximum output power 6.2.2 UE maximum output power reduction 6.2.3 UE additional maximum output power 6.2.4 Configured transmitted power | For 6.2.1: The MOP requirements should be introduced for the new power classes, which would be concluded by the co-existence and/or feasibility studies, considering existing TN requirements as a reference. For 6.2.2: The MPR requirement should be introduced based on the agreed ACLR for the agreed power classes and study results, considering existing TN requirements as a reference. For 6.2.3 The A-MPR requirement should be introduced for the agreed power classes, considering existing TN requirements as a reference. For 6.2.4: The SAR solution (e.g. P-MPR) should be introduced for the HPUE, depending on discussion. |
| 6.5 Output RF spectrum emission 6.5.2 Out of band emission | For 6.5.2: The new ACLR requirements should be introduced for the new power classes, which would be concluded by the co-existence and/or feasibility studies. |

Observation 6: Similarly, for IoT-NTN HPUE, if we take the current structure of TS 36.102, the following Tx requirements can be impacted depending on further discussions and agreements:

- 6.2A.1 UE maximum output power for category M1;

- 6.2A.2 UE maximum output power reduction for category M1;

- 6.2A.3 UE additional maximum output power reduction for category M1;

- 6.2A.4 Configured transmitted power for category M1;

- 6.2B.1 UE maximum output power for category NB1 and NB2;

- 6.2B.2 UE maximum output power reduction for category NB1 and NB2;

- 6.2B.3 UE additional maximum output power reduction for category NB1 and NB2;

- 6.2B.4 Configured transmitted power for category NB1 and NB2;

- 6.5A.3 Out of band emission for category M1;

- 6.5B.3 Out of band emission for category NB1 and NB2.

Given the most existing NTN UE RF requirements in existing TS 38.101-5 and TS 36.102 are adopting the same structure or even referring directly to the traditional (terrestrial) 3GPP specs, the HPUE requirements for above mentioned impacted items of TS 38.101-1 and TS 36.101 can still be the reference to accelarate the work.

Observation 7: The HPUE requirements from TS 38.101-1 and TS 36.101 can still be the reference to avoid duplicate effort for the NR-/IoT-NTN HPUE in TS 38.101-5 and TS 36.102 respectively.

3	Conclusion

For the Tx requirements of enhanced requirements and test methodology for NR and IoT NTN, we propose the following.

Observation 1: For NR-NTN, the power class 2, 1.5 and 1 are considered at initial stage. For IoT-NTN, the power class 2 and 1 are considered in initial stage.

Observation 2: The consideration of the power classes depend on the outcome of co-existence studies.

Proposal 1: The power classes 2, 1.5 and 1 for NR-NTN, power classes 2 and 1 for IoT-NTN are all considered for RF requirement discussion at starting point, and it will depend on the outcomes of co-existence studies.

Proposal 2: The RF requirements could be different for handheld UE and non-handheld UE. The discussion can start with one set of requirement, and whether two sets of requirements will be needed can depend on the co-existence and feasibility studies later.

Observation 3: SAR solution is to be specified only for handheld UE. However, this depends on whether the handheld UE and non-handheld UE eventually need two sets of RF requirements.

Proposal 3: It is proposed to adopt P-MPR for SAR solution for handheld NTN HPUE as the starting point.

Observation 4: Given the difference in structure and technology, the NR-NTN and IoT-NTN HPUE RF requirements are suggested to be seaprately discussed.

Observation 5: For NR-NTN HPUE, if we take the current structure of TS 38.101-5, the following Tx requirements can be impacted depending on further discussions and agreements:

| Section | Expected changes/update |
| --- | --- |
| 6.2 Transmitted power 6.2.1 UE maximum output power 6.2.2 UE maximum output power reduction 6.2.3 UE additional maximum output power 6.2.4 Configured transmitted power | For 6.2.1: The MOP requirements should be introduced for the new power classes, which would be concluded by the co-existence and/or feasibility studies, considering existing TN requirements as a reference. For 6.2.2: The MPR requirement should be introduced based on the agreed ACLR for the agreed power classes and study results, considering existing TN requirements as a reference. For 6.2.3 The A-MPR requirement should be introduced for the agreed power classes, considering existing TN requirements as a reference. For 6.2.4: The SAR solution (e.g. P-MPR) should be introduced for the HPUE, depending on discussion. |
| 6.5 Output RF spectrum emission 6.5.2 Out of band emission | For 6.5.2: The new ACLR requirements should be introduced for the new power classes, which would be concluded by the co-existence and/or feasibility studies. |

Observation 6: For IoT-NTN HPUE, if we take the current structure of TS 36.102, the following Tx requirements can be impacted depending on further discussions and agreements:

- 6.2A.1 UE maximum output power for category M1;

- 6.2A.2 UE maximum output power reduction for category M1;

- 6.2A.3 UE additional maximum output power reduction for category M1;

- 6.2A.4 Configured transmitted power for category M1;

- 6.2B.1 UE maximum output power for category NB1 and NB2;

- 6.2B.2 UE maximum output power reduction for category NB1 and NB2;

- 6.2B.3 UE additional maximum output power reduction for category NB1 and NB2;

- 6.2B.4 Configured transmitted power for category NB1 and NB2;

- 6.5A.3 Out of band emission for category M1;

- 6.5B.3 Out of band emission for category NB1 and NB2.

Observation 7: The HPUE requirements from TS 38.101-1 and TS 36.101 can still be the reference to avoid duplicate effort for the NR-/IoT-NTN HPUE in TS 38.101-5 and TS 36.102 respectively.

4	Reference

[1] RP-240857, New WID on Enhanced requirements and test methodology for NR and IoT NTN, Samsung, Xiaomi, RAN #103.

[2] RP-241281, Revised WID on Enhanced requirements and test methodology for NR and IoT NTN, Samsung, Xiaomi, RAN #104.