3GPP TSG-RAN4 Meeting # 112	R4-2412559

# Maastricht, Netherlands, 19th – 23rd August, 2024

Title:	Discussion on NTN HPUE Rx requirements

Source:	Samsung

Agenda item:	8.8.2.3

Document for:	Approval

1	Introduction

In the RAN#103 meeting, a new WID on enhanced requirements and test methodology for NR and IoT NTN was approved [1] and it was revised in RAN #104 meeting [2]. In [2], the objective include to specify HPUE for NR-NTN and IoT-NTN for single uplink (UL) carrier, and the related content is captured below:

High power UE (HPUE) for NTN - Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study. - Support other power classes based on the co-existence and feasibility study - Non-hand held UE is prioritized - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands - Necessary Tx requirements, at least including - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary

In this paper, we share our initial views and observations to the agreed objectives on Rx requirements aspects.

2	Rx requirements

According to the WID, and also from previous HPUE spec disucssions, the reference sensitivity degradation (RSD) need to be specified for the later agreed power classes as a relaxation to the reference sensitivity defined for PC3 as in existing TS 38.101-5 and TS 36.102. This is due to the worsen reception sensitivty by increasing the transmitting power due to higher power levels.

The previous discussions for FR1-NTN and IoT-NTN has adopted the traditional 3GPP UE RF architecture and components as the assumptions, and also the example bands in the WID is fully or partially overlapping with the existing 3GPP bands as well.

With this, the refsense degradation due to different power classes can also use the RSD requirements for different power classes from TS 38.101-1 and TS 36.101 as reference. And it is encouraged that the interested companies to provide any analysis for NR-NTN and especially IoT-NTN considering different characteristics of RF compoenent for NTN HPUEs scenarios if different to the previous assumptions.

Observation 1: The reference senstivity degradation will be impacted by the introduction of HPUE, and it is encouraged that companies to provide the analysis for the actual RSD values for the agreed power classes based on co-existence and feasibility studies later.

3	Conclusion

For the Rx requirements of enhanced requirements and test methodology for NR and IoT NTN, we propose the following.

Observation 1: The reference senstivity degradation will be impacted by the introduction of HPUE, and it is encouraged that companies to provide the analysis for the actual RSD values for the agreed power classes based on co-existence and feasibility studies later.

4	Reference

[1] RP-240857, New WID on Enhanced requirements and test methodology for NR and IoT NTN, Samsung, Xiaomi, RAN #103.

[2] RP-241281, Revised WID on Enhanced requirements and test methodology for NR and IoT NTN, Samsung, Xiaomi, RAN #104.