3GPP TSG-RAN4 Meeting # 112	R4-2412560

# Maastricht, Netherlands, 19th – 23rd August, 2024

Title:	Discussion on Ku-band co-ex studies

Source:	Samsung

Agenda item:	8.9.2

Document for:	Approval

1	Introduction

In the RAN#104 meeting, a new WID on Introduction of Ku Band for NR-NTN was approved [1], and in which the objectives include to conduct coexistence studies as shown below:

The following assumptions are taken as the baseline for this work: - GSO and NGSO - NTN capable UE type 1 to 5 (38.101-5 Table 9.2.1.0-1) for above 10 GHz (no handheld devices) - FDD Mode - GNSS Receiver capability The objectives are: - Use the regulation requirements in ITU Regions 1, 2 and 3 to identify relevant adjacent band co-existence scenarios for NTN Ku Band covering the following frequency ranges, considering targeted deployment scenarios: - For ITU regions 1 and 3: - Downlink 10.70 – 12.75 GHz - Uplink 13.75-14.5 GHz - For ITU regions 2 excluding US: - Downlink 10.70 – 12.70 GHz - Uplink 13.75-14.5 GHz - For ITU regions 1 and 3 - Downlink 10.70 – 12.75 GHz - Uplink 12.75 - 13.25 GHz - For ITU regions 2 excluding US - Downlink 10.70 – 12.70 GHz - Uplink 12.70 - 13.25 GHz - Note: Appropriate TX/Rx separation for uplink downlink pairing will be required. - Conduct coexistence study. Reuse Ka band coexistence study assumptions as much as possible, with modifications where necessary, e.g. for frequency range and subcarrier spacing. Conduct new co-existence studies based on adjacent services . - Specify RF requirements for satellite access node and NTN VSAT types in 38.101-5 also considering existing regulations on antenna sizes for certain parts of the Ku band. - Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B to enable the future possibility of supporting satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths based on existing channel bandwidths. Potential approaches for achieving this shall be studied, from which one shall be selected including but not limited to: - Ku band #1a and #1b as FR1-NTN supporting 20 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing - RAN4 will select which one of the above approaches for DL and UL will be pursued by RAN #106. - Priority 2: for all ITU regions excluding the US in region 2, using the approach in Priority 1, define Ku band #2a and #2b as appropriate for the downlink block and uplink block A to support satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths. - Specify RRM requirements Note: Support for NTN CA is not in the scope of this WID

In this paper, we share our initial views and observations on the differences of this Ku-band co-ex studies compared to the previous Ka-band co-ex studies for discussion.

2	Deployment scenarios

In previous Ka-band studies, the co-ex scenario assumed the NTN UE in FR1 and FR2 is highly possible to support TN bands. And following this assumption, the co-ex studies adopted an isolation distance between the NTN network and TN network. It is because when an UE is close to the TN network on the ground within a certain range, it always received better SNR compared to the SNR from NTN SAN.

However, there’s a question mark whether the above assumption is still true with the introduction of Ku-band. The spectrum studied in the WID[1] is in 10~15GHz range where it was not the traditional 3GPP bands and products were developed and deployed. In this case, the devices working for Ku-band may be highly possible not supporting the TN network for a certain period of time. In this case, the isolation distances assumed in Ka-band study may not be applied to the Ku-band studies.

Observation 1: The isolation distance adopted in previous R17 FR1-NTN and R18 FR2-NTN is based on the assumption that the UE probably will support both NTN and TN bands, so the NTN UE will not connect to a NTN SAN if it is within a certain range, i.e. the isolation distance, to a TN network on the ground.

Observation 2: The UE operating in Ku-band probably will not support the TN network, given the 3GPP network was not widely developed or deployed in this frequency ranges. Thus the isolation distance may not be applicable to Ku-band co-ex studies.

Proposal 1: Given the difference between Ku-band and Ka-band, the isolation setting between TN and NTN should be revisited for co-ex studies. The co-ex without isolation distance, i.e. geographical overlapping case should not be skipped for Ku-band study from the beginning.

3	System characteristics

The WID[1] provided two settings for the channel bandwidth and SCS settings for the proposed Ku-band, which is different to the previous Ka-band studies. The two settings are the following:

- Ku band #1a and #1b as FR1-NTN supporting 20 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing

- Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing

These proposed bandwidth and SCS should be taken into account in the co-ex studies.

Proposal 2: The WID proposed channel bandwidth and SCS combinations should be taken into account for the Ku-band studies as the difference to previous studies. Among those, the worst case should be evaluated for co-ex purpose.

Besides, it is a question mark to us whether the previous Set-1 and Set-2 satellite Tx and Rx parameters from TR 38.811 are still applicable to this Ku-band.

Proposal 3: It is proposed to discuss and confirm whether Set-1 and Set-2 satellite Tx and Rx parameters are still applicable to this Ku-band studies.

4	NTN VSAT

Given in the WID, it clearly mentioned the existing regulation for antenna size, we encourage the proponent companies to provide more information about the regulations, and this should be adopted in the co-ex studies. And the survey may be needed to better understand other regulations that may impact the Tx and Rx of NTN VSAT in early stage. As those regulations need to be considered in the co-ex studies, to make sure the co-ex studies is practical and valid.

Proposal 4: The existing regulation, as mentioned by the WID, should be considered and taken into account when making assumptions on NTN VSAT antenna size, gain, max power, etc.

Also, when considering uplink power control for NTN uplink, we adopted the assumption of target SNR as 15 dB, which is simply borrowed from TN networks.

Observation 3: In previous NTN coex studies, 15dB as uplink power control SNR target was adopted which is the same as TN networks. This assumption results in significant large portion of the NTN UE studied were transmitting in max Tx power allowed.

However, given the different use scenario of NTN, we suggest to re-consider this 15dB SNR target for uplink power control. And it is encouraged to the proponent companies to provide a practical value for the uplink power control target in their existing and/or planned services for Ku-band, and the co-ex studies can take this value as an update to the previous assumptions.

Proposal 5: A different uplink control SNR target other than 15dB can be considered for Ku-band depending on the inputs from satellite companies.

5	Conclusion

For the assumptions for co-ex studies in Ku-band NTN, we propose the following.

Observation 1: The isolation distance adopted in previous R17 FR1-NTN and R18 FR2-NTN is based on the assumption that the UE probably will support both NTN and TN bands, so the NTN UE will not connect to a NTN SAN if it is within a certain range, i.e. the isolation distance, to a TN network on the ground.

Observation 2: The UE operating in Ku-band probably will not support the TN network, given the 3GPP network was not widely developed or deployed in this frequency ranges. Thus the isolation distance may not be applicable to Ku-band co-ex studies.

Proposal 1: Given the difference between Ku-band and Ka-band, the isolation setting between TN and NTN should be revisited for co-ex studies. The co-ex without isolation distance, i.e. geographical overlapping case should not be skipped for Ku-band study from the beginning.

Proposal 2: The WID proposed channel bandwidth and SCS combinations should be taken into account for the Ku-band studies as the difference to previous studies. Among those, the worst case should be evaluated for co-ex purpose.

Proposal 3: It is proposed to discuss and confirm whether Set-1 and Set-2 satellite Tx and Rx parameters are still applicable to this Ku-band studies.

Proposal 4: The existing regulation, as mentioned by the WID, should be considered and taken into account when making assumptions on NTN VSAT antenna size, gain, max power, etc.

Observation 3: In previous NTN coex studies, 15dB as uplink power control SNR target was adopted which is the same as TN networks. This assumption results in significant large portion of the NTN UE studied were transmitting in max Tx power allowed.

Proposal 5: A different uplink control SNR target other than 15dB can be considered for Ku-band depending on the inputs from satellite companies.

6	Reference

[1] RP-241690, New WID on Introduction of Ku Band for NR NTN