3GPP TSG-RAN4 Meeting # 112	R4-2412561

# Maastricht, Netherlands, 19th – 23rd August, 2024

Title:	Discussion on Ku-band UE RF

Source:	Samsung

Agenda item:	8.9.4

Document for:	Approval

1	Introduction

In the RAN#104 meeting, a new WID on Introduction of Ku Band for NR-NTN was approved [1], and in which the objectives include to conduct coexistence studies as shown below:

The following assumptions are taken as the baseline for this work: - GSO and NGSO - NTN capable UE type 1 to 5 (38.101-5 Table 9.2.1.0-1) for above 10 GHz (no handheld devices) - FDD Mode - GNSS Receiver capability The objectives are: - Use the regulation requirements in ITU Regions 1, 2 and 3 to identify relevant adjacent band co-existence scenarios for NTN Ku Band covering the following frequency ranges, considering targeted deployment scenarios: - For ITU regions 1 and 3: - Downlink 10.70 – 12.75 GHz - Uplink 13.75-14.5 GHz - For ITU regions 2 excluding US: - Downlink 10.70 – 12.70 GHz - Uplink 13.75-14.5 GHz - For ITU regions 1 and 3 - Downlink 10.70 – 12.75 GHz - Uplink 12.75 - 13.25 GHz - For ITU regions 2 excluding US - Downlink 10.70 – 12.70 GHz - Uplink 12.70 - 13.25 GHz - Note: Appropriate TX/Rx separation for uplink downlink pairing will be required. - Conduct coexistence study. Reuse Ka band coexistence study assumptions as much as possible, with modifications where necessary, e.g. for frequency range and subcarrier spacing. Conduct new co-existence studies based on adjacent services . - Specify RF requirements for satellite access node and NTN VSAT types in 38.101-5 also considering existing regulations on antenna sizes for certain parts of the Ku band. - Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B to enable the future possibility of supporting satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths based on existing channel bandwidths. Potential approaches for achieving this shall be studied, from which one shall be selected including but not limited to: - Ku band #1a and #1b as FR1-NTN supporting 20 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing - RAN4 will select which one of the above approaches for DL and UL will be pursued by RAN #106. - Priority 2: for all ITU regions excluding the US in region 2, using the approach in Priority 1, define Ku band #2a and #2b as appropriate for the downlink block and uplink block A to support satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths. - Specify RRM requirements Note: Support for NTN CA is not in the scope of this WID

In this paper, we share our initial views and observations on the differences of this Ku-band UE RF compared to the previous Ka-band UE RF specifications in current TS 38.101-5.

2	UE RF impacts

Following the WID, the co-ex studies should be conducted first, and the UE RF requirements can be detailed discussed with the conclusion of the study.

Observation 1: The detailed UE RF requirements discussions can wait for the outcomes of the co-ex studies.

As mentioned by the WID, the regulatory requirements in Ku-band in different ITU regions should be taken into account when defining the requirements, such as MPR, A-MPR. And also the WID mentioned the UE antenna sizes and other reuglatory measures for sharing the spectrum with other services.

Given the complexity we had experienced in previous works, it is suggested to start with a thoroughly survey of the regulatory requirements in parallel with the co-ex studies, and these information can be helpful for defining the actual requirements when the co-ex conclusions are available.

Observation 2: The reuglatory requirements are better to be collected in parallel with the co-ex studies, and these information will be helpful for defining the Tx and Rx requirements for NTN VSAT in Ku-band after the co-ex study results are available.

3	Conclusion

For the UE RF for Ku-band NTN, we propose the following.

Observation 1: The detailed UE RF requirements discussions can wait for the outcomes of the co-ex studies.

Observation 2: The reuglatory requirements are better to be collected in parallel with the co-ex studies, and these information will be helpful for defining the Tx and Rx requirements for NTN VSAT in Ku-band after the co-ex study results are available.

5	Reference

[1] RP-241690, New WID on Introduction of Ku Band for NR NTN