3GPP TSG-RAN4 Meeting # 112	R4-2412562

# Maastricht, Netherlands, 19th – 23rd August, 2024

Title:	Discussion on Ambient IoT deployment scenarios and spectrum usage

Source:	Samsung

Agenda item:	8.20.2.1

Document for:	Approval

1	Introduction

In the RAN4#111 meeting, the WF R4-2410567 [1] had achieved progress for the co-existence evaluation on Ambient-IoT, but also left open issues for further discussions. In this paper, the remaining issues will be discussed and the solutions are proposed to further progress the work.

2	Discussions

2.1 Deployment scenarios

From the latest WID RP-240826 [2], the objectives clearly instructed to target on the deployment focusing on co-site scenarios for both D1T1 and D2T2:

- Deployment Scenarios with the following characteristics, referenced to the tables in Clause 4.2.2 of TR 38.848: - Deployment scenario 1 with Topology 1 - Basestation and coexistence characteristics: Micro-cell, co-site - Deployment scenario 2 with Topology 2 and UE as intermediate node, under network control - Basestation and coexistence characteristics: Macro-cell, co-site - The location of intermediate node is indoor

Observation 1: The WID clearly instructed the deployment scenario 1 with topology 1 and deployment scenario 2 with topology 2 will focus on co-site scenarios for co-existence.

Where the referenced Clause 4.2 of TR 38.848[3] is listed below:

4.2.0 Introduction Deployment scenarios for Ambient IoT have been studied on the basis of a list of characteristics, and the representative use case(s) applicable to a scenario. The possible descriptions of the characteristics are as follows: Table 4.2.0-1: Characteristics of deployment scenarios Characteristic; Possible description entries. Environment (of the device); Indoor Outdoor Indoor or outdoor. Basestation characteristic (if any); Macro-cell-based deployment Micro-cell-based deployment Pico-cell-based deployment None. Connectivity topology; See section 4.2.1. Spectrum; Licensed FDD Licensed TDD Unlicensed Note: In each connectivity topology of the study, if a BS is present, it is assumed that the BS uses licensed spectrum. Coexistence with existing 3GPP technologies; Deployed on the same sites as an existing 3GPP deployment corresponding to the basestation type. Deployed on new sites without an assumption of an existing 3GPP deployment.. Traffic assumption; Device-terminated (DT) Device-originated (DO) DO traffic includes DO autonomous (DO-A), and DO device-terminated triggered (DO-DTT). Device characteristic; See Section 4.3: Device A Device B Device C. ... 4.2.2 Deployment scenarios 4.2.2.1 Deployment scenario 1: Device indoors, basestation indoors With Ambient IoT device indoors and basestation indoors, this deployment scenario is characterized according to Table 4.2.2.1-1. Table 4.2.2.1-1: Characteristics of deployment scenario 1 Applicable representative use cases; Characteristics; Description (NOTE 1). Indoor inventory Indoor sensor Indoor positioning Indoor command; Environment (of device); Indoor. Basestation characteristic (if any); Micro- or pico-cell. Connectivity topology; Topology (1), (2), (3). Spectrum; Licensed FDD, licensed TDD, unlicensed. Coexistence with existing 3GPP technologies; Co-site or new site. Traffic assumption; DT and DO. Device characteristic; Device A or Device B or Device C. NOTE 1: Descriptions may not be applicable for some Devices (A, B). 4.2.2.2 Deployment scenario 2: Device indoors, basestation outdoors With Ambient IoT device indoors and basestation outdoors, this deployment scenario is characterized according to Table 4.2.2.2-1. Table 4.2.2.2-1: Characteristics of deployment scenario 2 Applicable representative use cases; Characteristics; Description (NOTE 1). Indoor inventory Indoor sensor Indoor positioning Indoor command; Environment (of device); Indoor. Basestation characteristic (if any); Macro- or Micro- cell BS. Connectivity topology; Topology (1), (2), (3) Note: The location of intermediate or assisting node (if any) is indoor or outdoor. Spectrum; Licensed FDD, licensed TDD, unlicensed. Coexistence with existing 3GPP technologies; Co-site or new site. Traffic assumption; DT and DO. Device characteristic; Device C may support Topology (1), (2), (3), Device A may support Topology (2), Device B may support Topology (2), (3). NOTE 1: Descriptions may not be applicable for some Devices (A, B).

Observation 2: The TR 38.848[3], which the objectives in WID[2] was referring, described the co-existence for A-IoT considered either ‘co-site’ or ‘new site’, where

- ‘co-site’ for ‘Deployed on the same sites as an existing 3GPP deployment corresponding to the basestation type.’

- ‘new-site’ for ‘Deployed on new sites without an assumption of an existing 3GPP deployment.’

The co-existence study requested to RAN4 is between A-IoT and NR, where the existing 3GPP deployment is assumed together with the A-IoT in the same area. Our understanding is that it is quite straightforward that the ‘co-site’ should be the baseline assumption for co-existence study from the content of WID and TR, and the nature of RAN4 co-ex study.

Observation 3: The existing 3GPP deployment should be the baseline assumption given the nature of RAN4 co-ex study. With this, the TR and WID clearly mapped to the ‘co-site’ scenario for NR and A-IoT, and should be focused in RAN4 co-ex studies.

However, in the current WF[1], the co-site scenario (option 2-1 and 2-2) are listed as FFS, while the ‘new-site’ options (option 1-1 and 1-2) are listed as to first evaluate. The current WF is not aligned with our understanding from the WID and TR. Hence, we propose to list the co-site scenarios (option 2-1 and 2-2)

Another rationale we’d like to discuss is for the current mismatch between the objectives and the current co-ex scopes, the WID asked to study the following:

- Deployment scenario 1 with Topology 1

  - Basestation and coexistence characteristics: Micro-cell, co-site

- Deployment scenario 2 with Topology 2 and UE as intermediate node, under network control

  - Basestation and coexistence characteristics: Macro-cell, co-site

  - The location of intermediate node is indoor

And if we compared the current scenario options in RAN4, the mapping relationship between the current scope and the WID is as following:

Table 1. The mapping between WID [2] requested co-ex scenarios and options in WF [1]

| WID [2] | Mapping options in RAN4 [1] |
| --- | --- |
| Deployment scenario 1 with Topology 1 - Basestation and co-existence characteristics: Micro-cell, co-site Note 1: Deployment scenario 1: Device indoors, basestation indoors [3] Note 2: Topology 1: BS ↔ Ambient IoT device [3] | D1T1 Option 2-1 and Option 2-2 |
| Deployment scenario 2 with Topology 2 - Basestation and co-existence characteristics: Macro-cell, co-site - The location of intermediate node is indoor Note 1: Deployment scenario 2: Device indoors, basestation outdoors [3] Note 2: Topology 2: BS ↔ intermediate node ↔ Ambient IoT device [3] | D2T2 Option 1-1 and Option 1-2 |

Observation 4: The D1T1 Option 2-1 and Option 2-2 are the listed “deployment scenario 1 with topology 1” tasked for co-existence in the WID.

Observation 5: The D2T2 Option 1-1 and Option 1-2 are the listed “deployment scenario 2 with topology 2” tasked for co-existence in the WID.

From the Observation 1~5 above, we have the following proposals for the scenarios:

Proposal 1: It is proposed to prioritize D1T1 Option 2-1 and Option 2-2 for the “Deployment scenario 1 with Topology 1” in WID.

Proposal 2: It is proposed to prioritize D2T2 Option 1-1 and Option 1-2 for the “Deployment scenario 2 with Topology 2” in WID.

2.2 Spectrum usage

For the spectrum usage of R2D, considering the regulatory concerns of using gNB to transmit R2D signal in uplink spectrum, and the concerns of using UE to transmit R2D signal in downlink spectrum, we agree with to start with the using DL for R2D in D1T1, and using UL for R2D in D2T2. And FFS other options.

Together with the assumption of spectrum usage, by using DL for gNB transmitting R2D and using UL for UE transmitting R2D, the follow-up question is how to distribute power and time resources for this gNB or UE to transmit its normal NR UL/DL and R2D signals.

To better study the scenario with current spectrum usage starting point, RAN4 should ask RAN1 to better clarify their agreements or working assumptions on how gNB and/or UE split their power and time resources between NR transmission and R2D transmission.

Proposal 3: RAN4 to ask or wait for RAN1 to conclude how would the gNB (for Topology 1) and UE (for Topology 2) would split its transmitting power between the legacy NR and R2D links, and whether they will transmit the NR and R2D simultaneously or in TDM manner.

In the meantime, the RAN4 could continue studies with some working assumptions. And given the IoT would be a plus to the NR gNB normal feature, it is reasonable to assume the R2D would not harm the NR gNB DL coverage and latency, hence the gNB should be able to transmit NR DL as normal while it should be upgraded with either hardware or software to transmit R2D in addition. Thus, we believe it’s reasonable to assume the gNB in topology 1

For topology 1:

Observation 6: For topology 1, to not introduce harmful impact to the existing NR DL coverage or latency, it is reasonable to assume the gnB in topology 1 can transmit the NR DL and R2D at the same time.

Observation 7: For topology 1, for simplification purpose, the co-existence study can start with the working assumption that the gNB can have upgraded capability, by either hardware or software, to fulfill the power required for both NR DL and R2D links.

Proposal 4: While waiting for conclusion from RAN1, for topology 1, RAN4 could continue the co-ex work with working assumptions that the gNB in topology 1 can boost its power or using separate hardware to fulfill the power demand for both NR and R2D links, and transmitting the R2D and NR DL simultaneously;

For topology 2:

Observation 8: For topology 2, for simplification purpose, the co-existence study can start with the working assumption that the UE to transmit NR UL and R2D in TDM manner.

Proposal 5: While waiting for conclusion from RAN1, for topology 2, RAN4 could continue the co-ex work with working assumptions that UE in topology 2 would transmit the R2D and NR UL in TDM manner.

3	Conclusion

For the Ambient IoT deployment scenarios and spectrum usage, we propose the following.

Observation 1: The WID clearly instructed the deployment scenario 1 with topology 1 and deployment scenario 2 with topology 2 will focus on co-site scenarios for co-existence.

Observation 2: The TR 38.848[3], which the objectives in WID[2] was referring, described the co-existence for A-IoT considered either ‘co-site’ or ‘new site’, where

- ‘co-site’ for ‘Deployed on the same sites as an existing 3GPP deployment corresponding to the basestation type.’

- ‘new-site’ for ‘Deployed on new sites without an assumption of an existing 3GPP deployment.’

Observation 3: The existing 3GPP deployment should be the baseline assumption given the nature of RAN4 co-ex study. With this, the TR and WID clearly mapped to the ‘co-site’ scenario for NR and A-IoT, and should be focused in RAN4 co-ex studies.

Observation 4: The D1T1 Option 2-1 and Option 2-2 are the listed “deployment scenario 1 with topology 1” tasked for co-existence in the WID.

Observation 5: The D2T2 Option 1-1 and Option 1-2 are the listed “deployment scenario 2 with topology 2” tasked for co-existence in the WID.

Proposal 1: It is proposed to prioritize D1T1 Option 2-1 and Option 2-2 for the “Deployment scenario 1 with Topology 1” in WID.

Proposal 2: It is proposed to prioritize D2T2 Option 1-1 and Option 1-2 for the “Deployment scenario 2 with Topology 2” in WID.

Proposal 3: RAN4 to ask or wait for RAN1 to conclude how would the gNB (for Topology 1) and UE (for Topology 2) would split its transmitting power between the legacy NR and R2D links, and whether they will transmit the NR and R2D simultaneously or in TDM manner.

Observation 6: For topology 1, to not introduce harmful impact to the existing NR DL coverage or latency, it is reasonable to assume the gnB in topology 1 can transmit the NR DL and R2D at the same time.

Observation 7: For topology 1, for simplification purpose, the co-existence study can start with the working assumption that the gNB can have upgraded capability, by either hardware or software, to fulfill the power required for both NR DL and R2D links.

Proposal 4: While waiting for conclusion from RAN1, for topology 1, RAN4 could continue the co-ex work with working assumptions that the gNB in topology 1 can boost its power or using separate hardware to fulfill the power demand for both NR and R2D links, and transmitting the R2D and NR DL simultaneously;

Observation 8: For topology 2, for simplification purpose, the co-existence study can start with the working assumption that the UE to transmit NR UL and R2D in TDM manner.

Proposal 5: While waiting for conclusion from RAN1, for topology 2, RAN4 could continue the co-ex work with working assumptions that UE in topology 2 would transmit the R2D and NR UL in TDM manner.

5	Reference

[1] R4-2410567, WF on co-existence evaluation for ambient IoT, CMCC

[2] RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA

[3] TR 38.848 V18.0.0