3GPP TSG-RAN4 Meeting # 112	R4-2412563

# Maastricht, Netherlands, 19th – 23rd August, 2024

Title:	Discussion on coex evaluation assumptions

Source:	Samsung

Agenda item:	8.20.2.2

Document for:	Approval

1	Introduction

In the RAN4#111 meeting, the WF R4-2410567 [1] had achieved progress for the co-existence evaluation on Ambient-IoT, but also left open issues for further discussions. In this paper, the remaining issues will be discussed and the solutions are proposed to further progress the work.

2	Discussions

NR Indoor BS parameters:

From the current WF[1], the indoor gNB parameters are still missing, which is required for certain scenarios, hence based on the previous agreement and offline dicsussion between two meetings, we propose to consider to use the agreed AioT indoor reader parameters as a starting point, as it was also derived from indoor BS.

Table 1. Parameters for NR indoor

| Indoor BS parameters | Values for calibration purposes |
| --- | --- |
| Indoor BS total Tx power | 33dBm |
| Indoor BS receiver Noise Figure（dB） | 10 |
| Indoor BS antenna gain (dBi) | 6 dBi |

And the antenna pattern for indoor gNB:

Proposal 1: It is proposed to re-use the previously discussed indoor reader antenna pattern for indoor gNB as a starting point for co-ex.

R2D SINR consideration:

Given the previous WF considered to use the SINR as the starting point for R2D link evaluation, and no selectivity was adopted if filter is not applied. In this paper, we provided the analysis for R2D with the LPF applied.

Figure 1 The received signal with both NR and D2R

It can be seen in the figure above, that the NR signal transmitted in the adjacent RBs will also be received as interference by the tag and it will impact the result after the envelope detector.

By applying a LPF with 2RB bandwidth, and by assuming the same received power from NR and R2D by the tag, the analysis shows the LPF can suppress at least 4.3 dB to the NR signals The analysis is shown in the figure below.

Figure 2. LPF can provide at least 4.3 dB adjacent RB selectivity

Given the gurad RB is still under discussion, and the actual LPF bandwidth can be even narrower, which would further suppress the interference from received NR interferences. We propose to consider the following for R2D SINR calculation

Table 2. R2D SINR considerations with LPF

| SINR | R2D with LPF | R2D without LPF |
| --- | --- | --- |
| Interference from NR | Frequency selectivity: > 4.3 dB | No frequency selectivity |
| Noise bandwidth | 180kHz | 10MHz |

Proposal 2-1: It is proposed to consider the device tag with LPF as one option for R2D evaluation.

Proposal 2-2: By adopting LPF for R2D, the frequency selectivity on interference and noise can be applied to the SINR. Where the attenuation for received NR interference by the tag can be at least 4.3 dB and the noise can be reduced for SINR calculation of R2D.

D2R SINR consideration:

It is slightly different for D2R in simulation, when multiple devices were reflecting/backscattering the CW2D power to generate D2R, the AioT reader/UE will receive several D2R signals, and the connecting/attaching scheme should be discussed and agreed in RAN4, as this will impact which D2R links would be evaluated in the monte-carlo methods.

The consideration should include two aspects:

- Which D2R link will be evaluated among all D2R links recevied by the reader?

- What SINR criteria can be taken as a succesful D2R link established, if there’re several D2R links received at similar power level.

In our understanding, when the reader in simulation received several D2R links, the tag with best D2R links will be evaluated in this snapshot. But if there’s another D2R link received at similar level, then we will have to ask RAN1 to provide the SINR criteria to determine at which SINR level the D2R link can be decoded correctly.

Proposal 3: In co-existence evaluation, to pick the D2R with highest received SNR in each snapshot for evaluation.

Proposal 4: The SINR threshold for correct decoding D2R by receiver, especially when multiple D2R received with similar power level, need input from RAN1.

D1T1 Option 2 Layouts

The D1T1 Option 1 assumptions were provided in the WF[1] in last meeting, and to extend the current assumptions to study the Option 2-1 and Option 2-2 scenarios, the following modifications are proposed:

| Parameter | Assumptions for D1T1 Option 2-1, 2-2 |
| --- | --- |
| Scenario | InF-DH |
| Hall size | 120x60 m |
| Room height | 10 m |
| Sectorization | None |
| Pathloss model | NLOS and LOS in TR38.901 |
| BS deployment / Intermediate UE dropping | For D1T1-A2: 18 BSs on a square lattice with spacing D, located D/2 from the walls. - L=120m x W=60m; D=20m - BS height = 8 m For D1T1-A1: - FFS on layout: one node as transmission and CW, the neighbour node as reception |
| Device distribution | Device Height= 1.5 m AIoT devices drop uniformly distributed over the horizontal area Number of A-IoTs = Total area × activated density (1.5 A-IOT devices/m²) 1 active AIOT device under one reader at one drop MCL between device and reader is 45dB for calibration. |
| NR BS deployment | Option 1-1 and 1-2: Outdoor Hexagonal grid, 19 macro sites, 3 sectors per site with wrap around, 1 AIOT indoor scenario per sector the minimum 2D distance between macro BS and indoor factory centre is set as 100m. Option 2-1 and 2-2: Indoor co-site Co-site with A-IoT indoor reader deployment. |
| NR BS Inter-site distance | Option 1-1 and 1-2: 750 Option 2-1 and 2-2: 20m |
| Minimum NR BS – NR UE distance (2D) | Option 1-1 and 1-2: 35 m Option 2-1 and 2-2: 0m |
| NR UE (D1T1) dropping | For scenario option 1-1, uniformly distributed outdoor. For scenario option 1-2, uniformly distributed, 80% indoor, 20% outdoor For scenario option 2-1 and 2-2: uniformly distributed indoor. UE number: - DL active UE: 1 UE per cell - UL active UE: 3UE per cell |
| O2I penetration loss | High penetration loss as in TR 38.901 for calibration |

Proposal 5: For D1T1 Option 2-1 and 2-2, we propose to amend information in above table for layouts, which include the following assumptions:

1) NR BS indoor gNB deployed co-site with A-IoT indoor reader;

2) ISD as 20m;

3) Min BS-UE distance: 0m;

4) NR indoor UE uniformly distributed.

3	Conclusion

For the Ambient IoT co-existence study assumption, we propose the following.

Proposal 1: It is proposed to re-use the previously discussed indoor reader antenna pattern for indoor gNB as a starting point for co-ex.

Proposal 2-1: It is proposed to consider the device tag with LPF as one option for R2D evaluation.

Proposal 2-2: By adopting LPF for R2D, the frequency selectivity on interference and noise can be applied to the SINR. Where the attenuation for received NR interference by the tag can be at least 4.3 dB and the noise can be reduced for SINR calculation of R2D.

Proposal 3: In co-existence evaluation, to pick the D2R with highest received SNR in each snapshot for evaluation.

Proposal 4: The SINR threshold for correct decoding D2R by receiver, especially when multiple D2R received with similar power level, need input from RAN1.

Proposal 5: For D1T1 Option 2-1 and 2-2, we propose to amend information in above table for layouts, which include the following assumptions:

1) NR BS indoor gNB deployed co-site with A-IoT indoor reader;

2) ISD as 20m;

3) Min BS-UE distance: 0m;

4) NR indoor UE uniformly distributed.

5	Reference

[1] R4-2410567, WF on co-existence evaluation for ambient IoT, CMCC

[2] RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA

[3] TR 38.848 V18.0.0