3GPP TSG-RAN WG4 Meeting #112                                   R4-2412565

Maastricht, Netherlands, August 19 – August 23, 2024

Source: 	MediaTek Inc.

Title: 	Discussion on IMT UE parameters for 7125 to 8400 MHz

Agenda Item:	8.2.3

Document for:	Approval

- Introduction

In RAN#103, a new SI [1] was agreed upon to study the IMT parameters for the frequency ranges of 4400 to 4800 MHz, 7125 to 8400 MHz, and 14800 to 15350 MHz. RAN4 has identified August 2024 (RAN4#112) as the estimated completion date for the frequency range of 7125 to 8400 MHz. In RAN4 #111, the WF for IMT parameters for 7125-8400 MHz was approved [2] and some UE parameters have not yet been determined. In this contribution, we will provide our views on the remaining IMT parameters for 7125-8400 MHz range for UE.

- Discussion

At the last RAN4 #111 meeting, some IMT parameters for the 7125-8400 MHz range were discussed, and a number of them were agreed upon based on n104. However, several UE parameters still require further discussion.

ACLR

The following was agreed at RAN4#110bis:

  - Option 1:  26dB, 27dB (study) for PC3

  - Option 2:  30dB (n104) for PC3, 31dB (n104) for PC2

According to the simulation results in clause 4.3 of TR 38.921, it has been agreed to specify a 26 dB ACLR for the 6425-7125 MHz range. Our study indicates that ACLR would be the limiting factor for the PA's maximum output power. Additionally, the propagation loss for the 7125-8400 MHz range is greater than that of FR1, resulting in poorer uplink coverage. In our view, relaxing the UE ACLR could help increase the PA's maximum output power and improve uplink coverage for the UE. Therefore, we prefer option 1, which specifies a 26 dB ACLR for PC3.

Proposal 1: RAN4 to consider Option 1 for the UE ACLR, which is 26dB for PC3.

Maximum output power

In the last RAN4 meeting, we have tentative agreement as follow:

Tentative agreement: - Capture 23dBm as typical maximum output power in the LS to ITU. - Other output powers are not precluded and refer to TR - Capture 20dBm, 23dBm, 26dBm, and higher power classes in TR

The 7125-8400 MHz frequency range exhibits approximately 6 dB more in total propagation loss compared to the 4 GHz frequency band, leading to limited uplink coverage. Uplink coverage is crucial for the rollout of wireless communication systems, and operators have shown significant interest in a higher maximum output power. In our view, FR3 BS are expected to be co-located with FR1 BS, and FR3 BS are anticipated to achieve similar coverage as FR1 BS. To maintain the same uplink coverage for the 8 GHz band as for the 4 GHz band, a maximum output power of 29 dBm is recommended. Considering the varied services for different areas, we are in agreement with the tentative decision made in the last meeting, which captures 23 dBm as the typical maximum output power in the LS to the ITU. It was also agreed that other output powers are not precluded and should refer to the TR, capturing 20 dBm, 23 dBm, 26 dBm, and higher power classes in the TR.

Proposal 2: RAN4 to agree that capture 23 dBm as the typical maximum output power in the LS to the ITU. It was also agreed that other output powers are not precluded and should refer to the TR, capturing 20 dBm, 23 dBm, 26 dBm, and higher power classes in the TR.

Noise figure

The following was agreed at RAN4#110bis:

  - Option 1: Follow n104 noise figure (12dB)

  - Option 2: Be consistent with information sent previously IMT-2020 28GHz, e.g. 10dB

  - Option 3: Be consistent with Previous LS to ITU-R on 6, 10GHz, NF was 9-13dB

According to TR 38.921, a noise figure of 9 dB is used as the baseline assumption, and 13 dB is used as the optional assumption for the co-existence evaluation for the 6425-7125 MHz range. A noise figure within the [9, 13] dB interval was finally agreed upon for reporting to the ITU WP5D sharing studies. Additionally, a noise figure of 12 dB was assumed during the band n104 discussions. Considering the higher implementation loss for the 7125-8400 MHz range, we prefer option 3, which uses 13 dB as the noise figure for this range.

Proposal 3: RAN4 to consider 13dB as the UE NF value.

Sensitivity

In our view, the sensitivity requirements for band n104 can serve as a baseline with a 1 dB noise figure relaxation. Therefore, the reference sensitivity is -84.8 dBm for 100 MHz with a 60 kHz SCS and 4Rx. However, according to TR38.921, sensitivity is not considered a critical parameter for sharing and compatibility studies and it was agreed not to mention any specific value for this parameter. Therefore, the sensitivity for UE can be considered to be specified.

Proposal 4: RAN4 to consider the sensitivity for the UE as “to be specified”.

ACS

According to the simulation results in clause 4.3 of TR 38.921, it has been agreed that the ACS will be 32 dBc for the 6425–7125 MHz range and 31 dBc for the 10 - 10.5 GHz range. Therefore, we prefer an ACS of 31 dBc, which is applied to the 7125-8400 MHz range for PC3.

Proposal 5: RAN4 to consider 31dBc as the UE ACS value.

- 3	Conclusion

In this contribution, we have presented our perspectives on the IMT parameters for 7125-8400MHz for UEs, and have provided the following proposals:

Proposal 1: RAN4 to consider Option 1 for the UE ACLR, which is 26dB for PC3.

Proposal 2: RAN4 to agree that capture 23 dBm as the typical maximum output power in the LS to the ITU. It was also agreed that other output powers are not precluded and should refer to the TR, capturing 20 dBm, 23 dBm, 26 dBm, and higher power classes in the TR.

Proposal 3: RAN4 to consider 13dB as the UE NF value.

Proposal 4: RAN4 to consider the sensitivity for the UE as “to be specified”.

Proposal 5: RAN4 to consider 31dBc as the UE ACS value.

- 4	References

- RP-240787, “Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz”, Ericsson.

- R4-2410741, “WF on IMT parameters study”, Ericsson.