3GPP TSG-RAN WG4 Meeting #112                                        R4-2412568

Maastricht, Netherlands, August 19 - 23, 2024

Source: 	Huawei, HiSilicon

Title: 	On power domain enhancements for single carrier

Agenda Item:	8.1.1.2.1

Document for:	Approval

Introduction

During RAN4#111, power domain enhancement for single carrier has been discussed. Following conclusions can be found in [1].

- Consideration on relaxation of requirements for identified scenarios Agreement in main session: - No relaxation of level of spurious emission requirements - FFS on whether to change the spurious emission boundary - Mechanisms for enabling MPR reduction and/or power boosting Way forward: - Study the approach to convert full RB allocation in UE CBW to “inner RB allocation” with an extended UE BW, e.g. each side of the UE BW is equal to ½ of the UE BW, inside a larger BS CBW as starting point with consideration of the following aspects - The edge of the extended UE BW, i.e. the UE CBW plus the shifted frequency symmetrically at each side of the UE CBW, should be inside BS CBW or at least aligned with the BS CBW edge - ① FFS feasibility of case where extended UE CBW edge exceeds the BS CBW edge, i.e. gap between edges of UE CBW and BS CBW < 1/2 UE CBW - ② FFS whether IBE is used between edges of UE CBW and BS CBW or between edges of UE CBW and extended UE CBW - ③ FFS ACLR and SEM are applicable from the edge of extended UE CBW or edge of BS CBW, i.e. the start point of ΔfOOB - ④ FFS integral region and boundary of OOBE is based on extended UE CBW or UE CBW - Other mechanisms are not precluded - Simulation assumption for power boosting and/or MPR reduction Way forward: Agree with the preliminary simulation assumptions for following evaluation. Assumptions are subject to revisions with further study. - PA model calibration - DFT-s-OFDM QPSK 20MHz - 100RB0 - 4dB post PA loss - 1dB MPR - Carrier Leakage: 28dBc - IQ Image: 28dBc - CIM3: 60dBc - EVM: 17.5% - ACLR: 30dB for PC3, 31dB for PC2

In this contribution, we would like to share our views regarding this topic.

Discussion

With the discussion outcome in previous meeting, resolution respective to following scenarios still needs to be discussed separately.

- Scenario A: No adjacent in-band/out-of-band co-existence issue (single operator)

For this scenario, one example is the NR band n34/n41 in China which is allocated to CMCC only. There may not be any concern about adjacent channel emission because in addition to existing implementation aiming for interference mitigation including BS filter design, guard band and spatial isolation, coordination based on measurement for single operator is always feasible. Another example is the band n28 deployment in China which is co-construction and sharing by CMCC and CBN. Identical situation happens for band n78 deployment in China, where China Unicom and China Telecom pool their resources for construction and sharing. Similarly, coordination can be expected at least for in-band.

Observation 1: There could be no concern on adjacent in-band/out-of-band co-existence for following examples, considering existing implementation aiming for interference mitigation including BS filter design, guard band, spatial isolation and coordination based on measurements:

- NR band n28 in China is co-construction and sharing by CMCC and CBN

- NR band n78 in China is co-construction and sharing by China Unicom and China Telecom

- NR band n34 and n41 in China are exclusively allocated to CMCC

Conspicuously, this is somehow regional since it is highly related to spectrum allocation in specific region along with network deployment.

Observation 2: No adjacent in-band/out-of-band co-existence issue scenario is somehow regional and frequency band dependent since it is highly related to spectrum allocation along with network deployment.

Naturally, out-of-band requirements relaxation controlled by network for such scenario can be considered as per region per band. Fortunately, existing A-MPR mechanism can be a reference here. Additionally, we can consider multi-level relaxation to keep the flexibility for the network and also to better accommodate different requirements in different region per band.

Example on the supported relaxation level per band

|  | Case #1 | Case #2 | Case #3 |
| --- | --- | --- | --- |
| NR band A | XdB relaxation on ACLR and SEM mask | YdB relaxation on ACLR and SEM mask | Waived on ACLR and SEM mask |
| NR band B | XdB relaxation on ACLR and SEM mask | YdB relaxation on ACLR and SEM mask |  |
| NR band C | XdB relaxation on ACLR and SEM mask |  |  |

Our initial measurements on PC2 50MHz UE CBW with QPSK CP-OFDM waveform with different relaxation level are provided as below. Obviously, UE can deliver different MPR gain under different level relaxation.

(a) 3dB                        (b) 5dB                        (c) waived

Figure 1. Measurement results to show MPR reduction gain when both ACLR and SEM mask are relaxed by 3dB or 5dB or waived

Observation 3: Measurements show that different MPR gain can be achieved under different level of ACLR/SEM mask relaxation:

- The outer RB allocations can achieve 0.5dB MPR gain under 3dB ACLR and SEM mask relaxation

- The outer RB allocations can achieve 0.8dB MPR gain under 5dB ACLR and SEM mask relaxation

- The outer RB allocations can achieve 1dB MPR gain when ACLR and SEM requirements are waived

With the feasibility as proved by the measurements, we have the following proposal.

Proposal 1: For the scenario when there is no adjacent in-band/out-of-band co-existence issue (single operator), introduce network controlled relaxation on ACLR and SEM mask.

- For ACLR, the relaxation means e.g. PC3 requirement 30dB can be relaxed to a smaller value.

- For SEM mask, the relaxation means an upward shift on the whole mask.

- No change on the reference channel bandwidth or the start of ΔfOOB

Proposal 2: Check if followings can be adopted for the network controlled relaxation towards no adjacent in-band/out-of-band co-existence issue (single operator) scenario.

- Three level of relaxation, e.g. 3dB, 5dB and waived

- Per band and per region (can be based on operator request)

- Scenario B: Narrower UE channel BW within wider BS bandwidth (with adjacent operators)

About the BS bandwidth, the term “BS BW for one operator holding spectrum” was mentioned in the WF [2].

- 1.4 Evaluation of relaxed requirements Way forward: - [No relaxation of ACLR/SEM/SE outside of the BS CBW for one operator holding spectrum for scenario 2, i.e. Narrower UE channel BW within wider BS bandwidth] - FFS whether outer, edge or inner RB allocation is prioritized for further evaluation - FFS impact on MPR by relaxing ACLR w/ or w/o relaxing SEM/Spurious Emissions - FFS whether IBE is considered instead of the relaxed ACLR/SEM/SE for the region between UE CBW and BS CBW.

Actually, according to the WID, the solution for this scenario should cover both normal UE and Redcap UE. For normal UE, the indicated capability on maximum UE CBW is likely to be near 100MHz for widely deployed NR bands. Thus if we confine the “wider BS bandwidth” to single carrier, the candidate solutions would not be applicable. As for Redcap UE, normally the 20MHz UE CBW would be configured at the edge of BS CBW, since any requirement change out of BS CBW may not be considered for this scenario due to the existence of adjacent operators, the aforementioned limitation will also make this feature useless for Redcap UE.

Figure 2. Motivation on not confine the BS bandwidth to single carrier

In conclusion, we think it should be clarified that the “wider BS bandwidth/BS CBW for one operator holding spectrum” actually means BS RF bandwidth that covers single carrier, multi-carriers and multi-RATs scenarios, which is officially defined in the TS 37.104 as following figure.

Proposal 3: Clarify that the BS channel bandwidth means BS RF bandwidth that covers single carrier, multi-carriers and multi-RATs scenarios.

As captured in the approved WF, the framework of the mechanism dedicated for narrower UE channel BW within wider BS bandwidth has been set up. Remaining issues on the detail can be formulated as follows.

- With the expectation that the outer region MPR can be reduced to e.g. inner region MPR:

#1: Since the bottleneck for UE to apply lower MPR in outer region would be meeting out of band emission limit, how to shift the start point of ΔfOOB away from the edge of configured UE CBW?

#2: Within the frequency resources between the shifted start point of ΔfOOB and the edge of configured UE CBW, what requirement should be filled in?

Before having discussion on the shifted ΔfOOB, it is worth mentioning that the so-called inner RB region is to reflect the relative length and position for a block of allocated RBs comparing with the maximum available RBs according to the definition in TS 38.101-1.

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1. RBStart,Low = max(1, floor(LCRB/2)) where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x. RBStart,High = NRB – RBStart,Low – LCRB The RB allocation is an Inner RB allocation if the following conditions are met RBStart,Low ≤ RBStart ≤ RBStart,High, and LCRB ≤ ceil(NRB/2) where ceil(x) is the smallest integer greater than or equal to x.

Following example can elucidate how a RB allocation is categorized under different total frequency span.

Figure 3. Example on an outer RB allocation that can be categorized as inner within wider CBW

Easily examined by the previous highlighted definition, the allocated 60MHz RBs would be categorized as outer within 100MHz UE CBW but it would be inner instead if we consider it from e.g. the whole BS CBW perspective.

Observation 4: For a certain RB allocation (LCRB, mainly) belongs to e.g. outer RB allocation, it can be categorized as inner RB allocation if it refers to wider channel bandwidth e.g. BS RF bandwidth instead of UE channel bandwidth.

- #1: How to shift ΔfOOB

Regarding how to shift start point of ΔfOOB, there are two options can be considered:

- Allow UE to indicate the minimum shifted frequency that is symmetrically distributed at both edge of UE CBW

  - To acquire the aggregated maximum number of RBs for inner RB determination, include the indicated shifted frequency together with the given UE CBW

- Use the edge of BS RF channel bandwidth as the start point of ΔfOOB

  - To acquire the aggregated maximum number of RBs for inner RB determination, replace the given UE CBW with BS CBW

For the first one, the network can flexibly configure the UE CBW with additional information on where reduced MPR can be expected on whole (original) outer region in case the shifted frequency can be reserved at each side adjacent to the UE CBW. But when there is no enough frequency resource reserved, the MPR reduction cannot be expected from UE. Following figure can show the scheduling restriction.

Figure 4. Illustration on the advantages and limitations for solution a)

For the second one, obviously there would not be any scheduling restriction comparing to the first solution. Specifically, for the case when there is no enough frequency resource to be reserved, still partial of the original outer region can be converted to inner region and enjoy less MPR. Following figure can be used to describe the situation.

Figure 5. Illustration on one case that can deliver MPR gain when solution b) is applied

Observation 5: Expected effect and necessary info exchange between BS and UE for the below two options:

|  | Expected effect | Necessary info exchange between BS and UE |
| --- | --- | --- |
| Solution a Allow UE to indicate the minimum shifted frequency that is symmetrically distributed at both edge of UE CBW - To acquire the aggregated maximum number of RBs for inner RB determination, include the indicated shifted frequency together with the given UE CBW | When there has enough frequency resource to be reserved for the indicated shifted frequency at each side of UE CBW, MPR reduction can be expected on the whole outer region. Otherwise it cannot be expected | UE minimum shifted frequency for a specific band |
| Solution b Use the edge of BS RF channel bandwidth as the start point of ΔfOOB - To acquire the aggregated maximum number of RBs for inner RB determination, replace the given UE CBW with BS CBW | Within the whole BS CBW, MPR reduction can be expected when the allocated RB belongs to inner region referring to BS CBW | The length of BS CBW and the location of UE CBW within BS CBW should be informed to UE |

Hence we have the following proposal regarding the necessary info exchange between BS and UE.

Proposal 4: If RAN4 picks the following solution for narrower UE channel BW within wider BS bandwidth (with adjacent operators) scenario, UE indication on the minimum shifted frequency of ΔfOOB per concerning band should be allowed.

- Symmetrically shift the start of ΔfOOB away from the UE CBW of the frequency equals to the minimum shifted frequency the UE indicated for the concerning band

  - To acquire the aggregated maximum number of RBs for inner RB determination, include the indicated shifted frequency together with the given UE CBW

Proposal 5: If RAN4 picks the following solution for narrower UE channel BW within wider BS bandwidth (with adjacent operators) scenario, the length of BS CBW and the location of UE CBW within BS CBW should be informed to UE.

- Use the edge of BS CBW band the start point of ΔfOOB

  - The BS channel bandwidth means BS RF bandwidth that covers single carrier, multi-carriers and multi-RATs scenarios

  - To acquire the aggregated maximum number of RBs for inner RB determination, include the indicated shifted frequency together with the given UE CBW

- #2: What requirement should be filled in

By any one of the solutions as being analyzed in the previous section, the frequency adjacent to the UE CBW would bear the brunt considering non-linear distortion can be increased since the UE is allowed to apply smaller MPR. Thus proper requirements can be introduced for weighing the MPR gain against the potentially increased noise level within the gap between the shifted ΔfOOB and the edge of UE CBW.

| Parameter description | Unit | Limit (NOTE 1) | Applicable Frequencies |
| --- | --- | --- | --- |
| General | dB |  | Any non-allocated (NOTE 2) |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of PRB - 30 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in NOTE 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. For pi/2 BPSK with Spectrum Shaping, the limit is expressed as a ratio of measured power in one non-allocated RB to the measured power in the allocated RB with highest PSD. …... NOTE 9: is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. ∆RB = 1 or ∆RB = -1 for the first adjacent RB outside of the allocated bandwidth. NOTE 10: is an average of the transmitted power over 10 sub-frames normalized by the number of allocated RBs, measured in dBm. |  |  |  |

It should be noted that since the carrier leakage and the image frequency are within UE CBW, the applied IBE requirement for the gap is actually the “general” component with following adaptation:

- Use the aggregated NRB which corresponds to the frequency span between two shifted ΔfOOB

- The index  of the first adjacent RB outside of the UE CBW should be 1 or -1

- No RF requirements apply to the legacy guard band

Proposal 6: To align the simulation assumption, it is the “general” part of IBE requirements that applies to the gap between the shifted ΔfOOB and the edge of UE CBW with following adaptation:

- Use the aggregated NRB which corresponds to the frequency span between two shifted ΔfOOB

- The index  of the first adjacent RB outside of the UE CBW should be 1 or -1

Proposal 7: Regardless what mechanism will be evaluated and introduced for Rel-19 single carrier MPR reduction, no RF requirements should be applied to the guard band defined in TS 38.101-1 clause 5.3.3.

With the shifted ΔfOOB, we simulate the required MPR for CP-OFDM QPSK waveform with PC2 at the original outer region, the UE CBW is 20MHz with 30kHz SCS configuration. For this case, the inner MPR can be met when there are 1/2 UE CBW frequency resources reserved at each side of CBW symmetrically.

Figure 6. Outer region MPR with 1/2 UE CBW shifted ΔfOOB

Observation 6: When the ΔfOOB is symmetrically shifted 1/2 UE CBW away from the edge of UE CBW and general part of IBE requirements applied to the gap created by the shift, inner region MPR can be applied to outer RB region.

Proposal 8: For the shifted start point of ΔfOOB, 1/2 UE CBW can be the default value for UE indication.

Further, since whether to allow MPR reduction must be controlled by the network, then from UE side more possibilities can be considered to serve the purpose of enriching applicable scenarios. For instance, the EVM parameter in the general part of IBE requirements can be fixed to the one for Pi/2-BPSK as a relaxation in exchange for potentially shorter shifted frequency. With that, large UE CBW can increase the chance to enjoy the MPR reduction gain.

Proposal 9: Consider to introduce shorter shifted frequency e.g. 1/3, 1/4 UE CBW in conjunction with the requirement that will be introduced to the shifted frequency.

- For instance, the modified general part of IBE requirements which is further relaxed by replacing the EVM corresponding to the scheduling modulation order with the one for BPSK

Conclusion

In this contribution we discussed on the Rel-19 MPR reduction for single CC. We have the following observations and proposals:

Observation 1: There could be no concern on adjacent in-band/out-of-band co-existence for following examples, considering existing implementation aiming for interference mitigation including BS filter design, guard band, spatial isolation and coordination based on measurements:

- NR band n28 in China is co-construction and sharing by CMCC and CBN

- NR band n78 in China is co-construction and sharing by China Unicom and China Telecom

- NR band n34 and n41 in China are exclusively allocated to CMCC

Observation 2: No adjacent in-band/out-of-band co-existence issue scenario is somehow regional and frequency band dependent since it is highly related to spectrum allocation along with network deployment.

Observation 3: Measurements show that different MPR gain can be achieved under different level of ACLR/SEM mask relaxation:

- The outer RB allocations can achieve 0.5dB MPR gain under 3dB ACLR and SEM mask relaxation

- The outer RB allocations can achieve 0.8dB MPR gain under 5dB ACLR and SEM mask relaxation

- The outer RB allocations can achieve 1dB MPR gain when ACLR and SEM requirements are waived

Observation 4: For a certain RB allocation (LCRB, mainly) belongs to e.g. outer RB allocation, it can be categorized as inner RB allocation if it refers to wider channel bandwidth e.g. BS RF bandwidth instead of UE channel bandwidth.

Observation 5: Expected effect and necessary info exchange between BS and UE for the below two options:

|  | Expected effect | Necessary info exchange between BS and UE |
| --- | --- | --- |
| Solution a Allow UE to indicate the minimum shifted frequency that is symmetrically distributed at both edge of UE CBW - To acquire the aggregated maximum number of RBs for inner RB determination, include the indicated shifted frequency together with the given UE CBW | When there has enough frequency resource to be reserved for the indicated shifted frequency at each side of UE CBW, MPR reduction can be expected on the whole outer region. Otherwise it cannot be expected | UE minimum shifted frequency for a specific band |
| Solution b Use the edge of BS RF channel bandwidth as the start point of ΔfOOB - To acquire the aggregated maximum number of RBs for inner RB determination, replace the given UE CBW with BS CBW | Within the whole BS CBW, MPR reduction can be expected when the allocated RB belongs to inner region referring to BS CBW | The length of BS CBW and the location of UE CBW within BS CBW should be informed to UE |

Observation 6: When the ΔfOOB is symmetrically shifted 1/2 UE CBW away from the edge of UE CBW and general part of IBE requirements applied to the gap created by the shift, inner region MPR can be applied to outer RB region.

Proposal 1: For the scenario when there is no adjacent in-band/out-of-band co-existence issue (single operator), introduce network controlled relaxation on ACLR and SEM mask.

- For ACLR, the relaxation means e.g. PC3 requirement 30dB can be relaxed to a smaller value.

- For SEM mask, the relaxation means an upward shift on the whole mask.

- No change on the reference channel bandwidth or the start of ΔfOOB

Proposal 2: Check if followings can be adopted for the network controlled relaxation towards no adjacent in-band/out-of-band co-existence issue (single operator) scenario.

- Three level of relaxation, e.g. 3dB, 5dB and waived

- Per band and per region (can be based on operator request)

Proposal 3: Clarify that the BS channel bandwidth means BS RF bandwidth that covers single carrier, multi-carriers and multi-RATs scenarios.

Proposal 4: If RAN4 picks the following solution for narrower UE channel BW within wider BS bandwidth (with adjacent operators) scenario, UE indication on the minimum shifted frequency of ΔfOOB per concerning band should be allowed.

- Symmetrically shift the start of ΔfOOB away from the UE CBW of the frequency equals to the minimum shifted frequency the UE indicated for the concerning band

  - To acquire the aggregated maximum number of RBs for inner RB determination, include the indicated shifted frequency together with the given UE CBW

Proposal 5: If RAN4 picks the following solution for narrower UE channel BW within wider BS bandwidth (with adjacent operators) scenario, the length of BS CBW and the location of UE CBW within BS CBW should be informed to UE.

- Use the edge of BS CBW band the start point of ΔfOOB

  - The BS channel bandwidth means BS RF bandwidth that covers single carrier, multi-carriers and multi-RATs scenarios

  - To acquire the aggregated maximum number of RBs for inner RB determination, include the indicated shifted frequency together with the given UE CBW

Proposal 6: To align the simulation assumption, it is the “general” part of IBE requirements that applies to the gap between the shifted ΔfOOB and the edge of UE CBW with following adaptation:

- Use the aggregated NRB which corresponds to the frequency span between two shifted ΔfOOB

- The index  of the first adjacent RB outside of the UE CBW should be 1 or -1

Proposal 7: Regardless what mechanism will be evaluated and introduced for Rel-19 single carrier MPR reduction, no RF requirements should be applied to the guard band defined in TS 38.101-1 clause 5.3.3.

Proposal 8: For the shifted start point of ΔfOOB, 1/2 UE CBW can be the default value for UE indication.

Proposal 9: Consider to introduce shorter shifted frequency e.g. 1/3, 1/4 UE CBW in conjunction with the requirement that will be introduced to the shifted frequency.

- For instance, the modified general part of IBE requirements which is further relaxed by replacing the EVM corresponding to the scheduling modulation order with the one for BPSK

- References

- R4-2410749, “WF on power domain enhancements”, Huawei, HiSilicon, 3GPP TSG-RAN4 Meeting #111, Fukuoka, Japan, May 20-24, 2024.

- R4-2406584, “WF on power domain enhancements”, Huawei, HiSilicon, 3GPP TSG-RAN4 Meeting #110bis, Changsha, China, April 14-19, 2024.