3GPP TSG-RAN WG4 Meeting #112                                        R4-2412569

Maastricht, Netherlands, August 19 - 23, 2024

Source: 	Huawei, HiSilicon

Title: 	On MPR applicability for FR1 intra-band UL CA

Agenda Item:	8.1.1.2.2

Document for:	Approval

Introduction

During RAN4#111, MPR applicability for FR1 intra-band contiguous/non-contiguous UL CA has been discussed. Following conclusions can be found in [1].

- MPR applicability for FR1 intra-band contiguous UL CA Way forward: - For PC3 and PC2 contiguous UL CA use the corresponding single CC MPR when only 1 CC is activated. - FFS: MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C. - FFS: MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated. - FFS: MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent. - FFS whether spurious emissions/ACLR/SEM are kept for aggregated CBW for CA - MPR applicability for FR1 intra-band UL non-contiguous CA Way forward: Enhancements not aligned with the WID will not be considered. - Further clarify the scope for NC CA in next meeting based on the WID objectives.

In this contribution, we would like to share our views regarding this topic.

Discussion

- FR1 intra-band contiguous UL CA

Reviewing the discussion in last meeting, agreement on applying single CC MPR to intra-band contiguous UL CA when SCell is deactivated was almost achieved given that UE vendors can be confident in meeting NW scheduling preference when UL traffic is low. Actually, MAC CE based mechanism could provide sufficient time for UE to adjust RF resources assignment then utilizing smaller MPR can be possible.

Proposal 1: RAN4 specifies the following MPR applicability:

- For PC3, the single CC MPR requirements (Table 6.2.2-1) apply to FR1 intra-band contiguous UL CA with only one UL CC activated.

- For PC2, the single CC MPR requirements apply to FR1 intra-band contiguous UL CA with only one UL CC activated:

  - If TxD is indicated for this intra-band contiguous UL CA, single CC with TxD MPR (Table 6.2D.2-1) should apply.

  - If TxD is not indicated for this intra-band contiguous UL CA, single CC without TxD MPR (Table 6.2.2-2) should apply.

Another issue is whether to keep the spurious emissions/ACLR/SEM for the aggregated UE CBW. Our preference is to keep such flexibility for the UE since smaller MPR can already be delivered through a faster scheduling procedure i.e. MAC CE, instead of pushing it more.

Proposal 2: For Rel-19 MPR applicability for FR1 intra-band contiguous UL CA, the spurious emissions/ACLR/SEM are kept for aggregated CBW.

- FR1 intra-band non-contiguous UL CA

As we have identified in our previous paper [2], currently single CC MPR requirements are explicitly referenced when there is no scheduling after configuration and activation on any one of the two component carriers.

For UE indicating dualPA-Architecture supported If OR (LCRB1 = 0, LCRB2 = 0) MPR defined in Table 6.2.2-1 and Table 6.2.2-2 for PC3 and PC2 UE respectively Else If AND( FIM3,low_block,low > SEM-13,low , FIM3,high_block,high < SEM-13,high ) MPR defined in Clause 6.2A.2.2.2.1 and Clause 6.2A.2.2.2.2 for PC3 and PC2 UE respectively. Else MPR defined in Clause 6.2A.2.2.1.1 and Clause 6.2A.2.2.1.2 for PC3 and PC2 UE respectively. For UE without indicating dualPA-Architecture supported If OR( LCRB1 = 0, LCRB2 = 0 ) For PC3 UE, MPR defined in Table 6.2.2-1, except for B < 9 MHz where 5.5 dB MPR is used; For PC2 UE without indicating TxD, MPR defined in Table 6.2.2-2 is used, except for B < 11.52 MHz where 6.5 dB MPR is used; For PC2 UE indicating TxD, MPR defined in Table 6.2D.2-1 is used, except for B < 11.52 MHz where the maximum value between 6.5 dB and MPR defined in Table 6.2D.2-1 is used. Else If AND( FIM3,low_block,low > SEM-13,low , FIM3,high_block,high < SEM-13,high ) MPR defined in Clause 6.2A.2.2.2.3 and Clause 6.2A.2.2.2.4 for PC3 and PC2 UE respectively. Else MPR defined in Clause 6.2A.2.2.1.3 and Clause 6.2A.2.2.1.4 for PC3 and PC2 UE respectively.

Regarding those “exception” cases for small RB allocation for the UE without indicating dualPA-architecture, we notice the requirements were formulated based on the measurements from [3], in which the worst case of fixed LO has been considered.

At this stage, we think it is better to keep the existing flexibility for UE implementation unless why the above “worst case” should be precluded can be justified.

Proposal 3: In Rel-19, the applicable MPR for FR1 intra-band non-contiguous UL CA doesn’t need further enhancement.

Conclusion

In this contribution we discussed on the remaining issue for Rel-19 MPR applicability for FR1 intra-band UL CA. We have the following proposals:

Proposal 1: RAN4 specifies the following MPR applicability:

- For PC3, the single CC MPR requirements (Table 6.2.2-1) apply to FR1 intra-band contiguous UL CA with only one UL CC activated.

- For PC2, the single CC MPR requirements apply to FR1 intra-band contiguous UL CA with only one UL CC activated:

  - If TxD is indicated for this intra-band contiguous UL CA, single CC with TxD MPR (Table 6.2D.2-1) should apply.

  - If TxD is not indicated for this intra-band contiguous UL CA, single CC without TxD MPR (Table 6.2.2-2) should apply.

Proposal 2: For Rel-19 MPR applicability for FR1 intra-band contiguous UL CA, the spurious emissions/ACLR/SEM are kept for aggregated CBW.

Proposal 3: In Rel-19, the applicable MPR for FR1 intra-band non-contiguous UL CA doesn’t need further enhancement.

- References

- R4-2410749, “WF on power domain enhancements”, Huawei, HiSilicon, 3GPP TSG-RAN4 Meeting #111, Fukuoka, Japan, May 20-24, 2024.

- R4-2405614, “On power domain enhancement for NR single carrier and NR intra-band UL CA for PC2 and PC3”, Ericsson, 3GPP TSG-RAN4 Meeting #110bis, Changsha, China, April 15-19, 2024.