3GPP TSG-RAN WG4 Meeting #112                                        R4-2412570

Maastricht, Netherlands, August 19 - 23, 2024

Source: 	Huawei, HiSilicon

Title: 	On MPR applicability for FR2 CA

Agenda Item:	8.1.1.2.3

Document for:	Approval

Introduction

During RAN4#111, MPR applicability for FR2 single carrier UL and intra-band UL CA with DL intra-band CA has been discussed. Following conclusion can be found in [1].

- Applicable MPR for FR2 single carrier UL with DL intra band CA Way forward: - For the case of single carrier UL with DL intra band CA, the MPR requirements of single carrier case in clause 6.2.2 of TS 38.101-2 applies with UE indication of new capability for MPR improvement. - Applicable MPR for FR2 UL CA with DL intra band CA Way forward: - MPR based on UL BWchannel_CA applies instead that based on cumulative aggregated channel BW (CABW) with UE indication of new capability for MPR improvement - If only 1 UL CC is activated, the MPR requirements of single carrier could be reused - FFS whether new MPR requirement could be defined for CABW < 400 MHz (e.g., 200 MHz).

In this contribution, we would like to share our views regarding this topic.

Discussion

With the current agreement on the substitution of UL BWchannel_CA for cumulative aggregated channel BW (CABW), we understand that following MPR applicability can be achieved for intra-band DL CA subject to new UE capability indication.

For the case of DL intra band CA with UL CA, the reference MPR (originally being Table 6.2A.2.4-1) is changed as:

|  |  | UL BWchannel_CA |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | ≤ 400 MHz | > 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz | > 1400 MHz and ≤ 2400 MHz |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.01 | ≤ 7.7 | ≤ 8.2 | ≤ 8.7 |
|  | QPSK | ≤ 5.01 | ≤ 7.7 | ≤ 8.2 | ≤ 9.7 |
|  | 16 QAM | ≤ 6.5 | ≤ 8.7 | ≤ 9.3 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |
| CP-OFDM | QPSK | ≤ 5.0 | ≤ 7.5 | ≤ 8.0 | ≤ 9.7 |
|  | 16 QAM | ≤ 6.5 | ≤ 8.7 | ≤ 9.2 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |
| NOTE 1: (Void). |  |  |  |  |  |

Note that there is an existing relaxation targeting CABW < 400 MHz but also subject to specific modulation order and waveform. Our understanding is that it should be ignored under this circumstance since the determination of MPR is always UL BWchannel_CA.

In case of a contiguous RB, DFT-s-BPSK or DFT-s-QPSK UL allocation in a single CC of a CA configuration with contiguous CCs, and whose cumulative aggregated BW 400 MHz, MPRC_CA shall be derived instead as MAX(MPR1, MPR2), where: MPR1 shall be determined from Table 6.2.2.3-1 if CABW 200 MHz, from Table 6.2.2.3-2 if CABW > 200 MHz. MPR2 shall be determined from Table 6.2.2.3-1 if UL BWchannel_CA 200 MHz, from Table 6.2.2.3-2 if UL BWchannel_CA > 200 MHz.

For the case of DL intra band CA with single UL CC (activated or configured), the reference MPR (originally being Table 6.2A.2.4-1) is changed as the MPR requirement for single carrier.

Observation 1: Subject to UE capability indication on the support of Rel-19 MPR applicability for FR2 CA, following specification clarifications are required.

- For the case of DL intra band CA with UL CA, the reference MPR (originally being Table 6.2A.2.4-1) is changed as:

|  |  | UL BWchannel_CA |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | ≤ 400 MHz | > 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz | > 1400 MHz and ≤ 2400 MHz |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.01 | ≤ 7.7 | ≤ 8.2 | ≤ 8.7 |
|  | QPSK | ≤ 5.01 | ≤ 7.7 | ≤ 8.2 | ≤ 9.7 |
|  | 16 QAM | ≤ 6.5 | ≤ 8.7 | ≤ 9.3 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |
| CP-OFDM | QPSK | ≤ 5.0 | ≤ 7.5 | ≤ 8.0 | ≤ 9.7 |
|  | 16 QAM | ≤ 6.5 | ≤ 8.7 | ≤ 9.2 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |
| NOTE 1: (Void). |  |  |  |  |  |

  - The existing relaxation targeting CABW < 400 MHz and also subject to specific modulation order and waveform should be ignored

In case of a contiguous RB, DFT-s-BPSK or DFT-s-QPSK UL allocation in a single CC of a CA configuration with contiguous CCs, and whose cumulative aggregated BW 400 MHz, MPRC_CA shall be derived instead as MAX(MPR1, MPR2), where: MPR1 shall be determined from Table 6.2.2.3-1 if CABW 200 MHz, from Table 6.2.2.3-2 if CABW > 200 MHz. MPR2 shall be determined from Table 6.2.2.3-1 if UL BWchannel_CA 200 MHz, from Table 6.2.2.3-2 if UL BWchannel_CA > 200 MHz.

- For the case of DL intra band CA with single UL CC (activated or configured), the reference MPR (originally being Table 6.2A.2.4-1) is changed as the MPR requirement for single carrier.

Regarding the FFS point, we think the necessity of it, possibly achieved by further splitting the ≤ 400 MHz column and adding correspondence MPR values, is not so strong, considering that the MPR for ≤ 400 MHz CABW is not unacceptably large.

Observation 2: The necessity of FFS on whether new MPR requirement could be defined for CABW < 400 MHz may need further evaluation.

Conclusion

In this contribution we discussed on the remaining issue for Rel-19 MPR applicability for FR2 CA. We have the following observation:

Observation 1: Subject to UE capability indication on the support of Rel-19 MPR applicability for FR2 CA, following specification clarifications are required.

- For the case of DL intra band CA with UL CA, the reference MPR (originally being Table 6.2A.2.4-1) is changed as:

|  |  | UL BWchannel_CA |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | ≤ 400 MHz | > 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz | > 1400 MHz and ≤ 2400 MHz |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.01 | ≤ 7.7 | ≤ 8.2 | ≤ 8.7 |
|  | QPSK | ≤ 5.01 | ≤ 7.7 | ≤ 8.2 | ≤ 9.7 |
|  | 16 QAM | ≤ 6.5 | ≤ 8.7 | ≤ 9.3 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |
| CP-OFDM | QPSK | ≤ 5.0 | ≤ 7.5 | ≤ 8.0 | ≤ 9.7 |
|  | 16 QAM | ≤ 6.5 | ≤ 8.7 | ≤ 9.2 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |
| NOTE 1: (Void). |  |  |  |  |  |

  - The existing relaxation targeting CABW < 400 MHz and also subject to specific modulation order and waveform should be ignored

In case of a contiguous RB, DFT-s-BPSK or DFT-s-QPSK UL allocation in a single CC of a CA configuration with contiguous CCs, and whose cumulative aggregated BW 400 MHz, MPRC_CA shall be derived instead as MAX(MPR1, MPR2), where: MPR1 shall be determined from Table 6.2.2.3-1 if CABW 200 MHz, from Table 6.2.2.3-2 if CABW > 200 MHz. MPR2 shall be determined from Table 6.2.2.3-1 if UL BWchannel_CA 200 MHz, from Table 6.2.2.3-2 if UL BWchannel_CA > 200 MHz.

- For the case of DL intra band CA with single UL CC (activated or configured), the reference MPR (originally being Table 6.2A.2.4-1) is changed as the MPR requirement for single carrier.

Observation 2: The necessity of FFS on whether new MPR requirement could be defined for CABW < 400 MHz may need further evaluation.

- References

- R4-2410749, “WF on power domain enhancements”, Huawei, HiSilicon, 3GPP TSG-RAN4 Meeting #111, Fukuoka, Japan, May 20-24, 2024.