3GPP TSG-RAN WG4 Meeting #112                                        R4-2412571

Maastricht, Netherlands, August 19 - 23, 2024

Source: 	Huawei, HiSilicon

Title: 	On 6Rx reference sensitivity requirements for FR1 UE

Agenda Item:	8.1.1.3.1

Document for:	Approval

Introduction

During RAN4#111, FR1 6Rx UE RF requirements have been discussed. Following conclusion regarding REFSENS can be found in [1].

- Topic 1: REFSENS (delta RIB,6R) - Sub-topic 1-1: General considerations for specifying ΔRIB,6R value Issue 1-1-1: Whether band n104 should be included in the high band (n77, n78 and n79) category for 6Rx case Agreement: For ΔRIB,6R, include n77, n78, n79 and [n104] into the same group. Issue 1-1-2: Whether to use same ΔRIB,6R value for handheld UE and FWA Way Forward: Further discuss the following options. - Option 1: RAN4 needs to determine whether to define different ΔRIB,6R value for handheld UE and FWA separately - Option 2: Same value for handheld UE and FWA - Option 3: Different value for handheld UE and FWA Issue 1-1-3: Release independence of ΔRIB,6R value Way Forward: Follow RAN4 Chair guidance and defer the release independence discussions until the requirements are stable. - Sub-topic 1-2: ΔRIB,6R values for handheld UE and FWA Issue 1-2-1: Proposed ΔRIB,6R values for handheld UE and FWA Way Forward: Further discuss the following options for the ΔRIB,6R values. Ultimately, the decision to specify separate or same requirements for handheld UE and FWA will be dependent on outcome of Issue 1-1-2. - Option 1: Consider range of ΔRIB,6R in brackets for each case below for further discussion. Operating bands; ΔRIB,6R for FWA; ΔRIB,6R for Handheld. n77, n78, n79, [n104]; [-3.0 ~ -3.3] dB; [-3.0 ~ -3.2] dB. n41; [-3.0 ~ -4.0] dB; [-3.0 ~ -4.0] dB. - Option 2: Consider average value of ΔRIB,6R from company proposals for further discussion. - Option 3: Other options are not precluded.

In this contribution, we would like to share our views regarding this topic.

Discussion

From the collected inputs as in above table, it can be observed that the demand for differentiated 6Rx delta Rib requirements is from UE form factor and operating band. These two factors can be further illustrated since they are inextricably linked with RF resource:

- Due to form factor limitation, physical antenna layout in order to accommodate increased Rx number could lead to compromised performance on:

  - RF isolation, and

  - Antenna efficiency

- Availability on RF resource could be different per operating band

Observation: The demand for differentiated 6Rx delta Rib requirements is from UE form factor and operating band which could be further illustrated from RF perspective as follows:

- Due to form factor limitation, physical antenna layout in order to accommodate increased Rx number could lead to compromised performance on:

  - RF isolation, and

  - Antenna efficiency

- Availability on RF resource could be different per operating band

According to WID objective [2], it is required to derive a set of 6Rx RF requirement applied for both handheld UE and FWA UE. Thus we think the aforementioned demand can be met from operation band point of view solely, unless other unexpected concern could show up. Therefore, the 4/8Rx delta Rib defined for FWA UE can be considered as reference, since the RAN4 consideration on margin and diversity gain are reflected to some extent.

Table 7.3.2-2: Four antenna port reference sensitivity allowance ΔRIB,4R Operating band; ΔRIB,4R (dB). n5, n8, n13, n28, n71, n85, n105; -2.71. n5, n8, n28, n71, n20, n26; -2.42. n1, n2, n3, n25, n30, n40, n7, n34, n38, n39, n41, n66, n70; -2.7. n48, n77, n78, n79, n104; -2.2. NOTE 1: When 4 Rx operation is supported by FWA form factor NOTE 2: When 4Rx operation is supported by handheld UE.. Table 7.3.2-2a: Eight antenna port reference sensitivity allowance ΔRIB,8R Operating band; ΔRIB,8R (dB). n7; -4.5. n41; -4.3. n77, n78, n79; -4.0. NOTE 1: 8 Rx operation is targeted for FWA/CPE/Vehicle/Industrial devices form factor..

In conclusion, we have the following proposal without simply considering any scaling on increment of Rx number or even more aggressive one for RAN4 to further check.

Proposal: Check whether below ΔRIB,6R for the example bands n41, n77/n78, n79 and n104 can be applied for both handheld UE and FWA UE:

- -3dB for band n77/78, n79 and n104

- -3.3dB for band n41

Conclusion

In this contribution we discussed on the REFSENS for FR1 6Rx UE. We have the following observation and proposal:

Observation: The demand for differentiated 6Rx delta Rib requirements is from UE form factor and operating band which could be further illustrated from RF perspective as follows:

- Due to form factor limitation, physical antenna layout in order to accommodate increased Rx number could lead to compromised performance on:

  - RF isolation, and

  - Antenna efficiency

- Availability on RF resource could be different per operating band

Proposal: Check whether below ΔRIB,6R for the example bands n41, n77/n78, n79 and n104 can be applied for both handheld UE and FWA UE:

- -3dB for band n77/78, n79 and n104

- -3.3dB for band n41

- References

- R4-2410751, “WF on requirements for 6Rx”, AT&T, 3GPP TSG-RAN4 Meeting #111, Fukuoka, Japan, May 20-24, 2024.

- RP-241656, “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, Huawei, HiSilicon, 3GPP TSG-RAN Meeting #104, Shanghai, China, June 17-20, 2024.