3GPP TSG-RAN WG4 Meeting #112                                                                                       R4-2412572

Maastricht, Netherlands, August 19 - 23, 2024

Title: 	On MIMO layer evaluation for 6Rx UE

Source: 	Huawei, HiSilicon

Agenda item:	8.1.1.3.2

Document for:	Discussion

Background

A WF on requirements for 6Rx was approved in RAN4 #111 meeting, one part is MIMO layers evaluation. This contribution provides our related views.

- Discussions

Evaluation methodology

To fairly compare the 6 Layers performance with 4 Layers, considering the test metric is throughput, it is straightforward to use link adaption for simulations, while fixed FRC is normally for testing purpose but not so fit for reflecting the real-time link level performance. So we propose to adopt link adaption for evaluation. Additionally, base station side scheduling algorithm such as OLLA is not considered here, so that MCS will be determined purely based on CSI feedback. However, considering the challenge of calibration, we are open to discuss in case any reasonable solution based on fix FRC can be proposed.

Proposal 1: RAN4 to use link adaption without OLLA for evaluation as baseline.

For fixed FRC, RAN4 could simulate serval MCSs for 4 Layers and 6 Layers respectively and compare the maximum Throughput at given SNR, noted that MCS corresponding to the max Throughput of 4 Layers and that of 6 Layers could be different.

Observation 1: For fixed FRC, RAN4 could simulate serval MCSs for 4 Layers and 6 Layers respectively and compare the maximum Throughput at given SNR. The MCS corresponding to the max Throughput of 4 Layers and that of 6 Layers could be different.

It is worth noticing that for DMRS type1, double DMRS symbol length is needed to support more than 4 Layers, which means DMRS overhead of 6 Layers is more than that of 4 Layers. Therefore, the throughput gain of 6 Layers is limited, assume there are 2 symbols allocate for PDCCH, 2 DMRS symbols for PDSCH with 4 layers and 4 DMRS symbols for PDSCH with 6 layers, for the same MCS, the peak Throughput ratio of 6 Layers to 4 Layers is 3/2*(14-2-4)/ (14-2-2)=1.2.

Observation 2: For DMRS type 1, 6 Layers will lead to more DMRS overhead compared to 4 Layers, which would limit the throughput gain.

BS EVM

Some companies propose to tighten the BS EVM when discussing the feasibility of 6 layers MIMO for 6Rx UEs:

Issue 3-1-1: Tightening BS EVM requirement Way Forward: RAN4 to further discuss if the BS EVM requirement needs to be tightened when discussing the feasibility of 6-Layer MIMO for 6Rx UEs.

EVM requirements defined in RF spec were assumed for demodulation requirements derivation and most importantly they are highly related to existing implementation. Furthermore, EVM was not tightened during 8Rx evaluation. So we don’t think it is necessary to tighten the BS EVM as specified in Table 6.5.2.2-1 in TS 38.101-4:

Table 6.5.2.2-1 in TS 38.101-4: EVM requirements for BS type 1-C and BS type 1-H carrier

| Modulation scheme for PDSCH | Required EVM |
| --- | --- |
| QPSK | 17.5 % |
| 16QAM | 12.5 % |
| 64QAM | 8 % |
| 256QAM | 3.5 % |
| 1024QAM | 2.5 %1 2.8 %2 |
| Note1: This requirement is applicable for frequencies equal to or below 4.2 GHz. Note 2: This requirement is applicable for frequencies above 4.2 GHz. |  |

Proposal 2: RAN4 shall not tighten the BS EVM requirements for 6 MIMO layers feasibility study.

Antenna correlation matrix

Antenna correlation highly depends on antenna design. So we think companies can start with providing reasonable antenna correlation based on e.g. measurement and run the simulation accordingly.

For antenna correlation at BS side, we propose to choose low correlation (α=0) to guarantee the performance of fixed FRC (Random PMI).

Proposal 3: Companies can start with providing reasonable antenna correlation based on e.g. measurement and run the simulation accordingly.

Proposal 4: RAN4 to use low correlation (α=0) at BS side.

MIMO isolation

Besides antenna correlation, antenna leakage is another factor affecting the supported MIMO layers. Therefore, MIMO isolation should also be considered when evaluating the MIMO layers, where 10dB is typical value for 4 antennas [TS 38.101-1], companies are encouraged to reuse it.

Proposal 5: RAN4 to consider 10dB MIMO isolation when evaluating the 6 MIMO layers feasibility.

Other parameters

For other parameters, we propose the listed ones in Table 1 in appendix as starting point.

- Initial simulation results

In Clause 3.1 and Clause 3.2 we provide our simulation results. In our simulations, the proposed antenna correlation is shown as follows:

RUE=1.000.100.101.000.300.10     0.300.150.100.141.000.12    0.130.120.300.200.200.120.150.140.130.300.120.20     0.121.000.200.110.120.30    0.110.301.000.100.101.00

For the time being, MIMO isolation are not modelled in our simulation.

Link adaption

Two cases are simulated:

4 Layers: UE is allowed to be scheduled up to 4 Layers.

6 Layers: UE is allowed to be scheduled up to 6 Layers.

Figure 1: simulation results for link adaption.

We can observe that the two curves for 4 Layers and 6 Layers are overlapped. To better explain it, we extract the rank distribution as in Table 2, it can be observed that Rank 5 and Rank 6 are not scheduled even the UE is allowed to.

Table 2: Rank distribution

|  | Rank distribution |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SNR | Rank1 | Rank2 | Rank3 | Rank4 | Rank5 | Rank6 |
| 20 | 0% | 0% | 14% | 86% | 0% | 0% |
| 22 | 0% | 0% | 8% | 100% | 0% | 0% |
| 24 | 0% | 0% | 0% | 100% | 0% | 0% |
| 26 | 0% | 0% | 0% | 100% | 0% | 0% |
| 28 | 0% | 0% | 0% | 100% | 0% | 0% |
| 30 | 0% | 0% | 0% | 100% | 0% | 0% |
| 32 | 0% | 0% | 0% | 100% | 0% | 0% |
| 34 | 0% | 0% | 0% | 100% | 0% | 0% |
| 36 | 0% | 0% | 0% | 100% | 0% | 0% |
| 38 | 0% | 0% | 0% | 100% | 0% | 0% |
| 40 | 0% | 0% | 0% | 100% | 0% | 0% |

Fix FRC

We provide our simulation results in Figure 2 based on following assumptions in Table 3.

Table 3: Simulation assumptions for fix FRC

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Bandwidth |  | MHz | 10 |
| SCS |  | kHz | 15 |
| Antenna configuration |  |  | 8T6R |
| Propagation channel |  |  | TDLA30-5 |
| Rank |  |  | Rank4: Rank6: |
| TxEVM |  |  | 16QAM:12.5% 64QAM: 8% 256QAM: 3.5% |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Test 1-2: Type 1 with start RB = 23, LRBs = 6 Other tests: Type 0 |
|  | RBG size |  | Test 1-2: N/A Other tests: Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |

Figure 2: simulation results for fixed FRC

We can observe that at each SNR point from 26dB to 40dB, the maximum throughput corresponding to all simulated MCSs for 4 layers is higher than 6 Layers, which means 4 Layers performance is better than 6 Layers. Based on the simulation results in clause 3.1 and 3.2, it can be concluded that under such conditions with proposed antenna correlation, 4 layers will outperform 6 layers.

Observation 3: Under the conditions with proposed antenna correlation, 4 layers outperforms 6 layers.

- Conclusion

- This contribution provides our views on MIMO layers evaluation for 6Rx, the observations and proposals are:

Observation 1: For fixed FRC, RAN4 could simulate serval MCSs for 4 Layers and 6 Layers respectively and compare the maximum Throughput at given SNR. The MCS corresponding to the max Throughput of 4 Layers and that of 6 Layers could be different.

Observation 2: For DMRS type 1, 6 Layers will lead to more DMRS overhead compared to 4 Layers, which would limit the throughput gain.

Observation 3: Under the conditions with proposed antenna correlation, 4 layers outperforms 6 layers.

Proposal 1: RAN4 to use link adaption without OLLA for evaluation as baseline.

Proposal 2: RAN4 shall not tighten the BS EVM requirements for 6 MIMO layers feasibility study.

Proposal 3: Companies can start with providing reasonable antenna correlation based on e.g. measurement and run the simulation accordingly.

Proposal 4: RAN4 to use low correlation (α=0) at BS side.

Proposal 5: RAN4 to consider 10dB MIMO isolation when evaluating the 6 MIMO layers feasibility.

- Reference

[1]    TS 38.101-4. v18.6.0

- Appendix

Table 1: Simulation assumptions

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 10 |
| Subcarrier spacing |  |  | kHz | 15 |
| Duplex Mode |  |  |  | FDD |
| Propagation channel |  |  |  | TDLA30-5 |
| Antenna configuration |  |  |  | 8T4R |
| TxEVM |  |  |  | 16QAM:12.5% 64QAM: 8% 256QAM: 3.5% |
| PDSCH configuration | Mapping type |  |  | Type A |
|  | k0 |  |  | 0 |
|  | Starting symbol (S) |  |  | 2 |
|  | Length (L) |  |  | 12 |
|  | PDSCH aggregation factor |  |  | 1 |
|  | PRB bundling type |  |  | Static |
|  | PRB bundling size |  |  | 2 |
|  | Resource allocation type |  |  | Test 1-2: Type 1 with start RB = 23, LRBs = 6 Other tests: Type 0 |
|  | RBG size |  |  | Test 1-2: N/A Other tests: Config2 |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |
|  | Antenna ports |  |  | Rank4: Port{1000,1001,1002,1003} Rank6: Port{1000,1001,1002,1003,1004,1005} |
| DMRS configuration | DMRS Type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | Rank4: 1 Rank6: 2 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 4 |
|  | CDM Type |  |  | FD-CDM2 |
|  | Density (ρ) |  |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,4 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 |
|  | CSI-RS periodicity and offset |  | slot | 5/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 8 |
|  | CDM Type |  |  | FD-CDM2 |
|  | Density (ρ) |  |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 6,(0) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 5/1 |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |
|  | CSI-IM RE pattern |  |  | 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 5/1 |
| ReportConfigType |  |  |  | Periodic |
| CQI-table |  |  |  | Table 2 |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |
| cqi-FormatIndicator |  |  |  | Wideband |
| pmi-FormatIndicator |  |  |  | Wideband |
| Sub-band Size |  |  | RB | 8 |
| csi-ReportingBand |  |  |  | 1111111 |
| CSI-Report periodicity and offset |  |  | slot | 5/0 |
| aperiodicTriggeringOffset |  |  |  | Not configured |
| Codebook configuration (Note 1) |  | Codebook Type |  | typeI-SinglePanel |
|  |  | Codebook Mode |  | 1 |
|  |  | (CodebookConfig-N1, CodebookConfig-N2) |  | (4,1) |
|  |  | (CodebookConfig-O1, CodebookConfig-O2) |  | (4,1) |
|  |  | two-one-TypeI-SinglePanel-Restriction |  | N/A |
|  |  | RI Restriction |  | 00111111 for UE supporting up to 6 layers 00001111 for UE supporting up to 4 layers |
| Physical channel for CSI report |  |  |  | PUCCH |
| CQI/RI/PMI delay |  |  | ms | 19 |
| Maximum number of HARQ transmission |  |  |  | 1 |
| Number of HARQ Processes |  |  |  | 4 |
| Note: No PDSCH is transmitted in CSI slot |  |  |  |  |