3GPP TSG-RAN WG4 Meeting #112                                        R4-2412573

Maastricht, Netherlands, August 19 - 23, 2024

Source: 	Huawei, HiSilicon

Title: 	On 6Rx SRS antenna switching requirements

Agenda Item:	8.1.1.3.3

Document for:	Approval

Introduction

During RAN4#111, FR1 6Rx UE RF requirements have been discussed. Following conclusion regarding ΔTRxSRS can be found in [1].

- Topic 2: SRS antenna switching and ΔTRxSRS - Sub-topic 2-1: General considerations for SRS antenna switching and ΔTRxSRS Issue 2-1-1: SRS antenna switching configurations Way forward: RAN4 to focus on discussing ∆TRxSRS for 1tr6 and t2r6 until RAN1 concludes work on 3t6r and 4t6r. Pending RAN1 feedback, RAN4 can consider any necessary updates to the work scope in a Revised WID for the SRS antenna configurations subject to RAN Plenary approval. Issue 2-1-2: Whether to use different ∆TRxSRS based on operating frequency Way forward: RAN4 defines different ∆TRxSRS values based on operating frequency. Further discuss whether to introduce an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104. - Sub-topic 2-2: ΔTRxSRS values Issue 2-2-1: Proposed ∆TRxSRS values Way forward: Further discuss the following options for the ∆TRxSRS requirements. - Option 1: Consider average value of ∆TRxSRS for further discussion. Operating bands; ΔTRxSRS requirement for t1r6; ΔTRxSRS requirement for t2r6; ΔTRxSRS requirement for t1r6-t2r6. Band n41, n77, n78; 4.0 dB; 3.4 dB; 4.4 dB. Band n79, [n104]; 5.3 dB; 4.5 dB; 5.7 dB. - Option 2: Consider range of ∆TRxSRS in brackets for each case below for further discussion. Operating bands; ΔTRxSRS requirement for t1r6; ΔTRxSRS requirement for t2r6; ΔTRxSRS requirement for t1r6-t2r6. Band n41, n77, n78; [2.8 ~ 5.0] dB; [2.3 ~ 4.0] dB; [3.3 ~ 5.5] dB. Band n79, [n104]; [3.6 ~ 6.0] dB; [3.0 ~ 5.5] dB; [4.2 ~ 6.5] dB. Note: Maximum level for t1r6 corrected for OPPO proposals per OPPO. - Option 3: Other options are not precluded.

In this contribution, we would like to share our views regarding this topic.

Discussion

While RAN1 will consider introducing 3T6R SRS for antenna switching in Rel-19 as requested in the latest WID [2], we provide our preference on the prioritized 1T6R and 2T6R ΔTRxSRS requirements.

As illustrated in our contribution about REFSENS requirements [3], following aspects should be carefully considered:

- Due to form factor limitation, physical antenna layout in order to accommodate increased Rx number could lead to compromised performance on:

  - RF isolation, and

  - Antenna efficiency

- Availability on RF resource could be different per operating band

And it was shared by some companies that the aforementioned implementation difficulties could be more serious for higher frequency like n104.

Observation 1: Additional breakpoint for bands whose FUL_high is higher than the FUL_low of band n104 may be needed.

As captured in the WID [2], the 6Rx SRS insertion loss requirement is targeting at both handheld and FWA UE. As for FWA, there seems to be less controversial given that the 8Rx requirements have already been introduced. When it comes to handheld UE, it could be more complicated because RF component layout and/or trace loss could vary depending on phablet or foldable phone. For instance, as one implementation challenge for foldable phone, the insertion loss would increase for a diversity branch lays through the hinge connecting different screens.

Observation 2: The derivation on 6Rx ∆TRxSRS requirement needs to consider real implementation in both phablet and foldable phone.

- For a foldable phone, increasing insertion loss can be expected for a diversity branch that lays through the hinge connecting different screens.

However, our intuitive idea is to reuse what have been specified for 8 Rx FWA UE, this can be further checked along with the previous two options for RAN4.

Proposal: Check if following framework regarding 6Rx ∆TRxSRS requirements can be adopted.

- When antenna switching SRS capability is indicated as 't1r6' or ‘t2r6’:

  - When the device is capable of power class 3 or power class 5 or power class 1.5 in the band, or when the device is capable of power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating txDiversity-r16.

    - For the bands whose FUL_high is lower than the FUL_low of n79, the value of ∆TRxSRS is [4] dB.

    - For the bands whose FUL_high is lower than the FUL_low of n104, the value of ∆TRxSRS is [5.5] dB.

    - For the bands whose FUL_high is higher than the FUL_low of n104, the value of ∆TRxSRS is [TBD] dB.

- When antenna switching SRS capability is indicated as 't1r6-t2r6':

  - When the device is capable of power class 3 or power class 5 or power class 1.5 in the band, or when the device is capable of power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating txDiversity-r16.

    - For the bands whose FUL_high is lower than the FUL_low of n79, the value of ∆TRxSRS is [4.5] dB.

    - For the bands whose FUL_high is lower than the FUL_low of n104, the value of ∆TRxSRS is [6] dB.

    - For the bands whose FUL_high is higher than the FUL_low of n104, the value of ∆TRxSRS is [TBD] dB.

Conclusion

In this contribution we discussed on the 6Rx SRS antenna switching requirements. We have the following observations and proposal:

Observation 1: Additional breakpoint for bands whose FUL_high is higher than the FUL_low of band n104 may be needed.

Observation 2: The derivation on 6Rx ∆TRxSRS requirement needs to consider real implementation in both phablet and foldable phone.

- For a foldable phone, increasing insertion loss can be expected for a diversity branch that lays through the hinge connecting different screens.

Proposal: Check if following framework regarding 6Rx ∆TRxSRS requirements can be adopted.

- When antenna switching SRS capability is indicated as 't1r6' or ‘t2r6’:

  - When the device is capable of power class 3 or power class 5 or power class 1.5 in the band, or when the device is capable of power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating txDiversity-r16.

    - For the bands whose FUL_high is lower than the FUL_low of n79, the value of ∆TRxSRS is [4] dB.

    - For the bands whose FUL_high is lower than the FUL_low of n104, the value of ∆TRxSRS is [5.5] dB.

    - For the bands whose FUL_high is higher than the FUL_low of n104, the value of ∆TRxSRS is [TBD] dB.

- When antenna switching SRS capability is indicated as 't1r6-t2r6':

  - When the device is capable of power class 3 or power class 5 or power class 1.5 in the band, or when the device is capable of power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating txDiversity-r16.

    - For the bands whose FUL_high is lower than the FUL_low of n79, the value of ∆TRxSRS is [4.5] dB.

    - For the bands whose FUL_high is lower than the FUL_low of n104, the value of ∆TRxSRS is [6] dB.

    - For the bands whose FUL_high is higher than the FUL_low of n104, the value of ∆TRxSRS is [TBD] dB.

- References

- R4-2410751, “WF on requirements for 6Rx”, AT&T, 3GPP TSG-RAN4 Meeting #111, Fukuoka, Japan, May 20-24, 2024.

- RP-241656, “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, Huawei, HiSilicon, 3GPP TSG-RAN Meeting #104, Shanghai, China, June 17-20, 2024.

- R4-2412571, “On 6Rx reference sensitivity requirements for FR1 UE”, Huawei, HiSilicon, 3GPP TSG-RAN4 Meeting #112, Maastricht, Netherlands, August 19-23, 2024.