3GPP TSG-RAN WG4 Meeting #112                                        R4-2412575

Maastricht, Netherlands, August 19 - 23, 2024

Source: 	Huawei, HiSilicon

Title: 	On RF requirements for Rel-19 NR-MIMO

Agenda Item:	8.18.2

Document for:	Approval

Introduction

For this RAN1 leading WI NR_MIMO_Ph5, this is the first RAN4 meeting according to the TU allocation [1]. RAN4 is tasked to identify potential specification impact.

The detailed objectives are as follows: RAN1: - Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting - Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH - Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. - Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. Acronyms: - sTRP: single TRP (transmit-receive point) - mTRP: multiple TRP (transmit-receive point) - CJT: coherent joint transmission - DU: distributed unit - CRI: CSI-RS resource indicator RAN2: Specify higher layer support of the enhancements listed above. RAN4: Specify necessary core requirements for the enhancements listed above.

In this contribution, we would like to share our views regarding this topic.

Discussion

From our understanding, the 3Tx would have RAN4 RF requirements impact. Following RAN1 progress on 3Tx can be found in [2, 3 and 4].

RAN#116 Agreement For non-coherent uplink precoding by a 3TX UE, at least following precoders are supported for single-layer transmission. Agreement For non-coherent uplink precoding by a 3TX UE, at least following precoders are supported for two-layer transmission. , , Agreement For non-coherent uplink precoding by a 3TX UE, at least following precoders are supported for three-layer transmission. Agreement For a 3-antenna-port codebook-based UL transmission, study power split for each port of SRS and PUSCH. Agreement For codebook-based uplink transmission by a 3TX UE, support full-power Mode 0, subject to UE capability. RAN#116bis Agreement For SRS configuration supporting codebook-based UL transmission by a 3TX UE, support Alt1, - Alt1: Support configuration of X 4-port SRS resources in a resource set where one the ports is muted - FFS muting mechanism where X can be up to 2, subject to UE capability. Agreement For a 3TX UE, to support 3-port SRS transmission with reusing a 4-port SRS resource, support the following for muting one of the ports of the configured 4-port SRS resource, - Option 3: Always a same port is muted, e.g., the 4th port Agreement For a 3TX UE, to support 3-port SRS transmission with reusing a 4-port SRS resource, UE splits a linear SRS power equally across the 3 unmuted antenna ports of the 4-port SRS resource. RAN#117 Agreement - Update the agreement made in RAN1 #116bis as the following. Agreement For a 3TX UE, to support 3-port SRS transmission with reusing a 4-port SRS resource, support the following for muting one of the ports of the configured 4-port SRS resource, Option 3: Always a same port is muted, e.g., i.e., the 4th port.

Thus from RAN4 perspective, we need to figure out following aspects during the analysis towards 3Tx:

- Applicable frequency range and UE form factor

- UE architecture – Power class and corresponding PA configuration

- MPR requirement for 3Tx

- Frequency range

We think it would be straightforward that Rel-19 3Tx discussion should be applicable to FR1, mostly due to the fact that Rel-18 3Tx for inter-band UL CA had been studied for FR1 only, also because of the unambiguous correspondence between “Tx” and physical antenna port, upon which RF requirements for 2Tx and 4Tx have been specified respectively.

As for FR2, two simultaneously activated joint/UL TCI states were assumed during Rel-18 STxMP discussion. More related to physical implementation, the UE is assumed to be equipped with two panels (dual polarization directions per panel) and activated simultaneously during MultiRx discussion. Since it has never been determined whether “Tx” can be assumed as panel or polarization direction for FR2, we think FR2 can be precluded from 3Tx discussion here, not to mention that the demands and applicable scenarios for even more panels is unclear for FR2.

Proposal 1: For frequency range, suggest to clarify the RAN4 3Tx discussion scope under Rel-19 NR_MIMO_Ph5 WI as FR1 only.

- UE form factor

In Rel-18, 3Tx discussion was dedicated for non-handheld UE. We believe this is a reasonable limitation because non-handheld UE would acquire more space comparing to handheld UE in a way to handle 3Tx for single band and/or CA/DC operation. To better extent, concern on heat dissipation and power consumption could be alleviated. Thus similar approach as above can be considered, which is to follow Rel-18 3Tx scope and focus on non-handheld UE types for Rel-19 discussion.

Proposal 2: For UE form factor, suggest to clarify the RAN4 3Tx discussion scope under Rel-19 NR_MIMO_Ph5 WI as non-handheld UE only.

- Power class, PA configuration and MPR

In terms of power class, it is notably that only PC1.5 associated requirements were specified for 4Tx. As this is natural considering how adequate the RF resource is for a 4Tx UE (so it would be less attractive to consider lower power class), we prefer to only focus on PC1.5 for 3Tx as well.

Proposal 3: For the power class targeting 3Tx UE, suggest to clarify the RAN4 3Tx discussion scope under Rel-19 NR_MIMO_Ph5 WI as PC1.5 only.

Hence the corresponding PA configuration can be listed as in the below table, while the optional support of full power mode 0, note that other modes have been precluded by RAN1, cannot be indicated for PC1.5 by any of the listed PA configurations in the below table.

| Power class | PA configuration (with Mode 0) | PA configuration (w/o Mode 0) |
| --- | --- | --- |
| PC1.5 | N/A | 3x26dBm 23dBm+2x26dBm 2x23dBm+26dBm |

Regarding MPR requirement for 3Tx, we think it is reasonable to reuse 4Tx PC1.5 MPR requirement rather than the one defined for 2Tx due to the poor antenna isolation that could be introduced by the increased Tx number. Whether the set defined in Table 6.2D.2-4 or Table 6.2D.2-5 is selected could be further studied.

Proposal 4: Reuse 4Tx PC1.5 MPR for 3Tx PC1.5 MPR requirement. FFS whether the one defined in TS 38.101-1 Table 6.2D.2-4 or Table 6.2D.2-5 can be selected.

Conclusion

In this contribution we discussed on the RF requirements for Rel-19 NR MIMO. We have the following proposals:

Proposal 1: For frequency range, suggest to clarify the RAN4 3Tx discussion scope under Rel-19 NR_MIMO_Ph5 WI as FR1 only.

Proposal 2: For UE form factor, suggest to clarify the RAN4 3Tx discussion scope under Rel-19 NR_MIMO_Ph5 WI as non-handheld UE only.

Proposal 3: For the power class targeting 3Tx UE, suggest to clarify the RAN4 3Tx discussion scope under Rel-19 NR_MIMO_Ph5 WI as PC1.5 only.

Proposal 4: Reuse 4Tx PC1.5 MPR for 3Tx PC1.5 MPR requirement. FFS whether the one defined in TS 38.101-1 Table 6.2D.2-4 or Table 6.2D.2-5 can be selected.

- References

- RP-240087, “WID revision: NR MIMO Phase 5”, Samsung, 3GPP TSG RAN Meeting #104, Maastricht, Netherlands, March 18-21, 2024.

- R1-2401798, “FL Summary Support for 3TX CB-based Uplink; Third Round”, Moderator (InterDigital), 3GPP TSG-RAN1 Meeting #116, Athens, Greece, February 26-March 1, 2024.

- R1-2402085, “FL Summary Support for 3TX CB-based Uplink; Second Round”, Moderator (InterDigital), 3GPP TSG-RAN1 Meeting #116bis, Changsha, China, April 15-19, 2024.

- R1-2403851, “FL Summary Support for 3TX CB-based Uplink; First Round”, Moderator (InterDigital), 3GPP TSG-RAN1 Meeting #117, Fukuoka, Japan, May 20-24, 2024.