3GPP TSG-RAN WG4 Meeting #112                                        R4-2412577

Maastricht, Netherlands, August 19 - 23, 2024

Source: 	Huawei, HiSilicon

Title: 	On potentially new requirements for SBFD

Agenda Item:	8.19.2.1

Document for:	Approval

Introduction

During RAN4#111, potentially new requirements for SBFD have been discussed. Following conclusions are reproduced from [1].

- Issue 4-3-1: Requirement for transient period between SBFD and non-SBFD - Agreement: - FFS requirement and cases for transient period between SBFD and non-SBFD: - Option 1: The existing TDD BS transmitter transient period, i.e., 10us for FR1 and 3us for FR2-1 - Other options are not precluded. - FFS location of transient period between SBFD and non-SBFD: - Option 1: The transmitter transient period shall be located within the SBFD slot - Other options are not precluded. - Issue 4-3-2: Requirement for transient period between different SBFD configurations - Agreement: - FFS requirement for transient period between different SBFD configurations

In this contribution, we would like to share our views regarding this topic.

Discussion

The requirement for transient period for SBFD

In previous meeting, conclusion on this issue was almost achieved but companies are allowed to further check. As one conclusion captured in TR 38.858, transmitter transient period is needed for SBFD operation, between the non-SBFD slot and SBFD slot and vice versa, or when SBFD reconfiguration happens.

We share the view regarding the necessity of introducing TP between non-SBFD slot and SBFD slot, since RF resources rearrangement, e.g. for antenna array, could be needed from BS perspective.

In TS 38.104, the transmitter transient period requirement for TDD BS is defined as below.

The transmitter transient period is the time period during which the transmitter is changing from the transmitter OFF period to the transmitter ON period or vice versa. The transmitter transient period is illustrated in figure 6.4.2.1-1. Transmitter output power Time Transmitter ON period (DL transmission) Transmitter OFF period Transmitter OFF period Transmitter transient period OFF power level ON power level UL transmission GP or UL transmission Figure 6.4.2.1-1: Example of relations between transmitter ON period, transmitter OFF period and transmitter transient period Table 6.4.2.2-1: Minimum requirement for the transmitter transient period for BS type 1-C and BS type 1-H Transition; Transient period length (µs). OFF to ON; 10. ON to OFF; 10.

Still, our thinking is that 10us can be reused and it can be reviewed if any particular concern from RF perspective can be identified in future meetings.

Proposal 1: For SBFD-capable BS, check if 10us can be reused as transmitter transient period for following cases:

- Case #1: Between SBFD slot and non-SBFD slot (The transmitter transient period is within the SBFD slot)

- Case #2: Between SBFD slots with different subband configurations

In-channel adjacent subband requirement for SBFD

On one hand, we do share the understanding that in-channel adjacent subband requirement could be needed in order to guarantee certain performance level under self-interference. On the other hand, the potential solutions to overcome that would highly depend on vendors but very hard to be reflected in this new requirement since it must be based on specific implementation assumptions. Somehow, the REFSENS requirement for SBFD will reflect the expected suppression on the self-interference from feasible solutions.

Observation: For in-channel adjacent subband leakage ratio derivation, it is difficult for RAN4 to agree on a reference scheme for self-interference suppression. While the REFSENS for SBFD will reflect such suppression to some extent.

We also have similar feelings for other intra-operator gNB-gNB interference e.g. inter-site or co-site inter-sector cases. It could be hard to align companies understanding regarding power level, interference handling solutions or other issues that will also be involved not only in requirements derivation but also in test setup.

Proposal 2: Prefer not to introduce new requirement for in-channel adjacent subband leakage ratio.

Proposal 3: Prefer not to introduce new requirement for in-channel adjacent subband selectivity or blocking.

Conclusion

In this contribution we discussed on the general aspects for SBFD. We have the following proposals:

Observation: For in-channel adjacent subband leakage ratio derivation, it is difficult for RAN4 to agree on a reference scheme for self-interference suppression. While the REFSENS for SBFD will reflect such suppression to some extent.

Proposal 1: For SBFD-capable BS, check if 10us can be reused as transmitter transient period for following cases:

- Case #1: Between SBFD slot and non-SBFD slot (The transmitter transient period is within the SBFD slot)

- Case #2: Between SBFD slots with different subband configurations

Proposal 2: Prefer not to introduce new requirement for in-channel adjacent subband leakage ratio.

Proposal 3: Prefer not to introduce new requirement for in-channel adjacent subband selectivity or blocking.

- References

- R4-2409958, “Way Forward for [111][313] NR_duplex_evo”, Samsung, 3GPP TSG-RAN4 Meeting #111, Fukuoka, Japan, May 20-24, 2024.