3GPP TSG-RAN WG4 Meeting#112							         			R4-2412579

Maastricht, Netherland, 19 – 23 August 2024

Title: 	On reduced MPR when BS CBW is larger than UE CBW

Source: 	Skyworks Solutions, Inc.

Agenda Item:	8.1.1.2.1	Power domain enhancements for single carrier	[NR_ENDC_RF_Ph4-Core]

Document for:	Approval

# 1	Background

In RAN4#111, a way forward [2] captured agreements related to the Release 19 power domain enhancement work item. Notably, it covered the MPR reduction for the case where the UE CBW is smaller than the BS CBW. In this contribution, further developing from our contribution [1] in RAN4#111, we propose a simple way of reusing the inner/outer MPR concept, but applied to the BS CBW.

# Discussion

## 2.1 Way forward inputs

Below in italics, we extracted the agreements and way forwards relative to single CC MPR reduction as captured in [2].

Agreement in main session:

- No relaxation of level of spurious emission requirements

  - FFS on whether to change the spurious emission boundary

Way forward:

- Study the approach to convert full RB allocation in UE CBW to “inner RB allocation” with an extended UE BW, e.g. each side of the UE BW is equal to ½ of the UE BW, inside a larger BS CBW as starting point with consideration of the following aspects

  - The edge of the extended UE BW, i.e. the UE CBW plus the shifted frequency symmetrically at each side of the UE CBW, should be inside BS CBW or at least aligned with the BS CBW edge

    - ① FFS feasibility of case where extended UE CBW edge exceeds the BS CBW edge, i.e. gap between edges of UE CBW and BS CBW < 1/2 UE CBW

    - ② FFS whether IBE is used between edges of UE CBW and BS CBW or between edges of UE CBW and extended UE CBW

    - ③ FFS ACLR and SEM are applicable from the edge of extended UE CBW or edge of BS CBW, i.e. the start point of ΔfOOB

    - ④ FFS integral region and boundary of OOBE is based on extended UE CBW or UE CBW

- Other mechanisms are not precluded.

In this contribution, we discuss a simple way to achieve MPR reduction by reusing the inner/outer concept. but applied to the BS CBW rather than the UE CBW.

## 2.2 Existing inner/outer MPR characteristic

For the QPSK and 16QAM, which are the main candidates for reduced MPR as they are not limited by EVM (while EVM for 64 QAM may become marginal if MPR is reduced), the inner/outer/edge aspects have different limitations:

- Inner QPSK and 16QAM are IBE limited

- Outer QPSK and 16QAM are SEM or ACLR limited.

- Edge allocations are limited by SEM but related to the windowing filtering done at BB.

When the BS CBW is larger than the UE CBW, essentially the SEM and ACLR should apply based on the BS CBW, as the interference with RBs within the BS CBW but outside the UE CBW should be guaranteed by respecting IBE, which both inner and outer MPR guarantees.

A simple way to improve the MPR for a large number of allocations is to re-assess the inner, outer, and edge boundaries based on the BS CBW as illustrated in Figure 1:

- On the left side, the classical allocation triangle based on the UE CBW where the inner allocations (green) region is limited to half the UE CBW and their position at half the allocation size from the channel edge; other allocations are outer (yellow), except for PC2 and PC1.5 where edge (red) allocations are needed.

- On the right side, the allocation triangle is now based on the BS CBW and a new UE allocation triangle can be defined by shifting the UE based triangle within the BS defined inner, outer, and edge regions by its RBstart position within the BS RBs.

Figure 1: illustration of UE based, and BS based allocation triangle defining inner, outer and edge allocations.

Observations on BS based allocation triangle:

- The inner allocation size can reach half of BS CBW (if the UE CBW is > ½ BS CBW).

- Many LCRB are inner whatever their position.

- No more edge allocations.

- Outer region is very small.

This change can be simply made by the UE by recalculating the inner, outer, and edge regions with the following parameter changes in the inner and edge equations:

- UE NRB is replaced by BS NRB

- UE RBstart is replaced by RB_BSleft+UE_RBstart (shifted RBstart)

- UE LCRB is kept

- For the UE to autonomously calculate this, it only needs to get from the BS the BS NRB and BS RBstart of the UE allocated RBs.

  - Note that the BS can still have the flexibility to report a smaller NRB and RBstart for margin on emissions.

Proposal: RAN4 to study a new inner, outer and edge allocation definition for MPR based on BS CBW by using in the already defined inner and edge equations:

- BS NRB instead of UE NRB.

- Adding the amount of RB by which the UE RBs are shifted within the BS RBs to the UE RBstart.

- The base station shall signal the extended NRB and RBstart shift for the UE to calculate the new inner, outer, and edge regions and reduce MPR.

- For margin or stricter emissions, the BS can signal a reduced NRB and RBshift.

# Conclusion

In this contribution, we extended the inner, outer and edge allocation triangle concept used for the UE MPR by using the BS parameters instead and make the following proposal.

Proposal: RAN4 to study a new inner, outer and edge allocation definition for MPR based on BS CBW by using in the already defined inner and edge equations:

- BS NRB instead of UE NRB.

- Adding the amount of RB by which the UE RBs are shifted within the BS RBs to the UE RBstart.

- The base station shall signal the extended NRB and RBstart shift for the UE to calculate the new inner, outer, and edge regions and reduce MPR.

- For margin or stricter emissions, the BS can signal a reduced NRB and RBshift.

# References

[1] R4-2407404 On emission requirement relaxations for power domain enhancements, Skyworks Solutions Inc., RAN4#111

[2] R4-2410749 WF on power domain enhancements, Huawei, HiSilicon, RAN4#111