3GPP TSG-RAN WG4 Meeting # 112	R4-2412588

Maastricht, Netherlands, 19th – 23rd August 2024

Agenda Item:	8.2.3

Source: 	Nokia

Title: 	BS parameters for 7125 to 8400 MHz frequency range

Document for:	Approval

# 1.	Introduction

The SI on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz was approved at TSG RAN#103 [1]. This study item aims as answering requests from ITU-R WP5D regarding NR in these frequencies for IMT [2].

One of the objectives of this SI is to study the IMT parameters relevant for sharing and compatibility for 7125 to 8400 MHz frequency range. This topic was discussed at TSG RAN4#111 and the WF was agreed [3].

This contribution further discusses the BS parameters based on the agreed WF.

# 2.	Discussion

2.1		BS Emissions mask

It is stated in the agreed WF.

Issue 2-7: Emissions mask Agreement: - AAS BS: - Use n104 unwanted emissions mask - [Aim to use 100MHz for delta_obue, conclude feasibility until RAN4#112] - Non-AAS BS: - [n104 unwanted emissions mask for IMT response] - Double check on whether there are any concerns towards sharing by RAN4#112 and in case concerns are identified, consider n96 if needed to resolve concerns

The emission mask parameters inside square brackets above have been specified for n104 in TS 38.104 [4], so they should be feasible for practical implementation and thus be used in the reply to ITU-R WP5D for sharing and compatibility for 7125 to 8400 MHz frequency range.

2.2		BS antenna array parameters

It is stated in the agreed WF.

Issue 2-14: BS antenna array parameters Agreement: Rural macro (If it’s available) Parameters proposed by ZTE; Suburban macro; Urban macro; Urban small cell (outdoor)/Micro cell; Indoor (small cell). 1; Base station antenna characteristics. 1.6; Antenna array configuration (Row × Column) (Note 4); 8 × 8 elements; Option 1: 8 × 16 (Nokia, Ericsson, Huawei, Spark) Option 2: 8 x 8 (ZTE, Samsung); Option 1: 8 × 16 (Nokia, Ericsson, Huawei, Cable Labs) Option 2: 8 x 8 (ZTE, Samsung); 8 × 8; 4 × 4. 1.7a; Number of element rows in sub-array, Msub; Option 1: 4 (Nokia, Spark, Cable Labs, Charter) Option 2: 3 (Ericsson, Huawei, Samsung); Option 1: 4 (Nokia, Spark, Cable Labs, Charter) Option 2: 3 (Ericsson, Huawei, Samsung); N/A; N/A. Agreement: - Remove the rural macro scenario from the table. - non-AAS can be used for Urban small cell/Micro cell scenario - non-AAS can be used for indoor scenario - FFS on where to capture the above two bullets

Figures 1 and 2 below show the vertical composite array pattern at the azimuth angle of 0° and the horizontal composite array pattern at the elevation angle of 100° (including a 10° mechanical downtilt), using the three BS antenna array options of 8x16 (Sub Array size 4), 8x16 (Sub Array size 3), and 8x8 (Sub Array size 3).

(a) 8x16 (Sub Array size 4)		(b) 8x16 (Sub Array size 3)			(c) 8x8 (Sub Array size 3)

Figure 1: Vertical Composite Array Pattern (0° azimuth angle)

(a) 8x16 (Sub Array size 4)		(b) 8x16 (Sub Array size 3)			(c) 8x8 (Sub Array size 3)

Figure 2: Horizontal Composite Array Pattern (100° elevation angle)

The urban macro simulation results of the downlink throughput loss of the victim UE using 38 dB BS ACLR and 33 dB UE ACS with 46 dBm BS maximum output power and 12 dB UE noise figure are provided in Table 1 below. The urban macro simulation results of the uplink throughput loss of the victim UE using 30 dB UE ACLR and 42 dB BS ACS with 23 dBm UE maximum output power and 6 dB BS noise figure are provided in Table 2 below. Other simulation assumptions and parameters follow those recorded in TR 38.921 [5].

| Case | Loss |
| --- | --- |
| Average throughput loss (a) | 0.0063 |
| 5%-tile throughput loss (a) | 0.0017 |
| Average throughput loss (b) | 0.0066 |
| 5%-tile throughput loss (b) | 0.0042 |
| Average throughput loss (c) | 0.0100 |
| 5%-tile throughput loss (c) | 0.0065 |

Table 1: Urban Macro DL throughput loss of victim UE

| Case | Loss |
| --- | --- |
| Average throughput loss (a) | 0.0039 |
| 5%-tile throughput loss (a) | 0.0012 |
| Average throughput loss (b) | 0.0040 |
| 5%-tile throughput loss (b) | 0.0205 |
| Average throughput loss (c) | 0.0071 |
| 5%-tile throughput loss (c) | 0.0252 |

Table 2: Urban Macro UL throughput loss of victim UE

It can be seen from the tables that both downlink and uplink 5%-tile throughput losses are below 1% using 8x16 (Sub Array size 4), while the uplink 5%-tile throughput loss (i.e., cell edge coverage) is above 2% using 8x16 (Sub Array size 3) or 8x8 (Sub Array size 3). Therefore, BS antenna array 8x16 (Sub Array size 4) should be adopted to achieve better cell edge coverage for 7125 to 8400 MHz frequency range.

# 3.	Conclusion

This contribution further discusses the BS parameters based on the agreed WF.

Proposal 1: Use band n104 unwanted emission mask and delta_obue for AAS and non-AAS BS.

Proposal 2: Adopt 8x16 (Sub Array size 4) as BS antenna array.

# References

[1]	RP-240787, “New SI proposal: Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz”, Ericsson.

[2]	R4-2400333, “Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35 GHz”, ITU-R WP5D.

[3]	R4-2410741, “WF on IMT parameters study”, Ericsson.

[4]	3GPP TS 38.104 v18.6.0, “NR; Base Station (BS) radio transmission and reception”.

[5]	3GPP TR 38.921 v18.0.1, “NR; Study on International Mobile Telecommunications (IMT) parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5 GHz”.