3GPP TSG-RAN WG4 Meeting # 112	R4-2412591

Maastricht, Netherlands, 19th – 23rd August 2024

Agenda Item:	8.2.4.2

Source: 	Nokia

Title: 	BS antenna and simulation parameters for 14800 to 15350 MHz frequency range

Document for:	Approval

# 1.	Introduction

The SI on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz was approved at TSG RAN#103 [1]. One of the objectives of this SI is to study the IMT parameters relevant for sharing and compatibility for 14800 to 15350 MHz frequency range.

This topic was discussed at TSG RAN4#111 and the WF was agreed [2].

This contribution further discusses the BS antenna and simulation parameters based on the agreed WF and the simulation results provided in [3].

# 2.	Discussion

2.1		BS antenna array size

It is stated in the agreed WF.

Issue 3-6: BS antenna array size Agreement: Per polarization, for simulation only: - 2048 (Sub Array size 8) - 1536 (Sub Array size 4) - Array size and Sub Array size for the response will be decided later, taking into account feasibility

The simulation results in [3] show that 5%-tile throughput (i.e., cell edge coverage) cannot be achieved using 64x32=2048 (Sub Array size 8) BS antenna configuration with FR1 like UE. This can be explained by the fact that higher grating lobes are formed in the vertical composite array pattern by the larger sub-array size of 8 while the horizontal composite array patterns are similar for both BS antenna configurations, which means that higher interference is received by the serving BS from other UEs in the neighbouring cells of the same network. Figures 1 and 2 below show the vertical composite array pattern at the azimuth angle of 0° and the horizontal composite array pattern at the elevation angle of 96° (including a 6° mechanical downtilt). Therefore, it is proposed to use 64x24=1536 (Sub Array size 4) BS antenna configuration to achieve cell edge coverage with FR1 like UE. Grating lobes are present in both configurations but the 8 element sub-array creates more grating lobes resulting in coexistence issues.

(a) 64x24 BS									(b) 64x32 BS

Figure 1: Vertical Composite Array Pattern

(a) 64x24 BS									(b) 64x32 BS

Figure 2: Horizontal Composite Array Pattern

2.2		ISD and Percentage indoor users for urban macro

It is stated in the agreed WF.

Issue 3-3: ISD Agreement: - For indoor, agree 20m - For outdoor, consider both 450m (1st priority) and 350 (2nd priority) until August. - Also other ISD not precluded as 3rd priority than 450 and 350. Issue 3-4: Percentage indoor users for urban macro Agreement: - Keep both options

The simulation results in [3] show that 5%-tile throughput (i.e., cell edge coverage) can be achieved with 450 m ISD and 20% indoor UE ratio except for the case using 64x32=2048 (Sub Array size 8) BS antenna configuration with FR1 like UE. Therefore, it is proposed to prioritize 450 m and 20% indoor UE ratio with the use of 64x24=1536 (Sub Array size 4) BS antenna configuration.

2.3		Co-ordinated and un-coordinated for outdoor

It is stated in the agreed WF.

Issue 3-5: Co-ordinated and un-coordinated for outdoor Agreement: - Both co-ordinated and un-coordinated

The simulation results in [3] show that the throughput losses for FR1 like UE are higher with un-coordinated operation than with coordinated operation, while the throughput losses for FR2 like UE are higher with coordinated operation than with un-coordinated operation. Therefore, it is proposed to apply un-coordinated operation with FR1 like UE and coordinated operation with FR2 like UE as these are more stringent cases for throughput losses.

2.4		UE type

It is stated in the agreed WF.

Issue 3-8: UE type Agreement: - For simulations, consider both options. - FR1 like - RX diversity gain: [0 or 5] dB assuming 4RX - TX: 0dBi omnidirectional - Max Power: 23 dBm, 26dBm - FR2 like - Two panels (one in each direction) 2x2 antenna - 5dBi element gain. Array gain comes on top - Power: 23 dBm as max TRP

The simulation results in [3] show that 5%-tile throughput (i.e., cell edge coverage) can be achieved with 23 dBm maximum UE transmit power except for the case using 64x32=2048 (Sub Array size 8) BS antenna configuration with FR1 like UE. Therefore, it is proposed to prioritize 23 dBm maximum UE transmit power with the use of 64x24=1536 (Sub Array size 4) BS antenna configuration.

# 3.	Conclusion

This contribution further discusses the BS antenna and simulation parameters based on the agreed WF at TSG RAN4#111 and the obtained simulation results.

Proposal 1: Use 64x24=1536 (Sub Array size 4) BS antenna configuration.

Proposal 2: Prioritize 450 m and 20% indoor UE ratio with the use of 64x24=1536 (Sub Array size 4) BS antenna configuration.

Proposal 3: Apply un-coordinated operation with FR1 like UE and coordinated operation with FR2 like UE.

Proposal 4: Prioritize 23 dBm maximum UE transmit power with the use of 64x24=1536 (Sub Array size 4) BS antenna configuration.

# References

[1]	RP-240787, “New SI proposal: Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz”, Ericsson.

[2]	R4-2410741, “WF on IMT parameters study”, Ericsson.

[3]	R4-2412589, “Urban macro coexistence simulation results for 14800 to 15350 MHz frequency range”, Nokia.