3GPP TSG-RAN WG4 Meeting # 112	R4-2412594

Maastricht, Netherlands, 19th – 23rd August 2024

Agenda Item:	8.4.2

Source: 	Nokia

Title: 	UE RF requirements for inter-band NR CA/DC with 3MHz CBW

Document for:	Approval

# 1.	Introduction

The revised WI for NR channel BW less than 5MHz for FR1 Phase 2 was approved at TSG RAN#104 [1]. One of the objectives of this WI is to:

- Define common co-located and non-co-located inter-band NR CA/DC UE RF requirements with 3MHz CBW in the one band and 5MHz or 10MHz CBW in the other band - Example band combination: CA/DC of 3MHz in band n100 and 5MHz or 10MHz in band n101

This topic was discussed at TSG RAN4#111, and the Way Forward was agreed [2].

This contribution provides proposals to progress the issues that were FFS in the agreed WF.

# 2.	Discussion

Two issues were FFS in the agreed WF as follows:

### Issue 1-6: UE capability and network signalling - FFS whether any updates to Rel-18 UE capability signalling are needed to enable less than 5 MHz CBW operation for CA/DC. - Further potential enhancements can be further discussed for SCell with less than 5MHz. - FFS whether UEs needs to be informed that SCell has 12 PRB or 20 PRB transmission bandwidth for 3MHz or 5MHz CBW, respectively ## Issue 1-7: Sync raster applicability for Scell with < 5MHz CBW for CA_n100-n101 - FFS whether any restrictions on ARFCN for SSB locations for SCell SSB need to be defined. - Option 1: Require less than 5 MHz SCell to be associated with the new sync raster points for less than 5MHz, similar as PCell. - Other options are not precluded

## Issue 1-6: UE capability and network signalling

On whether any updates to Rel-18 UE capability signalling are needed to enable less than 5 MHz CBW operation for CA/DC:

Currently, UE capability signalling related to less than 5 MHz as well as 12 PRB or 20 PRB transmission bandwidth for 3MHz or 5MHz CBW are specified in TS 38.306 [3] as follows:

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| support12PRB-CORESET0-r18 Indicates whether the UE supports reception of 12 PRB CORESET0 with an associated SS/PBCH block that is located according to Table 5.4.3.1-2 in TS 38.101-1 [2]. A UE supporting this feature shall also indicate support of support-3MHz-ChannelBW-Symmetric-r18. This feature is supported for 15kHz SCS only. This feature is only applicable to single-carrier operation. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE: The UE supporting this capability supports configuration of 12 PRB BWP operation. | Band | No | FDD only | FR1 only |
| support3MHz-ChannelBW-Asymmetric-r18 Indicates whether the UE supports 3 MHz channel bandwidth in uplink with larger than 3 MHz channel BW in DL, including short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS. This feature is supported for 15kHz SCS only. It is applicable only to single-carrier operation and applies to bands where the UE indicates support for asymmetricBandwidthCombinationSet with 3 MHz UL according to clause 5.3.6 of TS 38.101-1 [2]. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE 1: The UE supporting this feature supports configuration of 15 PRB UL BWP operation. NOTE 2: If the UE indicates support in asymmetricBandwidthCombinationSet for a 3MHz UL in a band according to clause 5.3.6 of 38.101-1 [2], this feature shall be indicated for the band. | Band | No | FDD only | FR1 only |
| support3MHz-ChannelBW-Symmetric-r18 Indicates whether the UE supports 3 MHz symmetric channel bandwidth in DL and UL, including the following functional components: - Reception of 12 PRB PBCH based on RB-level puncturing; - Short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS; - Reception of 15 PRB CORESET0. This feature is supported for 15kHz SCS only. It is applicable only to single-carrier operation and when an associated SS/PBCH block is located according to Table 5.4.3.3-2 in TS 38.101-1 [2]. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE: The UE supporting this capability supports configuration of 15 PRB BWP operation in DL and UL. | Band | No | FDD only | FR1 only |
| support5MHz-ChannelBW-20PRB-CORESET0-r18 Indicates whether the UE supports short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS, and the reception of 20 PRB CORESET0. This feature is supported for 15 kHz SCS only. This feature is only applicable when an associated SS/PBCH block is located in band n100 at GSCN 41638 of Table 5.4.3.1-3 in TS 38.101-1 [2]. This feature is only applicable to single-carrier operation. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE: The UE supporting this feature supports configuration of 20 PRB BWP operation. | UE | No | FDD only | FR1 only |
| support12PRB-CORESET0-GSCN-41637-r18 Indicates whether the UE supports reception of 12 PRB CORESET0 with an associated SS/PBCH block located at GSCN 41637. A UE supporting this feature shall also indicate support of support3MHz-ChannelBW-Symmetric-r18. This feature is supported for 15 kHz SCS only. This feature is only applicable when an associated SS/PBCH block is located in band n100 at GSCN 41637 of Table 5.4.3.1-3 in TS 38.101-1 [2]. NOTE: The UE supporting this FG supports configuration of 12 PRB BWP operation. This feature is only applicable to single-carrier operation. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. | UE | No | FDD only | FR1 only |

It can be seen from the yellow highlighted texts above that the current UE capability signalling are only applicable to single-carrier operation. Therefore, UE capability signalling for less than 5 MHz CBW operation for CA/DC need to be defined in TS 38.306, which was added to the impacted specification list in the revised WID in [1].

On whether UEs needs to be informed that SCell has 12 PRB or 20 PRB transmission bandwidth for 3MHz or 5MHz CBW, respectively:

If less than 5 MHz SCell is required to be associated with the new sync raster points for less than 5MHz similar as PCell, then there is no need to inform the UE that SCell has 12 PRB or 20 PRB transmission bandwidth for 3MHz or 5MHz CBW, because this information is given by the sync raster points specifically defined for 12 PRB transmission bandwidth (GSCN 41637) and 20 PRB transmission bandwidth (GSCN 41638), and the CORESET#0 indication is given by the initial BWP IE (SCellConfig > ServingCellConfigCommon > DownlinkConfigCommon > initialDownlinkBWP) to be 12 or 15 PRB transmission bandwidth for other sync raster points defined for less than 5MHz.

## Issue 1-7: Sync raster applicability for Scell with < 5MHz CBW for CA_n100-n101

As discussed above, requiring less than 5 MHz SCell to be associated with the new sync raster points for less than 5MHz similar as PCell will simplify UE signalling, and this should not pose significant deployment limitations for band n100. Therefore, option 1 in the WF should be confirmed.

# 3.	Conclusion

This contribution has provided proposals to progress the issues that were FFS in the agreed WF in last RAN4 meeting.

Proposal 1: UE capability signalling for less than 5 MHz CBW operation for CA/DC need to be defined in TS 38.306.

Proposal 2: No need to inform the UE that SCell has 12 PRB or 20 PRB transmission bandwidth for 3MHz or 5MHz CBW.

Proposal 3: Require less than 5 MHz SCell to be associated with the new sync raster points for less than 5MHz, similar as PCell.

# References

[1]	RP-241622, “Revised WID: NR channel BW less than 5MHz for FR1 Phase 2”, Intel Corporation.

[2]	R4-2410602, “WF on UE RF requirements for Rel-19 less than 5MHz”, Intel.

[3]	3GPP TS 38.306 v18.2.0: “NR; User Equipment (UE) radio access capabilities”.