3GPP TSG-RAN WG4 Meeting # 112	R4-2412595

Maastricht, Netherlands, 19th – 23rd August 2024

Agenda Item:	8.22.3

Source: 	Nokia

Title: 	BS RF requirements for low-power wake-up signal for NR

Document for:	Approval

# 1.	Introduction

The revised WI for low-power wake-up signal and receiver for NR was approved at TSG RAN#103 [1]. One of the objectives of this WI is to:

- Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS. - Current NR BS requirements is baseline

This topic was discussed at TSG RAN4#111, and the Way Forward proposals were agreed [2].

This contribution provides proposals to progress the issues that were FFS in the agreed WF.

# 2.	Discussion

2.1	Manufacture declaration on LP-WUS

It is stated in the agreed WF as follows:

FFS on the following options for next meeting: - Option 1: Set the LP-WUS power boosting a complete manufacture declaration feature, including whether supporting LP-WUS power boosting and the supported boosting level. - Option 2: Minimum power boosting level in core specification together with manufacturer declaration in the conformance test specification.

Currently, the RE power control dynamic range requirement for NR BS is specified in TS 38.104 [3] as follows:

### 6.3.2 RE power control dynamic range #### 6.3.2.1 General The RE power control dynamic range is the difference between the power of an RE and the average RE power for a BS at maximum output power (Pmax,c,AC or Pmax,c,TABC) for a specified reference condition. For BS type 1-C this requirement shall apply at the antenna connector supporting transmission in the operating band. For BS type 1-H this requirement shall apply at each TAB connector supporting transmission in the operating band. #### 6.3.2.2 Minimum requirement for BS type 1-C and BS type 1-H RE power control dynamic range: Table 6.3.2.2-1: RE power control dynamic range Modulation scheme used; RE power control dynamic range (dB). on the RE; (down); (up). QPSK (PDCCH); -6; +4. QPSK (PDSCH); -6; +3. 16QAM (PDSCH); -3; +3. 64QAM (PDSCH); 0; 0. 256QAM (PDSCH); 0; 0. 1024QAM (PDSCH); 0; 0. NOTE: The output power per carrier shall always be less or equal to the maximum output power of the base station..

In table 6.3.2.2-1 above, the RE power control dynamic range requirement is specified depending on the modulation schemes and physical channels. The RE power up and down range for each modulation scheme was derived by considering the relation between RE power boosting/de-boosting and other RF requirements like UEM, ACLR and EVM, as explained in TR 38.817-02 [4].

Since LP-WUS/LP-SS is a new physical channel using On-Off Keying (OOK) modulation, a new core requirement shall be added for the dynamic range for LP-WUS/LP-SS. In table 6.3.2.2-1 above, the RE power up and down range for modulation order higher than 16QAM is 0 dB considering the impact on PA with the higher PAPR of higher order modulation. It is shown in Figure 1 below that the PAPR of LP-WUS/LP-SS is more like 64QAM than 16QAM. Therefore, the core requirement for the dynamic range for LP-WUS/LP-SS shall be specified the same as 64QAM, i.e., 0 dB.

(a) LP-WUS (bit sequence 1101) using ZC sequence with M=1

(b) LP-WUS (bit sequence 1101) using ZC sequence with M=2

(c) LP-WUS (bit sequence 1101) using ZC sequence with M=4

(d) LP-WUS (bit sequence 1101) using M sequence with M=1

(e) LP-WUS (bit sequence 1101) using M sequence with M=2

(f) LP-WUS (bit sequence 1101) using M sequence with M=4

(g) Typical QAM signals

Figure 1: PAPR comparison of LP-WUS and typical QAM signals

On the other hand, support of higher levels of LP-WUS power boosting will increase BS implementation complexity, like NB-IoT power boosting in TS 36.141 [5] and TS 38.141-1 [6], BS manufacturers should be allowed to declare the supported LP-WUS power boosting level in the manufacturer declarations for BS supporting LP-WUS operation to differentiate their product performance.

2.2	Concept of LP-WUS dynamic range/power boosting

It is stated in the agreed WF as follows:

FFS on the following options for next meeting: - Option 1: Stick with the one in RAN4#110bis approved WF R4-2406140, which is: - The LP-WUS RB power dynamic range (or LP-WUS power boosting) is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs). - Option 2: Define EPRE ratio between LP-WUS and NR signals instead of power dynamic range.

As shown in clause 6.3.2 above, the current RE power control dynamic range requirement is specified with reference to the average RE power for a BS at maximum output power, and this is the same for the NB-IOT power boosting requirement with reference to the average power over all REs. Therefore, the definition of LP-WUS dynamic range/power boosting in RAN4#110bis approved WF R4-2406140 should continue to be used to avoid potential confusion among the various requirements related to power boosting.

2.3	Whether to preclude small CBW for consideration of LP-WUS power boosting

It is stated in the agreed WF as follows:

FFS on the following options for next meeting: - Option 1: Consider a power degradation limit, e.g., 2dB, for validating a configuration for LP-WUS representing by (EPRE ratio, channel bandwidth). - Option 2: Only consider LP-WUS power boosting for CBWs larger than 20MHz. - Option 3: Focus on CBWs >= 10MHz and different power boosting values can be considered for different CBW. - Option 4: Not preclude small CBW for LP-WUS power boosting.

On the other hand, it was agreed at RAN1#117 as follows [7]:

Update agreement in last meeting as below: From RAN1 perspective, support X PRBs for LP-WUS and LP-SS with SCS 30kHz (blanked guard RBs are not included) for a channel bandwidth equal or larger than 5MHz - X to be down-selected between = 11 and 12 PRBs - FFS the number of PRBs for 15kHz - FFS if other number of PRBs needed, for LP-SS and LP-WUS with a channel bandwidth equal or less than 5MHz FFS: Whether the above is applicable to FR2

Therefore, RAN1 has not precluded LP-SS and LP-WUS with a channel bandwidth equal or less than 5MHz. Besides, as discussed in section 2.1 above, the core requirement for the dynamic range for LP-WUS/LP-SS shall be specified the same as 64QAM, i.e., 0 dB, and BS manufacturers should be allowed to declare the supported LP-WUS power boosting level, as such there is no need to preclude small CBW for LP-WUS power boosting if BS manufacturers declare to support power boosting with small CBW for LP-WUS operation.

2.4	On applicable BS type for LP-WUS

It is stated in the agreed WF as follows:

FFS on the following proposals for next meeting: - Proposal 1: Not to set restriction on applicable BS types to support LP-WUS. - Proposal 2: To decide on the applicable BS types after the applicable frequency range and bands for LP-WUS have been decided. - Proposal 3: Consider BS type 1-C as applicable type to further discuss of LP-WUS power boosting. FFS other BS types. - Proposal 4: Depend on power boosting level for certain BS type. - If power boosting is limited to 3 dB, all BS type 1-C, 1-H and 1-O can be considered.

As discussed in section 2.1 above, the core requirement for the dynamic range for LP-WUS/LP-SS shall be specified the same as 64QAM, i.e., 0 dB, and BS manufacturers should be allowed to declare the supported LP-WUS power boosting level, as such there is no need to set restriction on applicable BS types for LP-WUS if BS manufacturers declare to support power boosting with LP-WUS operation for their BS types.

2.5	On minimum value for LP-WUS power boosting

It is stated in the agreed WF as follows:

FFS on the following options for next meeting: - Option 1: Use [3dB] as starting point - It should be considered in conjunction with the supported CBWs. - Option 2: To consider the power degradation of RBs other than LP-WUS signal within the carrier after the number of LP-WUS RBs have been decided in RAN1.

As discussed in section 2.3 above, it was agreed at RAN1#117 to support 11 PRBs for LP-WUS and LP-SS with SCS 30kHz. The power degradation of RBs other than LP-WUS signal for this setting with different LP-WUS power boosting levels are provided in Table 1 below, which shows that the power reduction for other RBs because of LP-WUS power boosting varies between 0.1 dB to 1.1 dB depending on the power boosting level and channel bandwidth.

Table 1: Impact of LP-WUS power boosting

| LP-WUS power boosting (dB) | SCS (KHz) | Number of RBs | LP-WUS RBs | Rated carrier output power (dBm) | Channel bandwidth (MHz) | Power reduction for other RBs (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 3 | 30 | 162 | 11 | 46 | 60 | -0.32687277 |
| 3 | 30 | 189 | 11 | 46 | 70 | -0.27568025 |
| 3 | 30 | 217 | 11 | 46 | 80 | -0.237165576 |
| 3 | 30 | 245 | 11 | 46 | 90 | -0.208095127 |
| 3 | 30 | 273 | 11 | 46 | 100 | -0.185374077 |
| 4 | 30 | 162 | 11 | 46 | 60 | -0.506770566 |
| 4 | 30 | 189 | 11 | 46 | 70 | -0.425992238 |
| 4 | 30 | 217 | 11 | 46 | 80 | -0.365577629 |
| 4 | 30 | 245 | 11 | 46 | 90 | -0.32017752 |
| 4 | 30 | 273 | 11 | 46 | 100 | -0.284811572 |
| 6 | 30 | 162 | 11 | 46 | 60 | -1.063293018 |
| 6 | 30 | 189 | 11 | 46 | 70 | -0.884288268 |
| 6 | 30 | 217 | 11 | 46 | 80 | -0.752987437 |
| 6 | 30 | 245 | 11 | 46 | 90 | -0.655701573 |
| 6 | 30 | 273 | 11 | 46 | 100 | -0.580711194 |

As discussed in section 2.1 above, the core requirement for the dynamic range for LP-WUS/LP-SS shall be specified the same as 64QAM, i.e., 0 dB, and BS manufacturers should be allowed to declare the supported LP-WUS power boosting level.

2.6	Whether a cap for LP-WUS power boosting should be considered

It is stated in the agreed WF as follows:

FFS on the following options for next meeting: - Option 1: Limit to 3dB for BS type 1-C, 1-H and 1-O. - Option 2: FFS on the value in conjunction with the supported CBWs. - Option 3: To consider the power degradation of RBs other than LP-WUS signal within the carrier after the number of LP-WUS RBs have been decided in RAN1. - Option 4: Not consider to cap the LP-WUS power boosting.

As discussed in section 2.1 above, the core requirement for the dynamic range for LP-WUS/LP-SS shall be specified the same as 64QAM, i.e., 0 dB, and BS manufacturers should be allowed to declare the supported LP-WUS power boosting level, as such there is no need to cap the LP-WUS power boosting if BS manufacturers declare to support higher levels of power boosting with LP-WUS operation.

2.7	Requirements other than power boosting that should be considered

It is stated in the WF as follows:

FFS on the following proposals for next meeting: - Proposal 1: Unwanted emissions requirements of SEM and spurious emissions should be considered for transmitted signal with LP-WUS and NR in the same carrier. - Proposal 2: FFS whether transmitted signal quality requirements should be defined for LP-WUS, at least for the EVM requirement. - Proposal 3: Multi-band requirements at gNB side for LP-WUS.

The RE power up and down range for each modulation scheme was derived by considering the relation between RE power boosting/de-boosting and other RF requirements like UEM, ACLR and EVM, as explained in TR 38.817-02 [4]. As discussed in section 2.1 above, the core requirement for the dynamic range for LP-WUS/LP-SS shall be specified the same as 64QAM, i.e., 0 dB, and BS manufacturers should be allowed to declare the supported LP-WUS power boosting level, as such there is no need to adjust other requirements like UEM, ACLR and EVM, and the same LP-WUS/LP-SS power boosting requirement and declaration can be applicable to multi-band BS.

# 3.	Conclusion

This contribution has provided proposals to progress the issues that were FFS in the agreed WF in last RAN4 meeting.

Proposal 1: The core requirement for the dynamic range for LP-WUS/LP-SS shall be specified the same as 64QAM, i.e., 0 dB, and BS manufacturers should be allowed to declare the supported LP-WUS power boosting level.

Proposal 2: The definition of LP-WUS dynamic range/power boosting in RAN4#110bis approved WF R4-2406140 should continue to be used to avoid potential confusion among the various requirements related to power boosting.

If proposal 1 is agreed, then:

Proposal 3: There is no need to preclude small CBW for LP-WUS power boosting.

Proposal 4: There is no need to set restriction on applicable BS types for LP-WUS.

Proposal 5: There is no need to cap the LP-WUS power boosting.

Proposal 6: There is no need to adjust other requirements like UEM, ACLR and EVM, and the same LP-WUS/LP-SS power boosting requirement and declaration can be applicable to multi-band BS.

# References

[1]	RP-240801, “Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)”, vivo, NTT DOCOMO, Ericsson, MediaTek, Samsung, Sony.

[2]	R4-2409956, “Way Forward for [111][314] NR_LPWUS”, Huawei, HiSilicon.

[3]	3GPP TS 38.104 v18.6.0: “NR; Base Station (BS) radio transmission and reception”.

[4]	3GPP TR 38.817-02 v15.11.0: “NR; General aspects for Base Station (BS) Radio Frequency (RF) for NR”.

[5]	3GPP TS 36.141 v18.4.0: “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) conformance testing”.

[6]	3GPP TS 38.141-1 v18.5.0: “NR; Base Station (BS) conformance testing Part 1: Conducted conformance testing”.

[7]	RP-241039, “Status report for WI: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)”, vivo.