3GPP TSG RAN WG4 Meeting #112	R4-2412599

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	5.14.1

Source: 	vivo

Title: 	Discussion on maintenance of RRM requirements for R18 RRM_enh3

Document for:	Discussion

- Introduction

In RAN4 #111 meeting, RAN4 continued the discussion on remain issues for RRM requirements for R18 RRM_enh3. The corresponding progress was captured in [1] for part 1 and [2] for part 2, respectively. In the contribution, we would like to provide our analysis on the remain issues for these two parts.

- Discussion on maintenance of Part 1

Issue 1-4: Other issues related with FR1 SCell activation enhancement - FFS following proposals: - Proposal 1: - RAN4 to discuss if measurement period shall be considered as a condition to differentiate the requirements for FR1 SCell activation enhancement with L3 report. - Like the legacy FR1 known SCell activation, SCell activation delay requirement with L3 report shall be differentiated according to measurement period below or above 2400ms, and decide if AGC refinement or T/F tracking is needed. - Proposal 2: - RAN4 also consider applying enhancements of L3 reporting during SCell activation in FR1 to the case when only one SSB is transmitted in ssb-PositionInBurst. - RAN4 further extend the requirement applicability of 8.3.17 and 8.3.18 to the scenarios in FR1 where only one SSB is considered. If only one SSB is considered, Tuncertainty_MAC, Tuncertainty_SP, Tuncertainty_RRC and TRRC_delay are counted as zero, and the 3ms MACE CE decoding delay for TCI state activation is removed, i.e. the overall delay Tactivation_time is 7ms + TL3,report+ THARQ + TFineTiming + 2ms.

The related section of TS 38.133 is cited as follows.

8.3.2 SCell Activation Delay Requirement for Deactivated SCell The requirements in this clause shall apply for the UE configured with at least one downlink SCell in EN-DC, or in standalone NR carrier aggregation or in NE-DC or in NR-DC and when one SCell is being activated. The delay within which the UE shall be able to activate the deactivated SCell depends upon the specified conditions. Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length , where: THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3] Tactivation_time is the SCell activation delay in millisecond. If the SCell is known and belongs to FR1, Tactivation_time is: - TFirstSSB+ 5ms, if the measurement period of the SCell being activated is equal to or smaller than 2400ms. - TFirstSSB_MAX + Trs + 5ms, if the measurement period of the SCell being activated is larger than 2400ms. If the SCell is unknown and belongs to FR1, and if one of the following conditions is met - ‘ssb-PositionInBurst’ indicates only one SSB is being actually transmitted, or - ‘ssb-PositionInBurst’ indicates multiple SSBs and TCI indication is provided in same MAC PDU with SCell activation, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, Tactivation_time is: - If UE supports shortMeasInterval-r18, then - TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + Trs, enhanced + 5ms, if the following conditions are met, - the SCell is contiguous to an active serving cell in the same band, and - its ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and - its SMTC offset is same as the one of contiguous FR1 active serving cell, and - its RTD with contiguous FR1 active serving cell is smaller than or equal to 260ns with respect to the to-be-activated SCell’s SSB numerology, and its reception power difference with contiguous FR1 active serving cell is smaller than or equal to 6dB; - TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + 2*Trs, enhanced + 5ms, otherwise. - Otherwise - TFirstSSB_MAX + TSMTC_MAX + Trs + 5ms, if the following conditions are met, - the SCell is contiguous to an active serving cell in the same band, and - its ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and - its SMTC offset is same as the one of contiguous FR1 active serving cell, and - its RTD with contiguous FR1 active serving cell is smaller than or equal to 260ns with respect to the to-be-activated SCell’s SSB numerology, and its reception power difference with contiguous FR1 active serving cell is smaller than or equal to 6dB; - TFirstSSB_MAX + TSMTC_MAX + 2*Trs + 5ms, otherwise. otherwise, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, Tactivation_time is: - If UE supports shortMeasInterval-r18, then - 6ms + TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + Trs, enhanced + TL1-RSRP, enhanced_measure + TL1-RSRP, report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), if semi-persistent CSI-RS is used for CSI reporting, - 3ms + TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + Trs, enhanced + TL1-RSRP, enhanced_measure + TL1-RSRP ,report + max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay), if periodic CSI-RS is used for CSI reporting. - Otherwise - 6ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP, measure + TL1-RSRP,report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), if semi-persistent CSI-RS is used for CSI reporting, - 3ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP, measure + TL1-RSRP,report + max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay), if periodic CSI-RS is used for CSI reporting. - However, when the following conditions are fulfilled, no activation requirement will be applied for this unknown SCell: - the SCell is contiguous to an active serving cell in the same band, and - A single SSB is used in the unknown SCell; or multiple SSBs are used in the SCell and TCI state indication for PDCCH is provided by the same MAC PDU used for SCell activation; and - its ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and - its SMTC offset is same as the one of contiguous FR1 active serving cell - its RTD with contiguous FR1 active serving cell is larger than 260ns with respect to the to-be-activated SCell’s SSB numerology, or its reception power difference with contiguous FR1 active serving cell is larger than 6dB;

For the yellow highlighted part above in FR1, generally L3 reporting is not needed. Hence re-using the same method from FR2 will not impact this part of requirements, since reporting valid L3 measurement results during SCell activation procedure is not done in legacy. However, for the blue highlighted part above, if UE supports l3-MeasUnknownSCellActivation-r18 and reports L3 measurement results during SCell activation procedure, the delay will be reduced to be even shorter than the scenario without L3 report in legacy.

Observation 1: For UE supporting l3-MeasUnknownSCellActivation-r18, and reports L3 measurement results during SCell activation procedure, the activation delay would be even shorter for multiple SSB case than the single SSB case.

Actually, UE with L3 measurement results would have valid AGC and DL timing information. There is room for reducing the latency also for the legacy case without L3 reporting. Note that for FR1 the maximal latency for L3 reporting is upper bounded by potential L1 reports, hence it would be always shorter than the delay in above yellow highlighted part.

Proposal 1: RAN4 also consider applying enhancements of L3 reporting during SCell activation in FR1 to the case when only one SSB is transmitted in ssb-PositionInBurst.

With L3 measurement result reported, the testability of the enhanced requirements would be ensured. Therefore, it is proposed to extend the applicability scenario of existing requirements in 8.3.17 and 8.3.18, while allowing some uncertainty terms to be zero if only one SSB is considered.

Proposal 2: RAN4 further extend the requirement applicability of 8.3.17 and 8.3.18 to the scenarios in FR1 where only one SSB is considered. If only one SSB is considered, Tuncertainty_MAC, Tuncertainty_SP, Tuncertainty_RRC and TRRC_delay are counted as zero, and the 3ms MACE CE decoding delay for TCI state activation is removed, i.e. the overall delay Tactivation_time is 7ms + TL3,report+ THARQ + TFineTiming + 2ms.

The change is also reflected in our CR [3].

Another issue in captured in agreed big CR [4].

8.3.17 SCell Activation Delay Requirement for Deactivated SCell with the L3 reporting during activation. The requirements in this clause shall apply for UE supporting l3-MeasUnknownSCellActivation-r18 and reporting valid L3 measurement results after receiving the SCell activation command for unknown SCell. The requirements in this clause shall apply for the UE configured with one downlink SCell in EN-DC, or in standalone NR carrier aggregation or in NE-DC or in NR-DC and when one SCell is being activated. Clause 8.3.2 is applied for UE who does not report L3 measurement results after receiving SCell activation command for unknown SCell. The delay within which the UE shall be able to activate the deactivated SCell depends upon the specified conditions. Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length , where: THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3] Tactivation_time is the SCell activation delay in millisecond. - Tactivation_time is: - 10ms + TL3,report+ THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), if semi-persistent CSI-RS is used for CSI reporting, - 7ms + TL3,report+ max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay), if periodic CSI-RS is used for CSI reporting, Editor’s Note: FFS How to define TL3,report and Tuncertainty_MAC for the case UE reports both L1-RSRP and L3-RSRP, and L1-RSRP is reported before L3-RSRP, and TCI activation command is received before L3-RSRP is reported. … Tuncertainty_MAC is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable) relative to - First valid L3-RSRP reporting for unknown case, when UE reports valid L3-RSRP - First valid L1-RSRP reporting for unknown case, when UE does not report L3-RSRP results Tuncertainty_RRC is the time period between reception of the RRC configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) relative to - First valid L3-RSRP reporting for unknown case, when UE reports valid L3-RSRP - First valid L1-RSRP reporting for unknown case, when UE does not report L3-RSRP results Tuncertainty_SP is the time period between reception of the activation command for semi-persistent CSI-RS resource set for CQI reporting relative to - First valid L3-RSRP reporting for unknown case, when UE reports valid L3-RSRP - First valid L1-RSRP reporting for unknown case, when UE does not report L3-RSRP results TRRC_delay is the RRC procedure delay as specified in TS38.331 [2].

The issue is further illustrated in Fig. 1.

Figure 1  Timeline of R18 unknown SCell activation delay reduction with L3 MR reporting

So far, companies understanding is that, even if UE reports L1 measurement results in PUCCH, and gNB sends TCI activation MAC CE, or SP CSI-RS activation MAC CE, or RRC reconfiguration of periodic CSI based on L1 reporting, UE still needs to report L3 MR in PUSCH. In this case, based on the timeline above, the related uncertainty shall be negative. In our view, maybe RAN4 may not need to define negative term. The simplest approach is set all the uncertainty related to 0.

Proposal 3: For R18 unknown SCell activation delay reduction with L3 MR reporting, Tuncertainty_MAC is zero if UE receives PDSCH carrying MAC CE based TCI activation command before L3 MR is sent in PUSCH.

Proposal 4: For R18 unknown SCell activation delay reduction with L3 MR reporting, Tuncertainty_RRC is zero if UE receives PDSCH carrying RRC configuration message for TCI of periodic CSI-RS for CQI reporting before L3 MR is sent in PUSCH.

Proposal 5: For R18 unknown SCell activation delay reduction with L3 MR reporting, Tuncertainty_SP is zero if UE receives PDSCH carrying MAC CE based activation command for semi-persistent CSI-RS resource set for CQI reporting before L3 MR is sent in PUSCH.

The change is also reflected in our CR [3].

- Discussion on maintenance of Part 2

Issue 1-5: PSCell activation delay and PSCell DRX: FFS: - Option 1: - UE PDCCH monitoring requirement in the newly activated PSCell is ambiguous. - UE shall start monitoring PDCCH on the activated PSCell immediately after the SCG activation delay. - Send LS to RAN2 clarifying PDCCH monitoring assumption with RAN2.

Regarding the UE behavior on PDCCH monitoring for SCG activation, as the highlighted part in SCG Activation/Deactivation procedure specified in TS38.321, ‘UE will apply normal SCG operation including PDCCH monitoring on the PSCell upon upper layers indicate that SCG is activated’.

Basically, in our understanding, ‘apply normal SCG operation’ means UE will follow the DRX operation to monitor the PDCCH discontinuously if DRX is configured. One thing to note is, from RAN4 perspective, the SCG activation contains two periods: one is the time point of receiving SCG activation command, and another is from the activation command reception to the first transmission occasion on PUCCH/PRACH (corresponding to the delay of activation procedure). Considering these two periods, there may be some ambiguity on whether normal SCG operation on the PSCell starts to be applied upon the command reception or upon the completion of SCG activation procedure. If the latter UE behavior is assumed, as some company raised in the last meeting, it may occur that the activation delay does not end during an on-duration. In this case, there may be the long delay of waiting for the next on-duration to start monitoring PDCCH. It is the reason why the solution proposed as the bold part in the box. For this solution, it seems to disable DRX operation once the SCG activation is completed. In this way, it is not clear that when UE would re-able DRX functionality and monitor the PDCCH discontinuously.

However, before discussing how to resolve this issue, to address the above ambiguity on when to starting monitoring PDCCH with/without DRX operation in RAN2 spec, we suggest to firstly send LS to RAN2 clarifying UE behavior on PDCCH monitoring upon the activation command reception and during SCG activation process.

TS38.321 clause 5.29 5.29 Activation/Deactivation of SCG The network may activate and deactivate the configured SCG. The MAC entity shall for the configured SCG: 1> if upper layers indicate that SCG is activated: 2> if BFI_COUNTER >= beamFailureInstanceMaxCount for the PSCell or the timeAlignmentTimer associated with PTAG is not running: 3> indicate to upper layers that a Random Access Procedure (as specified in clause 5.1.1) is needed for SCG activation. 2> activate the SCG according to the timing defined in TS 38.133 [11]. 2> (re-)initialize any suspended configured uplink grants of configured grant Type 1 associated with this PSCell according to the stored configuration, if any, and to start in the symbol according to rules in clause 5.8.2.2; 2> apply normal SCG operation including: 3> SRS transmissions on the PSCell; 3> CSI reporting for the PSCell; 3> PDCCH monitoring on the PSCell; 3> PUCCH transmissions on the PSCell; 3> transmit on RACH on the PSCell; 3> initialize Bj for each logical channel to zero.

Proposal 6: For activation of SCG, RAN4 to send LS to RAN2 clarifying the UE behavior on PDCCH monitoring

- Option 1: UE starts to monitor PDCCH with/without DRX applying once upper layers indicate that SCG is activated

- Option 2: UE won’t monitor PDCCH during SCG activation procedure and starts to monitor PDCCH with/without DRX applying after SCG activation delay (i.e., Tactivation_time specified in TS38.133)

- Conclusions

Observation 1: For UE supporting l3-MeasUnknownSCellActivation-r18, and reports L3 measurement results during SCell activation procedure, the activation delay would be even shorter for multiple SSB case than the single SSB case.

Proposal 1: RAN4 also consider applying enhancements of L3 reporting during SCell activation in FR1 to the case when only one SSB is transmitted in ssb-PositionInBurst.

Proposal 2: RAN4 further extend the requirement applicability of 8.3.17 and 8.3.18 to the scenarios in FR1 where only one SSB is considered. If only one SSB is considered, Tuncertainty_MAC, Tuncertainty_SP, Tuncertainty_RRC and TRRC_delay are counted as zero, and the 3ms MACE CE decoding delay for TCI state activation is removed, i.e. the overall delay Tactivation_time is 7ms + TL3,report+ THARQ + TFineTiming + 2ms.

Proposal 3: For R18 unknown SCell activation delay reduction with L3 MR reporting, Tuncertainty_MAC is zero if UE receives PDSCH carrying MAC CE based TCI activation command before L3 MR is sent in PUSCH.

Proposal 4: For R18 unknown SCell activation delay reduction with L3 MR reporting, Tuncertainty_RRC is zero if UE receives PDSCH carrying RRC configuration message for TCI of periodic CSI-RS for CQI reporting before L3 MR is sent in PUSCH.

Proposal 5: For R18 unknown SCell activation delay reduction with L3 MR reporting, Tuncertainty_SP is zero if UE receives PDSCH carrying MAC CE based activation command for semi-persistent CSI-RS resource set for CQI reporting before L3 MR is sent in PUSCH.

Proposal 6: For activation of SCG, RAN4 to send LS to RAN2 clarifying the UE behavior on PDCCH monitoring

- Option 1: UE starts to monitor PDCCH with/without DRX applying once upper layers indicate that SCG is activated

- Option 2: UE won’t monitor PDCCH during SCG activation procedure and starts to monitor PDCCH with/without DRX applying after SCG activation delay (i.e., Tactivation_time specified in TS38.133)

- Reference

[1] R4-2410259, WF for [111][205] NR_RRM_enh3_part1, Apple

[2] R4-2410343, WF on improvement of SCG activation requirements for FR1-FR1 NR-DC, OPPO

[3] R4-2412516, CR on R18 FR2 SCell activation delay reduction, vivo

[4] R4-2410397, Big CR to TS 38.133 on core requirement maintenance for Even Further RRM enhancement for NR and MR-DC, Apple