3GPP TSG RAN WG4 Meeting #112	R4-2412600

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	5.29.1

Source: 	vivo

Title: 	Discussion on maintenance of R18 NES RRM core requirements

Document for:	Discussion

Introduction

In RAN4 #111 meeting, some issues left in the core part were discussed. Accordingly, the latest progress was captured in WF [1]. In this paper, we would like to provide our thinking on the remain issues on inter-band SSB-less SCell and NES-based CHO.

Discussion on maintenance of inter-band SSB-less SCell

In last meeting, the following remaining issue is left.

Issue 1-1-1: Power difference conditions Agreement: Clarify that EPRE difference is based on the EPRE normalized by SCSs of SSB of reference Cell and A-TRS/P-TRS of SSB-less Cell. Only change [9] dB to [12] dB in this meeting. Further discuss whether and how to support larger EPRE difference in next meeting: - Option 1: Clarify that EPRE difference is smaller or equal to [12] dB + ¦20*log (f1/f2)¦ - Margin, where f1 and f2 is the center frequency of reference Cell and SSB-less Cell and the value of Margin is FFS. - Option 2: Further increase [12] dB and the value is FFS - Option 3: Update the requirements with one more P-TRS sample for larger EPRE difference - Other options are no precluded.

Generally, we do see companies have common understanding on what kind of compensation UE shall do. Moreover, from testability perspective, the EPRE difference at the antenna connector shall be considered, and how UE deal with all other impact, including pathloss difference, would be up to UE implementation without RRM requirements, since they are generally not testable.

Therefore, within the 3 options provided in [1], we prefer option 2 or 3. Given 2 TRS samples are available, we are OK to consider larger value for the EPRE difference. For option 1, our concern would be the impact to test cases. The test shall not only consider pathloss difference that bring out by frequency difference. ΔPL is also caused by uncertainty in UE antenna gain difference. For inter-band case, it is highly possible that UE uses different antennas for different bands. Therefore, we prefer a more general approach.

Proposal 1: To address the EPRE difference issue, RAN4 further discuss the value and whether one more P-TRS is needed.

Discussion on maintenance of NES-based CHO

Issue 1-2-1: Whether to differentiate requirements wrt Cell turning off. Agreement: - RAN4 will not define Cell switch off specific requirements/behaviour when receiving NES indication for CHO unless there is new agreement from other working group which can explicitly indicate “cell switch off”. Issue 1-2-2: When CHO condition is not met anymore Discuss in the CR directly.

Fig.1 the timeline of delay requirement on NES-based conditional handover

For NES-based conditional handover, regarding the case that if the CHO condition has been met but is not any more fulfilled when receiving the DCI 2-X command, companies failed to reach consensus on how to implement CR in RAN4#110bis meeting. To make it clear, we will provide our thinking on this issue with the help of the above Fig.1.

RAN2 agreement in RAN2#125bis meeting - Understanding is that “The fulfilment of the event includes the case that the NES mode indication has been received [Cond 1] and the entry condition is now fulfilled [Cond 2]”. No need to update the specifications - No further clarification to continue measurement and CHO evaluation is necessary for the case UE now receives NES mode-enabled indication while NES CHO event is not yet fulfilled, and for the case NES CHO event is now fulfilled and UE has received a NES mode-enabled indication.

After receiving RRC message implying NES-based handover, UE performs measurement periodically to check if entry condition is fulfilled. The checking points are shown as measurement Point 1/2/3/4 in Fig.1. Due to the existence of sufficient good channel condition at Point 5 and Point 6, UE realizes that entry condition is met at measurement Point 1 and Point 3, respectively. Based on the latest agreement reached in RAN2 as above, NES-based CHO will be triggered only when both Cond1 and Cond2 are fulfilled. That is to say, the Point 5 which the condition exists is not the actual ending point of TEvent_DU referred in spec description. After Point 5, the channel condition gets worse until Point 6 comes. Since UE realizes the entry condition is met at measurement Point 3 and subsequently receives DCI 2-9, the Point 6 becomes to the actual time point mentioned in definition of TEvent_DU.

To clearly describe every timing point which is referred to in the spec, it can be started with two constraints as follows:

- Constraint 1: The time gap between the time point that a condition exists (i.e., Point 6) and the time point hat UE actually realizes the entry condition is met (i.e., measurement Point 3) shall be less than Tidentify_intra_with_index or Tidentify_intra_without_index

- Constraint 2: The time gap between the time point that UE actually realizes the entry condition is met (i.e., measurement Point 3) and the time point that UE successfully decodes DCI 2-9 shall be less than Tidentify_intra_with_index or Tidentify_intra_without_index

For the constraint 2, it has already been reflected in the following highlighted description. And the ambiguity in current spec mainly lies in the ending point of TEvent_DU. Based on above two constraints, to consider the worst case, the time gap between the time point that a condition exists (i.e., Point 6) and the time point UE successfully decodes DCI 2-9 shall be less than 2* Tidentify_intra_with_index or 2*Tidentify_intra_without_index. Correspondingly, for reference, we provide one modification way to adapt the actual ending point of TEvent_DU to Point 6.

TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until - a condition exists at the measurement reference point which will trigger the conditional handover, or - a condition exists within 2*Tidentify_intra_with_index or 2*Tidentify_intra_without_index before UE successfully decodes DCI 2-9, at the measurement reference point which will trigger the NES-based conditional handover

##### 6.1.4.2.1 Handover delay Procedure delays for all procedures that can command a conditional handover are specified in TS 38.331 [2]. When the UE receives a RRC message implying conditional handover the UE shall be ready to start the transmission of the new uplink PRACH channel within DCHO seconds from the end of the last TTI containing the RRC command. DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution Where: TRRC is the RRC procedure delay defined in clause 12 in TS 38.331 [2]. TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until - a condition exists at the measurement reference point which will trigger the conditional handover, or - a condition exists at the measurement reference point which will trigger the NES-based conditional handover Tmeasure is the measurements time stated in clause 6.1.4.2.2. TCHO_execution is the conditional execution preparation time in clause 6.1.4.2.3. Tinterrupt is the interruption time stated in clause 6.1.4.2.4. The NES-based conditional handover delay requirements shall apply if UE receives a RRC message implying conditional handover before receiving the NES indication in DCI 2-9 command. When UE receives a RRC message implying NES-based conditional handover but no NES indication in DCI 2-9 command, or when UE only receives a NES indication in DCI 2-9 command before receiving a RRC message implying NES-based conditional handover, no NES-based conditional handover requirement is applied. For NES-based conditional handover, UE shall start RRM measurement before receiving the NES indication in DCI 2-9 command. The NES indication is specified in clause [5.5.4] in TS 38.331[2]. 6.1.4.2.2 Measurement time The measurement time delay is defined from the end of TEvent_DU until UE executes a handover to a target cell and interruption time starts. For conditional intra-frequency handover, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify intra with index or Tidentify_intra_without_index defined in clause 9.2.5.1 or clause 9.2.6.2. For conditional inter-frequency handover, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify_inter_with_index or Tidentify_inter_without_index defined in clause 9.3.4 or clause 9.3.9. For NES-based conditional intra-frequency handover: - If UE successfully decodes DCI 2-9 command earlier than the time at the end of TEvent_DU + Tidentify_intra_with_index or TEvent_DU + Tidentify_intra_without_index, then the measurement time delay equal to Tidentify_intra_with_index or Tidentify_intra_without_index - If UE successfully decodes DCI 2-9 command later than the time at the end of TEvent_DU + Tidentify_intra_with_index or TEvent_DU + Tidentify_intra_without_index, then the measurement time delay equals to the time from the end of Tevent_DU until UE successfully decodes DCI 2-9 command. For NES-based conditional inter-frequency handover: - If UE successfully decodes DCI 2-9 command earlier than the time at the end of TEvent_DU + Tidentify_inter_with_index or TEvent_DU + Tidentify_inter_without_index, then the measurement time delay equal to Tidentify_inter_with_index or Tidentify_inter_without_index - If UE successfully decodes DCI 2-9 command later than the time at the end of TEvent_DU + Tidentify_inter_with_index or TEvent_DU + Tidentify_inter_without_index, then the measurement time delay equals to the time from the end of Tevent_DU until UE successfully decodes DCI 2-9 command. Editor Notes: The measurement time delay for NES-based conditional handover is FFS. Editor Notes: TEvent_DU for NES-based conditional handover is FFS. When TTT or L3 filtering is used an additional delay can be expected. A cell is detectable only if at least one SSB measured from the cell being configured remains detectable during the time period Tidentify_intra_without_index or Tidentify_intra_with_index for intra-frequency handover or Tidentify_inter_without_index for inter-frequency handover. If a cell which has been detectable at least for the time period Tidentify_intra_without_index or Tidentify_intra_with_index for intra-frequency handover or Tidentify_inter_without_index for inter-frequency handover becomes undetectable for a period and then the cell becomes detectable again and triggers a handover, the measurement time delay shall be less than TSSB_measurement_period_intra or TSSB_measurement_period_inter provided the timing to that cell has not changed more than ± 3200/2µ Tc while the measurement gap has not been available and the L3 filter has not been used, where µ is the SCS configuration as defined in clause 4.2 of TS 38.211 [3]. When L3 filtering is used, an additional delay can be expected. ##### 6.1.4.2.3 Preparation time TCHO_execution is the UE execution preparation time for conditional handover, and starts after UE realizes the condition of CHO is met and identity of the target cell is determined. TCHO_execution can be up to 10ms.

Moreover, based on the highlighted description, the measurement time delay (i.e., Tmeasure) includes two parts: one is the actual measure delay from UE, and another is the time duration from UE finished measurement to UE successfully decodes DCI 2-9. Correspondingly, for NES-based CHO, the starting point of TCHO_execution specified in highlighted part shall correspondingly adapt as the time point that UE successfully decodes DCI 2-9 in some cases. For refence, we provide the example as follows:

##### 6.1.4.2.3 Preparation time TCHO_execution is the UE execution preparation time for conditional handover. For conditional handover, and it starts after UE realizes the condition of CHO is met and identity of the target cell is determined. For NES-based conditional handover, - It starts after UE realizes the condition of CHO is met and identity of the target cell is determined provided that UE has already successfully decoded DCI 2-9. - Otherwise, it starts after UE successfully decodes DCI 2-9. And TCHO_execution can be up to 10ms.

Proposal 2: For the delay requirement on NES-based CHO, RAN4 to revise the definition of TEvent_DU as follows:

TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until

- a condition exists within 2*Tidentify_intra_with_index or 2*Tidentify_intra_without_index before UE successfully decodes DCI 2-9, at the measurement reference point which will trigger the NES-based conditional handover

Proposal 3: RAN4 to revise the starting point of TCHO_execution as follows:

TCHO_execution is the UE execution preparation time for conditional handover.

For NES-based conditional handover,

- It starts after UE realizes the condition of CHO is met and identity of the target cell is determined provided that UE has already successfully decoded DCI 2-9.

- Otherwise, it starts after UE successfully decodes DCI 2-9.

Conclusions

Proposal 1: To address the EPRE difference issue, RAN4 further discuss the value and whether one more P-TRS is needed.

Proposal 2: For the delay requirement on NES-based CHO, RAN4 to revise the definition of TEvent_DU as follows:

TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until

- a condition exists within 2*Tidentify_intra_with_index or 2*Tidentify_intra_without_index before UE successfully decodes DCI 2-9, at the measurement reference point which will trigger the NES-based conditional handover

Proposal 3: RAN4 to revise the starting point of TCHO_execution as follows:

TCHO_execution is the UE execution preparation time for conditional handover.

For NES-based conditional handover,

- It starts after UE realizes the condition of CHO is met and identity of the target cell is determined provided that UE has already successfully decoded DCI 2-9.

- Otherwise, it starts after UE successfully decodes DCI 2-9.

- References

- R4-2410269, WF on RRM requirements for NR network energy saving, Huawei