3GPP TSG RAN WG4 Meeting #112	R4-2412601

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:			8.25.4

Source:	vivo

Title:	Consideration on RRM impacts for R19 NR NTN Phase 3

Document for:	Discussion

- Introduction

Based on the discussion in RAN plenary RAN#104 meeting, the revised WID of NTN for NR phase 3 is approved [1]. For Rel-19 NTN work item, as per the objectives listed in WID, so far only the enhancement of supporting Redcap NTN has clear RRM scope, which is specified as follows.

- Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex and half duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, specify enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB [RAN1] - Enhancements are targeted for the following HD collisions cases (cases 3 and 4): - Semi-statically configured DL reception collides with semi-statically configured UL transmission - Dynamically scheduled DL reception collides with dynamic scheduled UL transmission - Note: enhancements for other HD collision cases are not targeted. Enhancements for HD collision cases 3 and 4 that may improve other HD collision cases are not precluded. - Note: potential work in RAN2, if any, starts after RAN1 has decided on the solution. - Notes for this objective: - GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

To define RRM requirement on the new feature of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands, both the current Recap and NR NTN requirement shall be considered. Thus, we provide our preliminary analysis on how to handle the combination of these two features in this contribution.

- Discussion

In Rel-17, it was supported the Redcap UE with reduced maximum UE bandwidth and the minimum number of Rx branches, which correspond to 1Rx and 2Rx respectively. Accordingly, the RRM requirements are defined, which including mobility, signal characteristics, Measurement Procedure and Measurement Performance requirements, etc. Based on the fundamental work in Rel-17, the further evolution on Redcap, i.e., eRedcap, was introduced.

For NR NTN, it was also introduced in Rel-17. The corresponding RRM requirements for Satellite Access were defined. And in Rel-18, further enhancements mainly focus on several parts, including support of above 10GHz bands, NW verified UE location and optimization on mobility related.

In Rel-19, it aims to enable Redcap with NR NTN operating in FR1-NTN bands, which means we need to consider the combination of these two features. In general, when defining the RRM requirements on Redcap NTN, the significant characteristics from both Redcap and NTN need be considered as much as possible. For further analysis, we have summarized the specific characteristics of Redcap and NTN features compared with basic TN requirements as follows

Table.1 specific characteristic of Redcap and NTN features

|  |  | Redcap | NTN | Note |
| --- | --- | --- | --- | --- |
| Mobility requirement | Cell reselection | eDRX - Assumptions on measurements on eDRX with PTW - Idle state serving cell eDRX requirements - Idle state cell reselection requirements for eDRX - eDRX requirements for inactive state | - eDRX is not supported | To discuss if eDRX is supported in Redcap NTN |
|  |  | RRM relaxation - Application scenarios - 12 applicable scenarios - Transition period - Relaxation requirements | RRM relaxation - The requirements in clause 4.2.2.9 apply provided that target cell’s satellite is GEO. Time-based and location-based cell reselction | To discuss if RRM relaxation in redcap is supported in Redcap NTN |
|  | Handover | - Scenario: Support CD-SSB and NCD-SSB - 1 RX impact: Relax the Tsearch for FR1 RedCap HO delay requirement when SINR=-2dB and 1Rx: | - Support NR RACH-less HO in Rel-18 - Support Conditional handover in Rel-17(including time-based and location-based CHO for fixed cell) - Enhancements on CHO in Rel-18 - CHO without L3 measurement criteria - Support earth moving cell scenario - Satellite switch without re-sync | To discuss if NCD-SSB is supported in Redcap NTN |
|  | RRC-establishment | - 1 Rx impact: Relax Tidentify for reestablishment delay requirement by 1 sample for know/unknown intra/inter frequency target cell for FR1 - Redcap specific initial BWP impact: When this is configured for random access, T_(UE_re−establish_delay) in clause 6.2.1 is extended by 6ms | - Add factor K_satellite to Tidentify_inter - Support VSAT scenario in Rel-18(reuse FR1 NTN requirements) | For Redcap in Rel-19, only FR1 deployment is considered. Thus, the NTN requirements defined for VSAT in FR2 will not be discussed. |
|  | RRC Connection release with redirection: | - 1 RX impact: 1 sample relax for Tidentify-NR with 1Rx for FR1 - Redcap specific initial BWP impact: When the Redcap specific initial BWP is configured for random access, Tconnection_release_redirect_NR is extended by 6ms | - Add factor K_satellite to Tidentify_NR |  |
|  | Random access | - 1 RX impact: Offset to apply for thresholds for 1 Rx UE – rsrp-ThresholdSSB; msgA-RSRP-ThresholdSSB; msgA-RSRP-Threshold | - No enhancement compared with TN |  |
| Signalling characteristics | RLM/BFD | - RAN4 to not introduce requirements for 60 kHz SCS in FR1 for RedCap RLM requirements - 1 RX impact: - The lower bound RLM Qout evaluation period for FR1 is extended by factor 2 and the lower bound RLM Qin evaluation period for FR1 is not extended - Enhancements to hypothetical PDCCH parameters | - No enhancement compared with TN |  |
| Measurement procedures | CSSF | - one searcher shall be shared with all frequency layers which are measured without gap | - Not supported | To discuss if CSSF is used in Redcap NTN |
|  | Measurement gap | - Support gap pattern 0-25 | - Support gap pattern 0-11,24,25 for FR1 - Support gap pattern 12-25 for FR1 - Support concurrent gaps | To discuss the supported gap pattern in Redcap NTN To discuss if concurrent gap is supported in Redcap NTN Whether per-FR gap is considered in Redcap NTN |
|  | Measurement period for SSB based L3 measurement | - 1 RX impact: Reuse Rel-15 requirements on measurement period (samples) for 1 Rx requirements for SSB based L3 measurement in FR1 | - Add factor Kmulti_SMTC to T SSB_measurement_period_ |  |
|  | Cell identification | - 1 RX impact: - PSS/SSS detection period: extend requirements by 2 more samples - Time index detection requirement: The time index delay in FR1 is extended by 3 samples/SMTC, i.e. total 6 samples; Extend lower bound from 120ms as: max(160ms, ceil( 4 x Kp ) x SMTC period)x CSSFintra | - Add factor Kmulti_SMTC to TPSS/SSS_sync and TSSB_time_index |  |
|  | CGI reading | - 1 RX impact: SIB1 acquisition period is extended from 6 to 12 samples | - Not supported |  |
| Timing requirements | UE transmit timing requirements | - Scenario: Support CD-SSB and NCD-SSB | - NTN specific requirement with considering the error caused by the calculation of the distance between gNB and UE - Support VSAT scenario including - Fixed VSAT is served by GSO - Fixed VSAT is served by NGSO - mobile VSAT is served by GSO | To adopt NTN requirements as the baseline when considering redcap NTN scenario |
| Measurement performance requirements | Measurement period for SSB based L3 measurement | - 1 RX impact: Reuse Rel-15 requirements on measurement period (samples) for 1 Rx requirements for SSB based L3 measurement in FR1 | - To be confirmed if TN requirements can be reused |  |
|  | L1-RSRP measurements | - 1 RX impact: relaxed relative and absolute accuracy SSB based and CSI-RS based L1-RSRP for 1 Rx Redcap UE for FR1 | - To be confirmed if TN requirements can be reused |  |

In the above table, we highlight the noticeable difference of redcap with red and NTN with blue. In details, for Reacap, the differences compared with non-Redcap are mainly due to the introduction of 1Rx, the new RRM relaxation criteria introduced in Rel-17, eDRX related and support of NCD-SSB. And for NR NTN, compared with TN scenario, the main difference is seen in mobility enhancement, gap related and specific timing requirements, etc. Considering these significantly different features introduced from Redcap and NTN topic, it may be easier to firstly determine which feature to use as the basic framework and further discuss how to adapt the applicable characteristics from another feature when specifies the requirements on Redcap NTN. In our view, specifically in the context of NTN deployment, it is better to use the current framework for NTN requirement. specifically, regarding the highlighted mobility enhancements introduced in Rel-17 and Rel-18 NTN, they are not supported in Redcap so far. In our view, considering mobility management is one of the most important features in NTN, we prefer to support this part as the essential feature of Redcap NTN.

Further, based on this framework, RAN4 can further discuss the following potential adaptations:

- Introduce corresponding RRM requirements for 1 Rx and current requirements for Redcap UE with 1Rx can be the reference

- Determine if redcap specific features introduced in idle/inactive mode can be supported in Redcap NTN, which including eDRX, RRM relaxation, etc.

- Determine if NCD-SSB is supported in Redcap NTN

- Determine if CSSF and concurrent gap is supported in Redcap NTN

- Determine mobility related, e.g., RACH-less (C)HO, time-based/location cell reselection/CHO, Unchanged PCI are supported

Proposal 1: RAN4 to use the current NTN requirement as the framework of Redcap NTN and further discuss the necessary adaptations, which potentially including:

- Introduce corresponding RRM requirements for 1 Rx and current requirements for Redcap UE with 1Rx can be the reference

- Determine if redcap specific features introduced in idle/inactive mode can be supported in Redcap NTN, which including eDRX, RRM relaxatoin, etc.

- Determine if NCD-SSB is supported in Redcap NTN

- Determine if CSSF and concurrent gap is supported in Redcap NTN

- Determine mobility related, e.g., RACH-less (C)HO, time-based/location cell reselection/CHO, Unchanged PCI are supported

- Summary

In this contribution, we provided our views on general aspects for AI/ML. Based on above analysis, following proposals are present.

Proposal 1: RAN4 to use the current NTN requirement as the framework of Redcap NTN and further discuss the necessary adaptations, which potentially including:

- Introduce corresponding RRM requirements for 1 Rx and current requirements for Redcap UE with 1Rx can be the reference

- Determine if redcap specific features introduced in idle/inactive mode can be supported in Redcap NTN, which including eDRX, RRM relaxatoin, etc.

- Determine if NCD-SSB is supported in Redcap NTN

- Determine if CSSF and concurrent gap is supported in Redcap NTN

- Determine mobility related, e.g., RACH-less (C)HO, time-based/location cell reselection/CHO, Unchanged PCI are supported

- References

- RP-241667, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT