3GPP TSG RAN WG4 Meeting #112	R4-2412602

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:			8.26.3

Source:	vivo

Title:	Consideration on RRM impacts for R19 IoT NTN Phase 3

Document for:	Discussion

- Introduction

Based on the discussion in RAN plenary RAN#104 meeting, the revised WID of NTN for Internet of things phase 3 is approved [1]. For Rel-19 IoT NTN work item, as per the updated objectives listed in WID, so far only the enhancement to reduce the necessary uplink and downlink signaling to complete an EDT transaction has clear RRM scope, which is specified as follows.

- Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete - Study and specify RRM requirement, if identified [RAN4]

On the initial discussion from RAN4 aspect, it was triggered by the LS in RAN2#126 meeting [2]. In this contribution, we provide our analysis on the potential RRM impact raised in LS.

- Discussion

The content of RAN2 LS is provided as follows.

In RAN2#126 meeting, the following agreements related to Msg3 transmission for uplink capacity enhancement for R19 IoT NTN have been achieved: Agreements: => RAN2 focusses the study on contention-based Msg3 transmission to complete an EDT-like transaction (FFS on the details of Msg3. FFS on the procedural steps, e.g. how much we reuse of EDT and PUR procedures. FFS on allocation of resources). => If an IoT NTN UE in IDLE state is to use the new R19 contention-based procedure, the UE needs to verify/update the uplink synchronization (e.g. get GNSS fix, acquire TA) just before sending msg3.. Based on the above agreements, RAN2 kindly asks RAN4 and RAN1: Q1: Whether an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during legacy random access) can satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2? Q2: If the answer for Q1 is no, from RAN4 and RAN1 perspective, how the required timing accuracy for Msg3 transmission can be satisfied in this case?

Before analyse the questions on TA related as above, we provide initial analysis for the current design for the relationship between uplink and downlink timing. It is specified that uplink frame number  for transmission from the UE shall start TTA=NTA+NTAoffset+NTA,common+NTA,adjUETc before the start of the corresponding downlink frame at the UE. Among them, each part in this equation represents:

- For NTA, the value NTA of timing advance is indicated in Msg2. Specifically, NTA for PRACH is defined as 0 in the spec.

- For NTAoffset, A UE can be provided a value NTAoffset of a timing advance offset for a serving cell.

- For NTA,adjcommon, it is derived from the higher-layer parameters contained assistance information for the serving cell. It is the network-controlled common timing advanced value between reference point and satellite on feeder link;

- For NTA,adjUE, UE determines it by using the serving satellite position and its own position on service link. For the serving satellite position, it is calculated by UE based on higher-layer ephemeris parameters for a serving satellite.

Based on the description in LS on uplink capacity enhancement, the current RAN2 discussion focuses on the method to minimize the necessary UL and DL signalling to complete an EDT transaction or PUR procedure. The main idea is to skip the steps of preamble transmission (Msg1) and RAR reception (Msg2) and directly to send Msg3 on the dedicated resources.

As mentioned in LS above, if considering the enhancement on no Msg1/Msg2 are involved, one of the impacts is that no NTA will be provided. Regarding Q1, it is to ask that, without any indication of NTA value from NW side, whether the timing advance can be totally compensated by UE itself based on the calculation of the transmission delay between the UE location and the serving satellite location. Further, whether the timing accuracy can be satisfied by using this method. Specifically, regarding ‘whether [required timing accuracy] can be satisfied’ can be interpreted separately from two aspects:

Aspect 1: ‘the required timing accuracy’ refers to the requirement on UE transmit timing for NB-IoT for Satellite Access specified in section 7.20A in TS36.133. From UE perspective, the main point on whether the timing requirement can be satisfied is to check the error due to the calculation of the transmission delay between the UE location and the serving satellite location. For this part, it has already been considered in the current transmit timing requirement, which include the UE position error and satellite position estimation error, respectively. Specifically, for the satellite position estimation error, it highly depends on the frequency of update of the ephemeris information. For this issue, it has been discussed in Rel-18 NR FR2-NTN. After the studies in several meetings, it has been found that no further limitation on the how frequent the ephemeris information is sent. The timing requirement can be satisfied even with the maximum interval updating the broadcast ephemeris data (10.24s). Thus, from UE perspective, Q1 can be confirmed if ‘the required timing accuracy’ refers to the requirement on UE transmit timing for NB-IoT for Satellite Access specified in section 7.20A in TS36.133.

Aspect 2: ‘whether [the required timing accuracy] is satisfied’ is observed from NW perspective. For the legacy Msg3 transmission after Msg1 and Msg2, the NTA is considered into TA pre-compensated by UE when it is indicated as a non-zero value. Whereas in the case of Msg3 transmission without Msg1 and Msg2, NTA is always assumed to be 0. From NW perspective, the reception timing accuracy for Msg3 transmission may be different for the cases with and without Msg1/Msg2. For this timing difference, it is expected to be handled by network implementation.

To align with the understanding between RAN2 and RAN4 on which ‘the required timing accuracy’ refers to, we suggest to involve the consideration on both aspect 1 and aspect 2 in the Reply LS. For reference, we provide the draft version of reply LS in [3].

Proposal 1: From UE perspective, RAN4 to confirm that an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during legacy random access) can satisfy the requirement on UE transmit timing for NB-IoT for Satellite Access specified in section 7.20A in TS36.133 for Msg3 transmission without Msg1/Msg2

Proposal 2: From network perspective, the reception timing accuracy for Msg3 transmission may be different for the cases with and without Msg1/Msg2. It is expected that this timing difference should be handled by network implementation

- Summary

In this contribution, we provided our views on general aspects for Rel-19 IoT NTN. Based on above analysis, following proposal is present.

Proposal 1: From UE perspective, RAN4 to confirm that an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during legacy random access) can satisfy the requirement on UE transmit timing for NB-IoT for Satellite Access specified in section 7.20A in TS36.133 for Msg3 transmission without Msg1/Msg2

Proposal 2: From network perspective, the reception timing accuracy for Msg3 transmission may be different for the cases with and without Msg1/Msg2. It is expected that this timing difference should be handled by network implementation

- References

- RP-241624, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek Inc

- R2-2405763, LS on UL synchronization for contention based Msg3 transmission without Msg1/Msg2, ZTE

- R4-2412603, Reply LS on UL synchronization for contention based Msg3 transmission without Msg1/Msg2, vivo