3GPP TSG RAN WG4 Meeting #112	R4-2412604

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:			8.15.3

Source:	vivo

Title:	Initial discussion on fast SCell activation for UE supporting Rel-18 EMR

Document for:	Discussion

- Introduction

The new WID of RRM phase 5 was approved in the RAN plenary RAN#103 meeting [1]. The objectives are shown as below. In RAN4#110bis meeting, RAN4 started the discussion on the FR2-1 SSB based L3 measurement delay reduction for connected mode. And for the objective of fast SCell activation, as per the working arrangement, the corresponding discussion will be triggered in this meeting.

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - For UE not in multiple-Rx simultaneous reception mode: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

In this contribution, we provide our preliminary thinking on the detailed scope and technical issues for the fast SCell activation for UE supporting Rel-18 EMR.

- Discussion

In Rel-16, to enable faster configuration of CA/DC during connection setup, the feature on early measurement reporting has been discussed based on the LTE early measurement procedure in Rel-15. The UE shall perform idle mode measurement on the CA/DC candidate frequencies/cells indicated by higher layers. Further, the UE shall be able to report idle mode CA/DC measurements when the measurement reporting is requested by the network. One noticeable issue for EMR is, UE can choose whether to continue measurements after T331 expiration, which may cause the reported measurement results may be obtained from long time ago and NW has no idea about this. For this point, the discussion on enhancement of EMR was raised in Rel-18.

In Rel-18, RAN4 studied the possible enhancements for EMR from several aspects and finally introduced improvements on EMR, which mainly include:

- UE supports to report valid measurement results of cell reselection for fast CA/DC setup

- Measurement validation based on EMR/non EMR measurement during connection setup/resume

In Rel-19, RAN4 aims to utilize the measurement validation of Rel-18 EMR to further enhance SCell activation procedure, specifically reduce the SCell activation delay. For the highlighted description of WID, there are several parts to be made clear. Next, we will provide our analysis one by one.

- Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting

Issue 1: Whether valid cell reselection reporting can be used for fast SCell activation in Rel-19

As we mentioned above, for Rel-18 EMR capable UE, measurement validation procedure is supported for both EMR and cell reselection measurement. UE is also allowed to report valid cell reselection measurement results. Therefore, RAN4 needs to clarify if valid cell reselection reporting also can be used for reducing activation delay in Rel-19.

Proposal 1: RAN4 to clarify if valid cell reselection reporting can be used for fast SCell activation in Rel-19

Issue 2: Whether Rel-19 improvement aims for all the SCell activation scenarios specified in RAN4 spec

In RAN4 spec, the following SCell activation scenarios are supported and the corresponding RRM requirements are defined, which including SCell activation of deactivated SCell, direct SCell activation, fast SCell activation and SCell activation of multiple SCell. For reference, we summarized all the SCell activation scenarios defined in 38.133 and the corresponding known condition of target SCell as follows. It can be observed that, under different SCell activation scenarios and different FR deployment, the length of known window is not unified. Further, it may have impact on the discussion on definition of known condition for valid EMR reporting. Thus, for Rel-19 improvement on fast SCell activation, RAN4 needs to firstly discuss the supported scenario among the following cases.

SCell activation scenarios defined in TS38.133 - SCell Activation Delay Requirement for Deactivated SCell - Direct SCell Activation at SCell addition - Direct SCell Activation at Handover - Direct SCell Activation at RRCResume - SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink Scells - Direct SCell Activation of Multiple Downlink SCells at SCell addition - Direct SCell Activation of Multiple Downlink SCells at Handover - Direct SCell Activation of Multiple Downlink SCells at RRC Resume - SCell Activation Delay Requirement for Deactivated PUCCH SCell - SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple Downlink SCells - Fast SCell Activation Delay Requirement for Deactivated SCell

Table.1 typical SCell scenario and corresponding the definition of ‘known condition window’

|  | SCell activation scenario | FR1 | FR2 |
| --- | --- | --- | --- |
| Scenario 1 | SCell Activation Delay Requirement for Deactivated SCell | During the period equal to max (5*measCycleSCell, 5*DRX cycles) for FR1 before the reception of the SCell activation command: | During the period equal to 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable) |
| Scenario 2 | Direct SCell Activation at SCell addition | During the last 5 seconds before the reception of the direct SCell configuration command | same as scenario 1 in FR2 |
| Scenario 3 | Direct SCell Activation at Handover | same as scenario 2 in FR1 | same as scenario 1 in FR2 |
| Scenario 4 | Direct SCell Activation at RRCResume | same as scenario 2 in FR1 | same as scenario 1 in FR2 |
| Scenario 5 | SCell Activation Delay Requirement for Deactivated PUCCH SCell | same as scenario 1 in FR1 | same as scenario 1 in FR2 |
| Scenario 6 | Fast SCell Activation Delay Requirement for Deactivated SCell | same as scenario 1 in FR1 | same as scenario 1 in FR2 |

Proposal 2: RAN4 to discuss the applicable scenarios for fast SCell activation improvement, which potentially including:

- SCell Activation Delay Requirement for Deactivated SCell

- Direct SCell Activation at SCell addition

- Direct SCell Activation at Handover

- Direct SCell Activation at RRCResume

- SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink Scells

- Direct SCell Activation of Multiple Downlink SCells at SCell addition

- Direct SCell Activation of Multiple Downlink SCells at Handover

- Direct SCell Activation of Multiple Downlink SCells at RRC Resume

- SCell Activation Delay Requirement for Deactivated PUCCH SCell

- SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple Downlink SCells

- Fast SCell Activation Delay Requirement for Deactivated SCell

Issue 3: The possible solutions of reducing the SCell activation delay with leveraging valid EMR reporting

For Rel-16 and Rel-18 EMR, the reported measurement results somehow like the assistant information for NW and UE. It is up to UE implementation on how to utilize reported measurement result. There is no spec-level provision on how to utilize EMR reporting to reduce SCell activation delay. In Rel-19, based on the justification on this objective in WID, the general idea is to define new known SCell condition with consideration of valid EMR reporting. In more details, the valid EMR reporting which including the SCell to be activated can be the known condition to verify if the target cell can be activated in ‘known cell’ way. It is beneficial to reduce unnecessary long delay requirement for the SCell activation procedure.

To follow this logic, we will share our initial thinking on how to define known condition with valid result. In general, there are several candidate Options for further discussion and we list them as below. Before analysing each Option, we will put the possible cases when considering the relationship among validation window, the time point of receiving activation command and the ‘known condition’ checking window.

- Case 1: X and Y partially overlapped

- Case 2: X and Y are not overlapped

- Option 1	Use the existing known condition to verify if valid measurement results are obtained within ‘known cell’ checking window

The first Option is to reuse the current known condition and to check if validity window is within the known window. Only under this assumption, the SCell to-be-activated can be regarded as the known cell, which may require X is configured as the relatively small value and the activation command is received as soon as possible when UE enters connected mode.

Specifically, for the Case 1, when the validation window and ‘known’ checking window is partially overlapped, only some of reported measurement result can meet the current known condition. However, as there is no further timing information (e.g., time stamp or ranking) for the reported measurement result according to the current design, we cannot distinguish which are the known results.

And for the Case 2, there is no overlapped part between validation window and ‘known cell’ checking window. If we follow Option 1, then no valid results can be regarded as ‘known cell’.

Based on the above analysis, it can be observed that there may be some limitations on scenarios and conditions by using the existing known condition.

On the other hand, for the measurement validation in Rel-18 EMR, it mainly means that UE will verify if the measurements are performed before Msg1 transmission within the last [measIdleValidityDuration or measReselectionValidityDuration] seconds and the results satisfy measurement accuracy requirement at the measurement instance. The duration of validity timer is indicated by NW and the candidate values are {5s,10s,20s,50s,100s}, which are shown as below.

MeasurementValidityDuration-r18 ::= ENUMERATED { s5, s10, s20, s50, s100, spare3, spare2, spare1

As for the checking window of known SCell, based on the summary in Table 1, we can observe that the length may be the fixed value or related to the value of measCycleSCell/DRX cycle. The general range is from 800ms to 12.8s. Considering the misalignment on the length of checking window between known cell and valid EMR results, we propose the second Option, which is defined from ‘valid’ condition perspective.

- Option 2	Define new known condition of SCell activation with consideration of valid EMR reporting.

  - Option 2A	SCell is known when the UE has sent a valid measurement report of the SCell being activated during IDLE/INACTIVE state for fast CA/DC setup

  - Option 2B	SCell is known when valid measurement report is sent within [Z] seconds before SCell activation command reception

For Option 2A, it means if the results are pass the validity check or new check, the SCell to-be-activated can be regarded as the known cell. when validity timer is configured such as 50s,100s, the measurement results may be out of date for SCell activation. Therefore, if we go with this Option 2A, it may define applicability rule, that only workable when validity window is short.

Further, Option 2B is to define a new ‘known’ condition window that valid measurement report is sent within the specific time gap before SCell activation command reception.

Proposal 3: RAN4 to discuss how to define known condition with consideration of valid EMR reporting, the following Options can be considered:

  - Option 1	The SCell to-be-activated can be regarded as known cell when the UE has sent a valid measurement report of the SCell being activated during IDLE/INACTIVE state for fast CA/DC setup

  - Option 2	The SCell to-be-activated can be regarded as known cell when valid measurement report is sent within [Z] seconds before SCell activation command reception

- Summary

Proposal 1: RAN4 to clarify if valid cell reselection reporting can be used for fast SCell activation in Rel-19

Proposal 2: RAN4 to discuss the applicable scenarios for fast SCell activation improvement, which potentially including:

- SCell Activation Delay Requirement for Deactivated SCell

- Direct SCell Activation at SCell addition

- Direct SCell Activation at Handover

- Direct SCell Activation at RRCResume

- SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink Scells

- Direct SCell Activation of Multiple Downlink SCells at SCell addition

- Direct SCell Activation of Multiple Downlink SCells at Handover

- Direct SCell Activation of Multiple Downlink SCells at RRC Resume

- SCell Activation Delay Requirement for Deactivated PUCCH SCell

- SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple Downlink SCells

- Fast SCell Activation Delay Requirement for Deactivated SCell

Proposal 3: RAN4 to discuss how to define known condition with consideration of valid EMR reporting, the following Options can be considered:

  - Option 1	The SCell to-be-activated can be regarded as known cell when the UE has sent a valid measurement report of the SCell being activated during IDLE/INACTIVE state for fast CA/DC setup

  - Option 2	The SCell to-be-activated can be regarded as known cell when valid measurement report is sent within [Z] seconds before SCell activation command reception

- References

- RP-240830, New WID: WI resulting from discussion on RRM enhancements, vivo