3GPP TSG-RAN4 Meeting # 112	R4-2412607

# Maastricht, Netherlands, 19th – 23rd August, 2024

Title:	Discussion on NTN phase 3 RedCap UE RF

Source:	Samsung

Agenda item:	8.25.2.1

Document for:	Approval

Introduction

In this paper, we follow the moderator summary and WF from last meeting [1~3], and continue to discuss the left open issues.

Discussions

- Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, check whether any essential changes are needed for their support (i.e. focusing on HD collision rules) by end of Q2/2024 [RAN1] - Depending on feasibility assessment above, define the RF and RRM requirements [RAN4] - Notes for this objective: - GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

In the WF of last meeting, it is discussed and most companies support to adopt previous agreements for redcap for the NTN redcap 2Rx HD-FDD refsens as follows;

| FD-FDD 5MHz REFSENS | HD-FDD REFSENS Tightening |
| --- | --- |
| ≤ -100 dBm | 0 dB |
| > -100 dBm and ≤ -99 dBm | 0.5 dB |
| > -99 dBm | 0.8 dB |

And the WF[3] is referenced below:

Issue 1-2: HD-FDD refsens for 2 Rx Agreement: [0.5] dB tightening for n254 and n256, FFS for n255. n256 to be confirmed in RAN4#112. Note: RAN4#111 maintenance agreement on n256 refsens in R4-2408787 was not considered for this agreement.

As expressed in our last contribution, the n254, n255 and n256 RF design should be no significant difference with the overlapping or partially overlapping TN bands, as previously discussed for redcap, hence the REFSENS tightening is better to keep the same.

Proposal 1: Propose to have 0.5 dB tightening for n254 and n256, and 0dB for n255.

Conclusion

The following observations and proposals were submitted in this document for the meeting to disucss:

Proposal 1: Propose to have 0.5 dB tightening for n254 and n256, and 0dB for n255.

Reference

[1] R4-2405291, “Topic summary for [110bis][138] NR_NTN_Ph3_UERF”, Qualcomm

[2] R4-2406610, “WF on NR_NTN_Ph3_UERF”, Qualcomm

[3] R4-2410574, “WF on Rel-19 phase 3 NR-NTN UE RF requirements”, Qualcomm