3GPP TSG-RAN WG1 Meeting #112	R4-2412609

Maastricht, Netherlands, August 19th – 23rd 2024

Agenda Item:	8.17.4

Source:	Ericsson

Title:	CSI compression option 3 and option 4 considerations

Document for:	Discussion

# 1	Introduction

During RAN4#111, the topic of CSI compression was further discussed and it was agreed to use a set of temporary simulation assumptions in order to compare results of using a common model and training/test set in RAN4 and investigate the level of convergence. In addition to this, further discussion took place on the nature and objective of the options 3 and 4.

# 2	Further elaboration of option 3

## 2.1	Definition of option 3

The purpose of the 3GPP RAN1-2, and 4 specifications is to enable interoperability between equipment on either side of the Uu interface and predictable, manageable behaviors and performance to be maintained across different combinations of network vendor and UE equipment. To achieve this goal, RAN4 defines minimum requirements on RF, performance, RRM etc.

In order to define a fair, common, testable minimum performance requirement there are several important criteria that need to be fulfilled:

- The conditions for the performance requirements are clearly defined and publicly available.

- The performance requirements need to be implementable on test equipment in a common way

- The requirements and test conditions must be repeatable.

- The pass/fail criteria must be clear and well defined.

- The requirement, associated tests and test conditions should provide sufficient coverage so that similar behavior can be expected from the device under test when operating in real networks.

For the two-sided model, a question arises how to ensure that tests can be defined such that they can be implemented on different test equipment and be repeatable. A second question is how to ensure that the requirement and test setup is sufficiently related to real networks so that the tests have a meaningful relation to actual operation.

To achieve these criteria, according to option 3, a test decoder is standardized. By standardizing the test decoder, a known, repeatable test can be implemented on different test equipment.

## 2.2	On the scope and derivation of the test decoder

During RAN4#111, there was discussion on the scope of the test decoder and whether the validation should take into account all encoder/decoder pairs presented during the 3GPP discussion.

As mentioned in section 2.1, to be a useful requirement and test, passing the requirement and test should imply that similar performance in real networks can be achieved. Furthermore, for a 2-sided model, the test decoder will in effect capture a latent space that must be achieved by all decoders.

The test decoder will set the latent space that is contained within real encoders and decoders.

Test conditions need to be defined that relate to real operation (even if this is with channel models). Thus, in order to perform well during testing, the test decoder needs to be trained with data that models real world conditions. This will enable correct performance from the decoder during the test, and ensure that the latent space of the decoder, which sets the latent space of encoders is reasonable for real conditions.

The test decoder and the latent space it represents must be sufficiently rich to be useful for real world operation.

In order to derive a test decoder, multiple companies will bring encoder/decoder pairs to RAN4 for evaluation based on agreements on the model structure, complexity etc. A large proportion of the discussion is likely to focus on the specific details of the assumptions on model structure, complexity and some fine implementation issues such as floating/fixed point representation, quantization etc.

A large amount of discussion will be needed to fix assumptions on model structure, complexity and some detailed implementation aspects in a real WI.

In the end, one of the decoder/encoder pairs will be selected for standardization (or alternatively, a combined dataset could be used to train a new decoder, or decoder models could be merged).

During the meeting, there was some discussion as to whether the test decoder needs to be interoperable with all of the encoders presented in RAN4 during the evaluation phase. In our understanding, this is not the case. The encoders presented during the RAN4 evaluation are not final encoders intended for products but are reference encoders paired to the proposed test decoders. It is not necessary, and probably not even possible for an efficient decoder to capture the latent space and performance of all of the reference encoders.

The test decoder need not be interoperable with all reference encoders considered during the RAN4 evaluation.

If there is convergence in the observed performance of the proposed encoder/decoder pairs from each of the contributing companies when evaluated under the same conditions, then the difference between the encoder/decoder pairs is the representation of the latent space. Even though many encoders will not be compatible with the test decoder, this is only because of latent space inversion, not any fundamental performance difference.

If convergence is observed in performance of different encoder/decoder pairs under the same evaluation conditions then the difference between different companies encoder/Decoder models is in the latent space representation, but not the performance.

Once the test decoder is standardized and known, then it will be possible for all parties to develop real, product encoders based on knowledge of the test decoder and it’s latent space. Before final adoption of a test decoder, companies can verify that it is indeed the case that they can create encoders based on the decoder.

Once the standardized test decoder is available, encoder vendors can create real encoders that can interoperate with the standardized encoder.

## 2.3	On the need for a reference encoder

A reference encoder is an encoder that is not mandatory to implement, but that may be part of the standardization process. The term reference encoder was clarified at RAN4#111 as follows:

Reference encoder:

The reference encoder is used in RAN4 discussions at least for simulation alignment/requirement derivation, test decoder derivation and/or test decoder verification. It could be documented (in TR, WF, etc) or captured in the specifications as necessary.

The test decoder will have an associated reference encoder, as this will be needed to create and to evaluate the test decoder. Furthermore, the reference encoder will be used in order to define the minimum performance requirements, since the performance requirements should be set based on the combination of the reference encoder and test decoder.

In our view, it would most likely also be beneficial to capture the reference encoder in the TS. Capturing the reference encoder would facilitate testing of the implementation of the test decoder in test equipment. Furthermore, capturing a reference encoder would assist network vendors in creating real decoders that will be inter-operable with UE encoders that pass conformance requirements with the test decoder.

For option 3, capture both the test decoder and associated reference encoder in the TS.

## 2.4	On evaluation of the test decoder

It is important to differentiate between two different simulation evaluations for test decoders:

- Simulations used when deciding on the test encoder to standardize

- Once a test decoder has been selected for standardization, simulations to determine the performance requirement based on the standardized test decoder.

In this section of the document, we consider the simulations needed to decide on a test decoder to standardize. The following section considers the requirement itself.

Consider the simulation assumptions and criteria for selection of the test decoder in RAN4 separately to the simulation assumptions and criteria for setting the performance requirement.

As discussed in sections 2.1 and 2.2, the test decoder needs to be well designed so that it performs well in tests that represent real network scenarios. In order to select a good test decoder, it logically follows that evaluation of the test decoder should be based on a dataset that is wide enough to represent the spectrum of real world conditions under which the encoder is expected to operate. For this reason, the evaluation of test decoders should be done using system level data.

Base the simulations for evaluation of which test decoder to select on system data in order that the selected test encoder can be shown to have reasonable performance across the range of operating conditions.

## 2.5	On the requirement test set and metric

Once a test decoder has been standardized, it is necessary to set criteria for the performance requirement itself. The performance requirement should be set using a wide enough range of conditions such that the device and model under test can be proven to generalize and work across the range of expected conditions. A good way to achieve this is to use system level data also for testing. During RAN4#111, it was pointed out that using system data may create an issue, since throughput metrics may not capture poor performance for slots in which the throughput is low due to low SNR. This deficiency could potentially be addressed by basing the metric on BLER rather than throughput.

For the performance requirement, discuss to use system data and BLER as a metric.

System level data may not be appropriate if the model is doing temporal prediction. If system data is not used, then a variety of different link conditions, considering different channel model states, Doppler, SNR points etc. should be presented to the UE during the test or tests.

If link level evaluation is needed for performance requirements, define link level assumptions for a wide spread of channel conditions, Doppler, SINR etc.

# 3	Further elaboration of option 4

## 3.1	Definition of option 4

Option 4 is an option in which a reproduceable test decoder is used for testing, but the test decoder itself is not captured in the specification. The information captured in the specification must be sufficient such that different TE vendors (and eventually NW vendors) can create decoders that produce reproduceable and consistent test results.

## 3.2	On the need for a reference encoder for option 4

In order to achieve reproduceable tests, the specification will need to capture the following aspects:

- The latent space representation to be used by the encoder/decoder

- The dataset over which the encoder/decoder is to be trained

One way to capture these two aspects would be to capture a dataset for training the test decoder. However, the dataset would need to be large enough to create a sufficient test decoder. It is also not entirely intuitive that decoders trained with the dataset would all end up with the same latent space representation.

The reference encoder used to derive the encoder/decoder pair would contain a representation of the latent space. If a decoder would be trained with the reference encoder, the test conditions would be wide and diverse enough to capture real network conditions and a decoder trained based on interoperation with the reference decoder would achieve all of the performance requirements, then the decoder would be a suitable test decoder. For this reason, the most efficient way to implement option 4 would be to capture the reference encoder and the test description in the specifications

For option 4, capture the reference encoder in the TS.

## 3.3	On the difference between option 3 and option 4

The end goal of both option 3 and option 4 is to enable TE to be based on a common and standardized decoder that enables reproducible and consistent testing. For option 3, the decoder is explicitly specified. For option 4, an encoder is specified that enables the decoder to be derived by the TE vendor.

- For option 3, a test decoder, reference encoder and a set of test conditions/data are specified

- For option 4, a reference encoder and a set of test conditions/data are specified

For both options, it is necessary to firstly identify an encoder/decoder pair in RAN4 with agreeable characteristics and performance. The difference comes in whether a test decoder (and reference encoder) are standardized or a reference encoder. The methodology for selecting the encoder/decoder pair can be common for both options.

Use the same procedure for identifying an encoder/decoder pair in RAN4 for either option 3 or option 4.

# 4	Simulations for determining the encoder / decoder pair

In order to agree on an encoder / decoder pair for option 3 or 4, it is necessary to do the at least the following:

- Agree on backbone and structure for the encoder and decoder models.

- Agree on implementation considerations such as fixed/floating point etc.

- Agree on some conditions outside of the model, such as pre-processing for the data input, quantization of the encoder output and any post-processing at the decoder output.

- Agree on the dataset to be used for evaluating the proposed encoder / decoder pairs.

As observed in observation 3, the above points will need a lot of discussion during a real standardization WI.

After these aspects have been agreed, companies will develop and bring encoder/decoder models to RAN4. As part of the evaluation, it is necessary to determine the extent of convergence between companies, understand why and how different proposed encoder/decoder pairs perform differently. Possibly merge models or datasets and then finally select an encoder/decoder pair. The stages of evaluating different proposed encoder/decoder are not well understood, since RAN4 has not attempted this kind of standardization before.

At RAN4#111, to facilitate study and an improved understanding of the process needed to get to an agreeable model, a set of temporary assumptions on model backbone, structure, other conditions, and test dataset was agreed. The agreed assumptions are not intended to be the same as the assumptions that would be made in a WI. If a WI is commenced, then based on guidance from RAN1 a detailed discussion will be needed in order to select real assumptions for model backbone, structure etc. However, since a real set of assumptions is not feasible at the current time, temporary assumptions are used to bypass the assumptions discussion and work on studying the process for getting to an agreeable model (which may or may not be a useful model in the end).

The assumptions suggested after RAN4#111 are captured in the annex. Based on these assumptions, we have created and evaluated an encoder / decoder pair. In addition to the evaluation results, we present other observations on assumptions that we have made

## 4.1	Assumptions used for developing the model

Model architecture

Based on the discussion in RAN4#111 and the subsequent email thread, the model architecture as shown in Figure 1 is agreed to be evaluated.

| Encoder block | Decoder block |
| --- | --- |

Figure 1. The agreed architecture to be evaluated.

From the above figure, the output of the Fully Connected layer (FC) layer will be fed to the Scalar Quantization (SQ) block. However, it is not completely clear on what is consisted in that SQ block. The simplest form of SQ is the fixed uniform SQ, in which an input with a finite and fixed range will be divided into Q quantization points (for 2 bits quantization, there will be 4 quantization points) where the distance between the quantization points is equal. Here, the most straightforward range for the quantization input is between -1 to 1 (or 0 to 1 depends on the preference). To transform the wide range of the FC layer to a more fixed predetermined range (e.g., between -1 to 1), an activation function can be added between the FC block and the SQ. In certain, such operation (e.g., the activation function) may be included as part of the SQ block. However, for the sake of clarity, we prefer such operation is put between the FC layer and the SQ block. In our evaluation, tanh activation function is used, i.e., to limit the range of the input to the SQ block to -1 to 1. The block diagram between the FC layer and the encoder output can be seen in Figure 2.

Figure 2. An activation function is applied before a fixed-uniform scalar quantization in the encoder.

An activation function is agreed before the uniform scalar quantization

Dataset generation

For the training and testing purpose, a training dataset and a testing dataset have been generated under the following assumption:

Table 1. Parameters used to generate the training and testing dataset.

| Parameters | Values |
| --- | --- |
| Duplexing mode | FDD, OFDM |
| Multiple access | OFDMA |
| Frequency range | FR1 2GHz |
| Inter-BS distance | 200m |
| Antenna setup and port layouts at gNB | 32 ports: (8,8,2,1,1,2,8), (dH, dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE | 4RX: (1,2,2,1,1,1,2), (dH, dV) = (0.5, 0.5)λ |
| BS Tx power | 41dBm for 10MHz |
| BS antenna height | 25m |
| UE receiver noise figure | 9dB |
| Slots | 14 OFDM symbols |
| SCS | 15kHz |
| Bandwidth | 10 MHz |
| Frame structure | All downlink |
| MIMO scheme | SU-MIMO, layer = 1 |
| UE distribution | 80% indoor (3km/h); 20% outdoor (30km/h) |
| Channel estimation | Ideal |

As there is no agreement on the number of training dataset that should be used during the training, we apply several sizes of training dataset, in particular, we use between 100K to 600K training samples. Note that during Rel. 18 AI PHY study in RAN1, companies have varying size of training dataset, ranging from 100K to 1200K samples. In generating the dataset, an ideal channel estimation is used. Here, the ideal channel in our dataset also includes the condition of no interference from other UEs. I.e., the sample is generated and logged in a per single active UE basis instead of having many active UEs in one deployment.

Hyper-parameter

During the training phase, hyper-parameters need to be setup (fine-tune). Using different hyper-parameter setups (although sometimes not dramatic), may result in different model performance during. In our evaluation, the followings are used during the training.

Table 2. Hyper-parameters used during the training.

| Hyper parameters | Values |
| --- | --- |
| Batch size | 32 |
| Loss function | NMSE |
| Optimizer | Adam |
| Learning rate | Warmup cosine decay, 10-4 to 10-5 |

## 4.2	Evaluation results

Basic evaluation

The followings are the evaluation results according to our setup:

Table 3. Evaluation results for the agreed CNN model

| Compression method | # Training dataset | SGCS | Gain |
| --- | --- | --- | --- |
| Rel. 16 eType II PC 1 | #N/A | 0.7001 |  |
| AI-based compression (64 bit) | 200 K | 0.6704 | -4.2% |
|  | 400 K | 0.6891 | -1.6% |
|  | 800 K | 0.7085 | 1.2% |

From the above table, we can observe that the AI Compression with the above architecture can provide a gain compared to the legacy mechanism when the training dataset is sufficient, e.g., 800K in this case. Note that adding more training dataset may further improve the performance. However, a saturation on the performance improvement after a certain size of training dataset is expected. Considering the differences on the performance for a plurality of training dataset sizes, 3GPP may need to agree, e.g., on the minimum size of training dataset, e.g., if a discussion on determining the standard/reference model takes place.

A dataset size is identified

Other evaluation

As mentioned previously, this “calibration evaluation” should only serve as a calibration phase so that companies have the same understanding/assumption on the evaluation. I.e., this should not hinder the implementation of different models and/or input types for the future discussion.

As an example, we conduct an evaluation with a different input type. In the model depicted in Figure 1, the input of the model is the eigenvector of the channel. Instead of using eigenvector, a pre-processing is done to the channel to obtain a beam-delay domain representation of the channel (similar to part of steps done to obtain Rel. 16 eType II codebook, CB). However, a larger number of beams and taps (compared to the maximum taps and beams in Rel. 16 CB are used, i.e., 10 beams and 12 taps are used as the input of the model. In addition, the selected beams and taps are not quantized and remains as floating point when is used as the encoder inputs (i.e., in Rel. 16 CB, 3-bit amplitude quantization and 4-bit phase quantization is used). The evaluation results can be seen in Table 4.

Table 3. Evaluation results for the agreed CNN model with beam-delay domain inputs

| Compression method | # Training dataset | SGCS | Gain |
| --- | --- | --- | --- |
| Rel. 16 eType II PC 1 | #N/A | 0.7001 |  |
| AI-based compression (64 bit) - W2-like input | 100 K | 0.7178 | 2.5% |
|  | 200 K | 0.7235 | 3.3% |

From the above figure, we can observe that the performance of having beam-delay domain representation of the channel gives better performance compared to having eigenvector as inputs. Considering this, the model input type may need to also be considered if a discussion on determining the standard/reference model takes place, e.g., during WI.

## 4.3	Format for sharing datasets and models

To facilitate convergence for option 3 or option 4, it may be necessary to share datasets. Whether the dataset sharing would be temporary, permanently stored somewhere, part of a TR or even part of the TS depends on the final details of the RAN4 approach.

However dataset sharing would be used, a common format for the dataset would be needed facilitate access by multiple parties. Different formats are available, such as a compress Numpy dataset, a CSV. There is a need to discuss carefully and ensure that formats that are used enable wide and open access. The storage location is also an important point.

There is a need to carefully consider a representation for datasets that enables wide and open access.

Apart from datasets, transfer or storage of models may be needed. Capturing of models in the TS is obviously necessary for option 3. Furthermore, capturing of reference models may be needed for developing requirements, even for other use cases, either temporarily or in technical reports. A range of different frameworks for model representation exist and they might not always be compatible between each other (Pytorch, TensorFlow, Keras,SciKit Learn, Jax, …). Similarly to the case with dataset sharing, it is important to ensure an open and wide access.

There is a need to consider a model representation that enables a wide and open access.

The selected method to share models might change depending on what is the purpose; it could be only for performance verification, or it could be for integrating a reference model into hardware, which requires a deep and detailed descriptions of the architecture and weights. If only a performance verification is necessary, then a Github repository containing all the details might be enough to allow all company to reproduce the result, it could include a Docker file and/or a python environment requirement. When more detail about the model is required, a graph representation of the model could be required, together with the weights. The graph representation of model will describe all the operations, based on an existing set of available operation, and will need to be agreed in RAN4. This common representation would then be used to recreate the ML model in each preferred framework.

The framework used for sharing models may depend on the purpose of sharing (performance verification, detailed implementation in hardware etc.)

A common graph representation of the shared ML model might be used to describe completely the models architecture.

# Conclusion

In the previous sections we made the following observations:

Observation 1	The test decoder will set the latent space that is contained within real encoders and decoders.

Observation 2	The test decoder and the latent space it represents must be sufficiently rich to be useful for real world operation.

Observation 3	A large amount of discussion will be needed to fix assumptions on model structure, complexity and some detailed implementation aspects in a real WI.

Observation 4	The test decoder need not be interoperable with all reference encoders considered during the RAN4 evaluation.

Observation 5	If convergence is observed in performance of different encoder/decoder pairs under the same evaluation conditions then the difference between different companies encoder/Decoder models is in the latent space representation, but not the performance.

Observation 6	Once the standardized test decoder is available, encoder vendors can create real encoders that can interoperate with the standardized encoder.

Observation 7	There is a need to carefully consider a representation for datasets that enables wide and open access.

Observation 8	There is a need to consider a model representation that enables a wide and open access.

Observation 9	The framework used for sharing models may depend on the purpose of sharing (performance verification, detailed implementation in hardware etc.)

Observation 10	A common graph representation of the shared ML model might be used to describe completely the models architecture.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For option 3, capture both the test decoder and associated reference encoder in the TS.

Proposal 2	Consider the simulation assumptions and criteria for selection of the test decoder in RAN4 separately to the simulation assumptions and criteria for setting the performance requirement.

Proposal 3	Base the simulations for evaluation of which test decoder to select on system data in order that the selected test encoder can be shown to have reasonable performance across the range of operating conditions.

Proposal 4	For the performance requirement, discuss to use system data and BLER as a metric.

Proposal 5	If link level evaluation is needed for performance requirements, define link level assumptions for a wide spread of channel conditions, Doppler, SINR etc.

Proposal 6	For option 4, capture the reference encoder in the TS.

Proposal 7	Use the same procedure for identifying an encoder/decoder pair in RAN4 for either option 3 or option 4.

Proposal 8	An activation function is agreed before the uniform scalar quantization

Proposal 9	A dataset size is identified

# Annex

Fig 1. Detailed model diagrams of encoder.      Fig 2. Detailed model diagrams of decoder.

Table 1. CNN model parameters for encoder.

| Model architecture parameters for encoder (convolutional) | Model type | CNN (Resnet like design) |
| --- | --- | --- |
|  | Num of Resnet blocks | 6 |
|  | Layer after resnet blocks | 2 layers (one FC layer, one Conv layer) |
|  | Layer before resnet blocks | 1 layers (one conv layer) |
|  | Layer within a resnet block | 2 Conv layer + 1 layer for combination |
|  | Kernel/Channel size within a resnet block | (3,3,32) |
|  | Size of message (feature map) passed across layers | (13,32) |

Table 2. CNN model parameters for decoder.

| Model architecture parameters for decoder (convolutional) | Model type | CNN (Resnet like design) |
| --- | --- | --- |
|  | Num of Resnet blocks | 6 |
|  | Layer before resnet blocks | 2 layers (one FC layer, one Conv layer) |
|  | Layer after resnet blocks | 1 layers (one conv layer) |
|  | Layer within a resnet block | 2 Conv layer + 1 layer for combination |
|  | Kernel/Channel size within a resnet block | (3,3,128) |
|  | Size of message (feature map) passed across layers | (13,32) |

Other model related parameters:

| Activation function | 1:ReLU or 2:LeakyReLU |
| --- | --- |
| Latent size | 64 (112 bits optional) |
| Quantization | 2 bits |

System parameters:

| Frequency range | 2GHz(4GHz optional) |
| --- | --- |
| MIMO Layers | 1 layer (2 layers optional) |
| Sub-bands | 13 |
| Channel estimation | ideal |
| Metrics | NMSE and SGCS |