3GPP TSG-RAN WG4#112	R4-2412610

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda item:	8.1.1.3.1

Source: 	Qualcomm France

Title: 	6Rx REFSENS

Document for:	Approval

- Introduction

Analysis and considerations 6Rx requirements are provided in this contribution.

- WID description and WF in RAN4#111

WID [1] includes the following content for 6Rx:

6Rx for handheld and FWA UE

- Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario

  - Example bands: n41, n77/n78, n79, n104

  - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers

  - Specify the Rx requirements including reference sensitivity requirements for support 6Rx

    - Note: the specified requirements can be applicable to both handheld UE and FWA devices

  - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability

  - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

General for all the areas

- Specify release independence requirements for the core part in TS 38.307/36.307, if needed and feasible.

WF [2] agreed in RAN4#111 says the following for REFSENS:

# Topic 1:	REFSENS (delta RIB,6R)

## Sub-topic 1-1:	General considerations for specifying ΔRIB,6R value

Issue 1-1-1: Whether band n104 should be included in the high band (n77, n78 and n79) category for 6Rx case

Agreement: For ΔRIB,6R, include n77, n78, n79 and [n104] into the same group.

Issue 1-1-2: Whether to use same ΔRIB,6R value for handheld UE and FWA

Way Forward: Further discuss the following options.

-	Option 1: RAN4 needs to determine whether to define different ΔRIB,6R value for handheld UE and FWA separately

-	Option 2: Same value for handheld UE and FWA

-	Option 3: Different value for handheld UE and FWA

Issue 1-1-3: Release independence of ΔRIB,6R value

Way Forward: Follow RAN4 Chair guidance and defer the release independence discussions until the requirements are stable.

## Sub-topic 1-2:	ΔRIB,6R values for handheld UE and FWA

Issue 1-2-1: Proposed ΔRIB,6R values for handheld UE and FWA

Way Forward: Further discuss the following options for the ΔRIB,6R values. Ultimately, the decision to specify separate or same requirements for handheld UE and FWA will be dependent on outcome of Issue 1-1-2.

-	Option 1: Consider range of ΔRIB,6R in brackets for each case below for further discussion.

| Operating bands | ΔRIB,6R for FWA | ΔRIB,6R for Handheld |
| --- | --- | --- |
| n77, n78, n79, [n104] | [-3.0 ~ -3.3] dB | [-3.0 ~ -3.2] dB |
| n41 | [-3.0 ~ -4.0] dB | [-3.0 ~ -4.0] dB |

-	Option 2: Consider average value of ΔRIB,6R from company proposals for further discussion.

-	Option 3: Other options are not precluded.

.

- Discussion

Issue 1-1-1: Whether band n104 should be included in the high band (n77, n78 and n79) category for 6Rx case

Open aspect in the WF was whether n104 is included in same group for ΔRIB,6R as bands n77/n78/n79.

n104 is obviously higher frequency band than n77/n78/n79, and the higher frequency is accounted in baseline 2Rx REFSENS for n104. Going from 4Rx for which ΔRIB,4R is already specified for n104 to 6Rx, the two additional RX chains may, depending on the implementation have a bit higher loss than those for n77/n78/n79.

Ideally, it would be good to have n104 belonging to the same group as n77/n78/n79, but to us this is conditional to the ΔRIB,6R value itself. If the ΔRIB,6R for n77/n78/n79 is -3.0dB, then our view is n104 can belong to the same group. If ΔRIB,6R for n77/n78/n79 is tighter than -3.0dB, then n104 should be in different group.

Proposal 1: If the ΔRIB,6R for n77/n78/n79 is -3.0dB then n104 can belong to the same group

Issue 1-1-2: Whether to use same ΔRIB,6R value for handheld UE and FWA

It is very challenging to assess the RFFE IL’s and general complexity differences in such a rigorous manner to justify 0.1dB or 0.2dB difference in ΔRIB,6R value. Looking at the spread among different companies’ proposals for FWA and handheld, the difference between those two is virtually negligible. We support the idea of having just a single ΔRIB,6R which applies for both FWA and handheld.

Proposal 2: Use same ΔRIB,6R value for handheld UE and FWA

Issue 1-2-1: Proposed ΔRIB,6R values for handheld UE and FWA

We did further evaluations on REFSENS by looking at currently agreed 4Rx and 8Rx values. We looked into the impact of adding RX chains #3 and #4, then adding RX chains #5, #6, #7, and #8 using the currently agreed ΔRIB,4R and ΔRIB,8R values, and finally evaluated respective ΔRIB,6R. We came up with the following values.

| Operating band | ΔRIB,6R (dB) |
| --- | --- |
| n41 | -3.4 |
| n77, n78, n79, n104 | -3.0 |

Proposal 3: Use -3.4dB ΔRIB,6R for n41 and -3.0dB ΔRIB,6R for n77/n78/n79/n104

- Conclusion

# REFSENS for 6Rx was discussed, with the following proposals

Proposal 1: If the ΔRIB,6R for n77/n78/n79 is -3.0dB then n104 can belong to the same group

Proposal 2: Use same ΔRIB,6R value for handheld UE and FWA

Proposal 3: Use -3.4dB ΔRIB,6R for n41 and -3.0dB ΔRIB,6R for n77/n78/n79/n104

- Reference

[1] RP-240828, “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, Huawei

[2] R4-2410751, “WF on requirements for 6Rx”, AT&T