3GPP TSG-RAN WG4#112	R4-2412611

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda item:	8.1.1.3.3

Source: 	Qualcomm France

Title: 	6Rx SRS antenna switching requirements

Document for:	Approval

- Introduction

Analysis and considerations 6Rx SRS AS requirements are provided in this contribution.

- WID description and WF in RAN4#111

WID [1] includes the following content for 6Rx:

6Rx for handheld and FWA UE

- Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario

  - Example bands: n41, n77/n78, n79, n104

  - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers

  - Specify the Rx requirements including reference sensitivity requirements for support 6Rx

    - Note: the specified requirements can be applicable to both handheld UE and FWA devices

  - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability

  - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

General for all the areas

- Specify release independence requirements for the core part in TS 38.307/36.307, if needed and feasible.

WF [2] agreed in RAN4#111 says the following for SRS antenna switching:

# Topic 2:	SRS antenna switching and ΔTRxSRS

## Sub-topic 2-1:	General considerations for SRS antenna switching and ΔTRxSRS

Issue 2-1-1: SRS antenna switching configurations

Way forward: RAN4 to focus on discussing ∆TRxSRS for 1tr6 and t2r6 until RAN1 concludes work on 3t6r and 4t6r. Pending RAN1 feedback, RAN4 can consider any necessary updates to the work scope in a Revised WID for the SRS antenna configurations subject to RAN Plenary approval.

Issue 2-1-2: Whether to use different ∆TRxSRS based on operating frequency

Way forward: RAN4 defines different ∆TRxSRS values based on operating frequency. Further discuss whether to introduce an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104.

## Sub-topic 2-2:	ΔTRxSRS values

Issue 2-2-1: Proposed ∆TRxSRS values

Way forward: Further discuss the following options for the ∆TRxSRS requirements.

-	Option 1: Consider average value of ∆TRxSRS for further discussion.

| Operating bands | ΔTRxSRS requirement for t1r6 | ΔTRxSRS requirement for t2r6 | ΔTRxSRS requirement for t1r6-t2r6 |
| --- | --- | --- | --- |
| Band n41, n77, n78 | 4.0 dB | 3.4 dB | 4.4 dB |
| Band n79, [n104] | 5.3 dB | 4.5 dB | 5.7 dB |

-	Option 2: Consider range of ∆TRxSRS in brackets for each case below for further discussion.

| Operating bands | ΔTRxSRS requirement for t1r6 | ΔTRxSRS requirement for t2r6 | ΔTRxSRS requirement for t1r6-t2r6 |
| --- | --- | --- | --- |
| Band n41, n77, n78 | [2.8 ~ 5.0] dB | [2.3 ~ 4.0] dB | [3.3 ~ 5.5] dB |
| Band n79, [n104] | [3.6 ~ 6.0] dB | [3.0 ~ 5.5] dB | [4.2 ~ 6.5] dB |

Note: Maximum level for t1r6 corrected for OPPO proposals per OPPO.

-	Option 3: Other options are not precluded.

- Discussion

Issue 2-1-2: Whether to use different ∆TRxSRS based on operating frequency

Current SRS Antenna switching specification is written in a way that it can be interpreted in two ways; one is that for a band starting at or above n79 FUL_low the same SRS AS allowance holds as for n79. Another interpretation is that because there was no band for which 4Rx requirements are specified above n79 at the time when SRS specification was agreed, the specification is meant for n79 and not above even the exact wording is ambiguous.

We compared and made a rough assessment of the worst case IL of the components in RFFE at n79 and at n104 frequencies. It’s well known that the IL of passives and PCB trace is higher at 7GHz than at 5GHz. Because of that, there is a need to have separate ∆TRxSRS for n104.

Figure 1 Worst case IL estimates for components in SRS path

Proposal 1: Add a new ∆TRxSRS value for bands whose FUL_low is higher than 5GHz

Issue 2-2-1: Proposed ∆TRxSRS values

We investigated different RFFE examples using the numbers shown in figure 1 and concluded that the difference between n79 and n104 IL is about 1dB. To offer enough flexibility on UE design, and to be able to have a single set of requirements for 6Rx, we suggest applying same ΔTRxSRS as specified for 1T8R, 2T8R, and 1T8R-2T8R for bands n41/n77/n78/n79, and to have 1dB larger allowance for n104 than for n79. On paper, it might be possible to show examples were the needed allowance is a bit less that that allowed for 8Rx but in practice there are so many aspects to be accounted which cannot be fully factored so it really would be beneficial to use 8Rx values which in part could ease using SRS AS for 6Rx.

Please note that these values are for the said “PC3” case in the specification, and 3dB is added to said “PC2” case as specified for 2Rx, 4Rx, 8Rx in current specification.

| Operating bands | ΔTRxSRS requirement for t1r6 | ΔTRxSRS requirement for t2r6 | ΔTRxSRS requirement for t1r6-t2r6 |
| --- | --- | --- | --- |
| Band n41, n77, n78 | 4 dB | 4 dB | 4.5 dB |
| Band n79 | 5.5 dB | 5.5 dB | 6 dB |
| Band n104 | 6.5 dB | 6.5 dB | 7 dB |

Proposal 2: Use the following ΔTRxSRS values for 1T6R, 2T6R, and 1T6R-2T6R when the device is power class 3 or power class 5 or power class 1.5 in the band, or when the device is power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating Tx diversity capability.

| Operating bands | ΔTRxSRS requirement for t1r6 | ΔTRxSRS requirement for t2r6 | ΔTRxSRS requirement for t1r6-t2r6 |
| --- | --- | --- | --- |
| Band n41, n77, n78 | 4 dB | 4 dB | 4.5 dB |
| Band n79 | 5.5 dB | 5.5 dB | 6 dB |
| Band n104 | 6.5 dB | 6.5 dB | 7 dB |

Proposal 3: Use the following ΔTRxSRS values for 1T6R, 2T6R, and 1T6R-2T6R when the device is power class 2 in the band and ΔPPowerClass = 0 dB and not indicating Tx diversity capability

| Operating bands | ΔTRxSRS requirement for t1r6 | ΔTRxSRS requirement for t2r6 | ΔTRxSRS requirement for t1r6-t2r6 |
| --- | --- | --- | --- |
| Band n41, n77, n78 | 7 dB | 7 dB | 7.5 dB |
| Band n79 | 8.5 dB | 8.5 dB | 9 dB |
| Band n104 | 9.5 dB | 9.5 dB | 10 dB |

- Conclusion

# 6Rx SRS antenna switching was considered in this contribution with the following proposals:

Proposal 1: Add a new ∆TRxSRS value for bands whose FUL_low is higher than 5GHz

Proposal 2: Use the following ΔTRxSRS values for 1T6R, 2T6R, and 1T6R-2T6R when the device is power class 3 or power class 5 or power class 1.5 in the band, or when the device is power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating Tx diversity capability.

| Operating bands | ΔTRxSRS requirement for t1r6 | ΔTRxSRS requirement for t2r6 | ΔTRxSRS requirement for t1r6-t2r6 |
| --- | --- | --- | --- |
| Band n41, n77, n78 | 4 dB | 4 dB | 4.5 dB |
| Band n79 | 5.5 dB | 5.5 dB | 6 dB |
| Band n104 | 6.5 dB | 6.5 dB | 7 dB |

Proposal 3: Use the following ΔTRxSRS values for 1T6R, 2T6R, and 1T6R-2T6R when the device is power class 2 in the band and ΔPPowerClass = 0 dB and not indicating Tx diversity capability

| Operating bands | ΔTRxSRS requirement for t1r6 | ΔTRxSRS requirement for t2r6 | ΔTRxSRS requirement for t1r6-t2r6 |
| --- | --- | --- | --- |
| Band n41, n77, n78 | 7 dB | 7 dB | 7.5 dB |
| Band n79 | 8.5 dB | 8.5 dB | 9 dB |
| Band n104 | 9.5 dB | 9.5 dB | 10 dB |

- Reference

[1] RP-240828, “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, Huawei

[2] R4-2410751, “WF on requirements for 6Rx”, AT&T