3GPP TSG-RAN WG4#112	R4-2412612

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda item:	8.5.2

Source: 	Qualcomm France

Title: 	Non-collocated Intra-band NR CA/EN-DC UE requirements

Document for:	Approval

- Introduction

Considerations on open aspects for Non-collocated NR CA/EN-DC UE are provided in this contribution.

- WF in RAN4#111

WF [1]:

  - Sub-topic 2-1: UE RF requirements for Type 4 EN-DC/NR-CA

< Issue 2-1-1:  Power imbalance requirements and REFSENS >

Agreement:

- Use 25dB power imbalance for 4Rx REFSENS + 1dB for Type 4

< Issue 2-1-2:  How to capture the power imbalance and REFSENS requirements >

Agreement:

- Define the Type 4 receiver imbalance requirements in the same table as for Type 2.

  - FFS on the details of modification

< Issue 2-1-3:  Minimum DL frequency separation >

Way Forward:

- Continue further discussion on the following two options and conclude it in the next meeting.

  - Option 1. Center of BWanother relative to edge of BWwanted is assumed to be at least 80MHz+BWanother/2 away from the edge of the wanted CC.

  - Option 2. Max (5/2* BWanother, [50] MHz).

< Issue 2-1-4:  Maximum DL frequency separation >

Agreement:

- No limitation for maximum DL frequency separation

  - Sub-topic 2-2: UE Capability(s)/UE behavior and network signalling for Type 4 EN-DC/NR-CA

< Issue 2-2-1:  New UE Capabilities for Type 4a(EN-DC) and 4b(EN-DC/NR-CA) >

Way Forward:

- Continue further discussion on the following two options and conclude it in the next meeting.

  - Option 1. Add new UE capabilities for Type 4a and Type 4b support indication.

  - Option 2. Add one new UE capability for Type 4 (no distinction between Type 4a and 4b) support indication.

< Issue 2-2-2:  Whether to support Type 2 capability by UE having Type 4 capabilities >

Agreement:

- If UE reports the Type 4 capability, Type 2 capability shall be deemed as support by default regardless of whether UE indicates Type-2 capability or not.

  - If UE reports the Type 4 capability, it is not necessary for UE to report Type 2 capabilities.

  - If the issue is identified for these bullets, RAN4 will revisit them.

< Issue 2-2-3:  New BS Signalling to switch between Type 4a/4b and Type 1(collocated) >

Way Forward:

- First, conclude discussions of whether to add new UE capability(s) for Type 4.

- And then, continue further discussion on BS signalling in the next meeting.

< Issue 2-2-4:  New BS Signalling to switch between Type 4a/4b and Type 2 >

Way Forward:

- First, conclude discussions of whether to add new UE capability(s) for Type 4.

- And then, continue further discussion on BS signalling in the next meeting.

< Issue 2-2-5: When to inform RAN2 the demand on new UE capability(s) and new BS signalling(s) >

Way Forward:

- Given the discussions on UE capabilities and network signalling, it is not hurry to send this.

< Issue 2-2-6:  UE behavior between Type 1, Type 2 and Type 4a/4b with new BS signalling >

Way Forward:

- First, conclude discussions of whether to add new UE capability(s) and BS signalling for Type 4.

- And then, continue further discussion on BS signalling/UE behavior in the next meeting.

  - Sub-topic 2-3: Other aspects (incl. clarification of contiguous LTE CCs)

< Issue 2-3-1:  The number of B42 CCs for EN-DC >

Way Forward:

- Check the number with operators until next meeting again.

< Issue 2-3-2:  The number of NR CCs for EN-DC and NR-CA >

Way Forward:

- Check the number with operators until next meeting again.

< Issue 2-3-3:  Terminology for power imbalance requirement to take into account multiple B42 CCs >

Way Forward:

- Align to the agreed WF R4-2406628(Huawei) in the last meeting.

- Discussion

< Issue 2-1-3:  Minimum DL frequency separation >

Way Forward:

- Continue further discussion on the following two options and conclude it in the next meeting.

  - Option 1. Center of BWanother relative to edge of BWwanted is assumed to be at least 80MHz+BWanother/2 away from the edge of the wanted CC.

  - Option 2. Max (5/2* BWanother, [50] MHz).

We suggest using Option 1, as that has been used already in Type 2 requirements.

Proposal 1: For Minimum DL frequency separation, choose Option 1.

< Issue 2-2-1:  New UE Capabilities for Type 4a(EN-DC) and 4b(EN-DC/NR-CA) >

Way Forward:

- Continue further discussion on the following two options and conclude it in the next meeting.

  - Option 1. Add new UE capabilities for Type 4a and Type 4b support indication.

  - Option 2. Add one new UE capability for Type 4 (no distinction between Type 4a and 4b) support indication.

Type 4a has 6Rx antennas/RX paths and Type 4b has 8Rx antennas/RX paths. UE supporting Type 4b EN-DC is obviously able to support Type 4a as well, but UE supporting Type 4a may not be able to support Type 4b EN-DC and NR CA for instance if UE has only 6Rx antennas/Rx paths. Hence, having only single capability could restrict some deployments in principle.

Observation 1: If there is only a single UE capability for Type4, then implementations supporting up to 6Rx for n42/n77/n78 cannot support this feature

Proposal 2: Specify separate UE capabilities for Type 4a and Type 4b

< Issue 2-2-3:  New BS Signalling to switch between Type 4a/4b and Type 1(collocated) >

Way Forward:

- First, conclude discussions of whether to add new UE capability(s) for Type 4.

- And then, continue further discussion on BS signalling in the next meeting.

Rel-18 has the following BS signaling:

nonCollocatedTypeMRDC This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD inter-band (NG) EN-DC with overlapping or partially overlapping bands. If this field is present, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in TS 38.133 [14] and inter-band RF requirements. If this field is absent, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 [14] and inter-band RF requirements when indicating support of interBandMRDC-WithOverlapDL-Bands-r16.

nonCollocatedTypeNR-CA This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA. If this field is present, the UE applies MRTD according to Table 7.6.4-1 in TS 38.133 [14] and UE RF requirements for intra-band NR-CA except for 7.10A in TS 38.101-1 [15]. If this field is absent, the UE applies MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 [14] and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in TS 38.101-1 [15] when indicating support of intraBandNR-CA-non-collocated-r18.

Scope of this WID is FWA only, so if there is no mobility at all for FWA device the deployment is “static” from UE perspective. However the power difference is not fixed, instead only the max power difference is specified. Hence depending on the FWA device location wrt. Basestations the power difference can be between 0…25dB.

If there is no R19 BS signaling, and UE would support Type 4a/4b and indicate MaxMIMO-layers=4 then R18 NW signaling elements would always be absent, which would lead UE to assume deployment is non-collocated.

During the offline discussion before this meeting, companies had quite diverse views on the necessity of new BS signaling and even on if FWA device has mobility or not, so discussion during the meeting is needed to make sure everyone has same understanding on FWA UE behaviour.

Observation 2: Rel-19 NW signaling to switch between Type 4 and Type 1 could be needed to ensure good UE functionality, pending further discussions.

If there is new signaling defined, cross-release functionality between R18 UE/NW and R19/UE/NW must be considered as well. Especially R19 UE could operate in R18 NW.

-R19 UE indicating Type 2 and Type 4a or 4b capability with MaxMIMO-layers=4:

-R18 NW understands only Type2 with MaxMIMO-layers=4 => Assumes Type 1

In this case, because NW assumes UE to support Type 1 with 4L/CC it could not configure UE at all if deployment is Non-collocated. If deployment is collocated NW could normally configure UE for Type1 with 4L/CC

-R19 does not see R19 NW signaling and R18 NW signaling would be absent, so UE would assume Non-collocated operation (Type 4a or 4b)

So even if NW configures UE for Type 1 with 4L/CC, UE would assume deployment is non-collocated and would configure it’s HW/SW accordingly which would be suboptimal.

Observation 3: Potential discrepancy at between R19 UE and R18 NW must be addressed if new R19 signaling is defined.

< Issue 2-2-4:  New BS Signalling to switch between Type 4a/4b and Type 2 >

Way Forward:

- First, conclude discussions of whether to add new UE capability(s) for Type 4.

- And then, continue further discussion on BS signalling in the next meeting.

Unless the previous assumption of Type 4 UE supporting Type 2 is reverted, there is no need for signaling.

Proposal 3: Unless the previous assumption of Type 4 UE supporting Type 2 is reverted, there is no need for signaling.

< Issue 2-2-5: When to inform RAN2 the demand on new UE capability(s) and new BS signalling(s) >

Way Forward:

- Given the discussions on UE capabilities and network signalling, it is not hurry to send this.

There is no rush, RAN4 should have the intermediate check first in December if Type 3a/3b is included in the WID scope. RAN4 should send just one LS into RAN2 including all UE capabilities and NW signaling decisions.

Proposal 4: RAN4 should first see if Type 3a/3b is included later into the WID scope, and after that send all UE capability and NW signaling decisions related into this WDI into RAN2 on one go.

< Issue 2-2-6:  UE behavior between Type 1, Type 2 and Type 4a/4b with new BS signalling >

Way Forward:

- First, conclude discussions of whether to add new UE capability(s) and BS signalling for Type 4.

- And then, continue further discussion on BS signalling/UE behavior in the next meeting.

UE supporting Type 4a/4b could optionally support 6L/8L per CC in collocated mode (Type 1). The default is anyhow 4L/CC for Type 1. There is no new UE capability/signaling required for that as UE can already in current specifications indicate MaxMIMO-layers.

Proposal 5: UE supporting Type 4a/4b could optionally support 6L/8L per CC in collocated mode (Type 1).

- Conclusion

Remaining aspects for Non-collocated Rel-19 Intra-band Non-contiguous CA/EN-DC were considered with the following observations and proposals.

Proposal 1: For Minimum DL frequency separation, choose Option 1.

Observation 1: If there is only a single UE capability for Type4, then implementations supporting up to 6Rx for n42/n77/n78 cannot support this feature.

Proposal 2: Specify separate UE capabilities for Type 4a and Type 4b.

Observation 2: Rel-19 NW signaling to switch between Type 4 and Type 1 could be needed to ensure good UE functionality, pending further discussions.

Observation 3: Potential discrepancy at between R19 UE and R18 NW must be addressed if new R19 signaling is defined.

Proposal 3: Unless the previous assumption of Type 4 UE supporting Type 2 is reverted, there is no need for signaling.

Proposal 4: RAN4 should first see if Type 3a/3b is included later into the WID scope, and after that send all UE capability and NW signaling decisions related into this WDI into RAN2 on one go.

Proposal 5: UE supporting Type 4a/4b could optionally support 6L/8L per CC in collocated mode (Type 1).

- Reference

[1] R4-2410686, “WF on Rel-19 non-collocated UE RF requirements”, KDDI