3GPP TSG-RAN WG4#112	R4-2412613

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda item:	5.2

Source: 	Qualcomm France

Title: 	About issue in current PC2 MSD specification

Document for:	Approval

- Introduction

Reviewing the most recent version of 38.101-1 we found an issue in PC2 MSD which needs to be addressed.

- Discussion

In all previous specification versions, the Cross-band and Harmonic Mixing MSD for PC2 included only cases where the aggressor band is a TDD band. For PC2 TDD, the Tx Diversity is allowed meaning PC2 can be done either with one or two TX paths. Hence the MSD PC2 specification for combinations with TDD aggressor holds both for UE’s supporting TX Diversity and for UE’s not supporting TX diversity. Consequently, PC2 Cross-band MSD had one table transparent to TX Diversity, and PC2 Harmonic Mixing had one table transparent to TX diversity.

In most recent version 18.6.0 the first PC2 Cross-band and Harmonic Mixing cases with FDD aggressor were included. Companies had done analysis, and TP’s respectively included the MSD requirements separately for 1TX (UE’s not supporting TX Diversity) and for 2TX (UE’s supporting TX diversity). The actual table format in Version 18.6.0 is anyhow problematic. We regret we didn’t realize that the TP format would lead into problematic specification table format. Version18.6.0 has separate tables for UE’s not supporting TX diversity and for UE’s supporting TX diversity. The cases with TDD aggressor are only in the table for UE’s not supporting TX diversity.

Hence, the most recent version of the specification has inadvertently removed MSD requirements for UE’s supporting TX diversity for combinations with TDD aggressor.

We note that for UL harmonic case, all PC2 aggressor cases until today are FDD bands so there PC2 MSD was split into UE’s supporting TX diversity and UE’s not supporting TX diversity which in principle is not problematic as long as all aggressors are PC2 FDD. In future there will be also PC2 UL harmonic cases with TDD aggressor so issues are foreseen also in UL harmonics for PC2.

Observation 1: Recent change in PC2 Cross-band and Harmonic Mixing tables inadvertently removed MSD from all TX Diversity combinations where aggressor is TDD band so current table format with separate 1TX and 2TX tables is not correct.

Observation 2: In future there will be also PC2 UL harmonic cases for TDD aggressor so current table format with separate 1TX and 2TX tables are not optimal.

At least our intention was to have just one table with old table header, but with Note for FDD aggressor cases whether MSD spec is for 1Tx or for 2Tx.

We propose to revert back to single table with original header shown above, and with FDD MSD cases shown as in example below.

Table 7.3A.6-1a: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC2 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n25 | n66 | 1890 | 40 | 15 | 50 (RBstart=176) | 2112.5 | 5 | 0.84 | >ACLR2 |
| n25 | n66 | 1890 | 40 | 15 | 50 (RBstart=176) | 2112.5 | 5 | 1.15 | >ACLR2 |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: Void. NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. NOTE 4: Applicable to UE supporting PC2 with single Tx. NOTE 5: Applicable to UE supporting PC2 with dual Tx. |  |  |  |  |  |  |  |  |  |

Proposal 1: Revert back to single PC2 MSD tables for Cross-band, UL Harmonic, and Harmonic mixing, and to use notes as shown above to distinguish 1TX and 2TX FDD aggressor cases.

- Conclusion

An issue found in PC2 MSD tables was found and discussed.

We propose to revert back to single PC2 MSD tables for Cross-band, UL harmonics and Harmonic mixing, and to use notes as shown above to distinguish 1TX and 2TX FDD aggressor cases.

The timing to do this is a bit complex. It would probably be best if the proposed change can be embedded in BigCR submitted for September 2024 Plenary. The general discussion around the future of PC2/PC1.5 MSD may also impact this timeline.

Observation 1: Recent change in PC2 Cross-band and Harmonic Mixing tables inadvertently removed MSD from all TX Diversity combinations where aggressor is TDD band so current table format with separate 1TX and 2TX tables is not correct.

Observation 2: In future there will be also PC2 UL harmonic cases for TDD aggressor so current table format with separate 1TX and 2TX tables are not optimal.

Proposal 1: Revert back to single PC2 MSD tables for Cross-band, UL Harmonic, and Harmonic mixing, and to use notes as shown above to distinguish 1TX and 2TX FDD aggressor cases.

Proposal 2: Implement Proposal 1 as part of BigCR to be submitted into September 2024 Plenary

- Reference

[1] R4-2410605, “WF on HPUE MSD simplifcation”, Skyworks, Murata, Qualcomm, Samsung, Nokia