3GPP TSG-RAN WG4#112	R4-2412614

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda item:	4.2

Source: 	Qualcomm France

Title: 	About n28/n83 30MHz channel confinement

Document for:	Approval

- Introduction

This contribution discusses the current wording for n28/n83 30MHz channel confinement and proposes how to change it.

- Current specification

Below one can find a shortened version of Table 5.3.5-1 including just n28 and n83, and associated Note 7.

Table 5.3.5-1 Channel bandwidths for each NR band

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n28 | 15 | 34 | 5 | 10 | 15 | 207 | 257 | 307 |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  | 10 | 15 | 207 | 257 | 307 |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n83 | 15 |  | 5 | 10 | 15 | 207 | 257 | 307 |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  | 10 | 15 | 207 | 257 | 307 |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 7: For the 20 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 713-723 MHz or 728-738 MHz. For the 25 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 715.5-720.5 MHz or 730.5-735.5 MHz. For the 30MHz bandwidth, the minimum requirements are specified for NR UL transmission bandwidth configuration confined to either 703-733 or 718-748 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

For both 20MHz and 25MHz, the frequency range restrictions are specified for the NR UL carrier frequencies. This means that the UL carrier including GB’s (Guard bands) must be within the boundary frequencies.

However, for 30MHz the confinement is not currently specified as per NR UL carrier but instead per UL transmission bandwidth configuration. Figure 5.3.1-1 from TS38.101-1 well illustrates that the definition of “UL transmission bandwidth configuration” is NRB which is 160RB for 30MHz carrier when using 15kHz.

The current definition or 30MHz channel confinement for n28 and n83 means that the RB allocation could start right from the channel edge, further meaning the entire GB could be outside the channel bandwidth. In other words, the 30MHz channel could be positioned so that it is either ~600kHz (15kHz SCS) outside the band or ~600kHz overlapping with neighboring operator’s spectrum depending on if NR UL transmission bandwidth configuration is at left edge or right edge, respectively. This is in our cannot be the intention and clearly is not correct, and it should be fixed from the release 30MHz BW was introduced for n28/n83 (Rel-16/Rel-17).

We propose to change the Note 7 wording for 30MHz case to be same as that for 20MHz and 25MHz:

Proposal 1: Modify Note 7 as follows:

For the 20 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 713-723 MHz or 728-738 MHz. For the 25 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 715.5-720.5 MHz or 730.5-735.5 MHz. For the 30MHz bandwidth, the minimum requirements are specified for NR UL transmission bandwidth configuration carrier frequencies confined to either 703-733 or 718-748 MHz.

We have accompanying R16/17/18 CR’s for this in Agenda item 4.2.

- Conclusion

n28/n83 30MHz channel confinement issue was considered with the following proposal

Proposal 1: Modify Note 7 as follows:

For the 20 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 713-723 MHz or 728-738 MHz. For the 25 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 715.5-720.5 MHz or 730.5-735.5 MHz. For the 30MHz bandwidth, the minimum requirements are specified for NR UL transmission bandwidth configuration carrier frequencies confined to either 703-733 or 718-748 MHz.

- Reference

# [1] R4-