3GPP TSG-RAN WG4#112	R4-2412618

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda item:	7.14.2

Source: 	Qualcomm France

Title: 	Initial considerations on RF filtering and UE Co-Existence

Document for:	Discussion

- Introduction

Initial considerations on RF filtering and UE Co-Existence are provided.

- WID

The objectives of core part of this work item [1] include:

Specify a new E-UTRA FDD operating band operating in the range 1800-1830 MHz, in a Release independent from Release 13, with the following characteristics:

  - UL: 1800 – 1810 MHz and DL: 1820 – 1830 MHz

  - Band numbering and RF characteristics

  - Support the following channelization: 10, 5, 3 and 1.4 MHz

  - Specify related UE RF core requirements (e.g. UE power class, Additional Maximum Power Reduction (A-MPR), Reference sensitivity, blocking performance)

  - Define PC3 UE power class

  - Consider UE categories M1&M2 and NB1&NB2

  - Specify related BS RF core requirements

  - Consider WA BS, MR BS and LA BS classes

  - Update RRM specification with new band

Note: 	Not impose new UE requirements with this new band as the protected band from the existing 3GPP bands (i.e., Table 6.6.3.2-1 in TS 36.101, and Table 6.5.3.2-1 in TS 38.101-1).

Additionally, and excerpt from justification part in the WID can be found below:

The following assumptions are to be considered:

- Fixed communications only are allowed (point to point and point to multipoint)

- The band is specified to support FDD operation assuming their form factor is CPE-like and PC3 power class and therefore removing constraints on size and some implementation aspects

- NB-IoT and eMTC devices are expected to operate in Half Duplex mode.

- No new requirements on existing bands

- Discussion

UE’s in this band are expected to operate in Half Duplex mode. Obviously, NB-IoT is always Half Duplex, so the limitation is for eMTC devices.

Half Duplex operation in general relaxes RF filtering needs for FDD bands, however there is still at least one important aspect to consider: UE Co-existence and in particular protection of own DL band. For this band, there is only 10MHz duplex gap, so achieving meaningful RF filter attenuation at own DL is difficult. For instance, band 25 is operating roughly same frequency range, and it is a very challenging yet doable band with certain RF filter technologies even duplex gap is 50% larger (15MHz).

If possible, it would be good not to impose stringent RF TX filtering requirements for Half-Duplex band as they would likely impact the RF filter technology and TX insertion loss and hence available UE output power. Achieving e.g. 10/20/30dB RF filter attenuation at 1820-1830MHz using typical RF filter technologies for Half Duplex is challenging based on initial assessment, so RAN4 should carefully consider how to deal with assumptions for RF filter attenuation at own DL.

Observation 1: Based on initial assessment, RF filter may not help at least significantly in own DL protection.

Observation 2: Simulations are needed to see what kind UE emission levels are expected for eMTC devices at own DL without RF filter attenuation.

- Conclusion

Initial considerations on RF filtering and UE Co-Existence are provided.

Observation 1: Based on initial assessment, RF filter may not help at least significantly in own DL protection.

Observation 2: Simulations are needed to see what kind UE emission levels are expected for eMTC devices at own DL without RF filter attenuation.

- Reference

[1] R4-241649, “New WID: Introduction of LTE FDD band in 1800 – 1830 MHz for Canada”, Novamint, Sequans, Semtech, Telit, Ubiik