3GPP TSG-RAN WG4#112	R4-2412619

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda item:	8.1.1.1.2

Source: 	Qualcomm France

Title: 	MSD for HPUE

Document for:	Approval

- Introduction

Considerations on MSD for HPUE is provided in this contribution.

- WF in RAN4#111

WF [1]:

As background information, up until now RAN4 must handle up to 6 different types of MSDs:

- Type 1: 1UL Intra-band MSD,

- Type 2: 2UL Intra-band MSD

- Type 3: Inter-band CA UL harmonic MSD,

- Type 4: Inter-band CA Rx harmonic mixing MSD,

- Type 5: Inter-band CA MSD due to dual-UL IMD interference, including triple beat,

- Type 6: Inter-band CA MSD due to cross-band isolation interference.

With the introduction of single PC2 FDD operation, several inter-band CA MSD test points need to be re-evaluated. Workload is heavy on RAN4.

<Way forward on simplifying HPUE MSD Analyses>

Interested companies are invited to share their views on the following options at the next meeting:

For all types of MSDs:

- Option 1: RAN 4 defines only the default power class inter-band CA and intra-band CA MSD requirements, and the new HPUE MSD requirements are no longer specified in the TS. The legacy agreed HPUE MSD test points are not impacted and are kept in the TS.

- Option 2: RAN 4 defines the default power class inter-band CA and intra-band CA MSD requirements, and RAN4 specifies equations or simplified guidelines (like in [3,4]) or lookup tables “LUT” (like in [5]) that define the relationship between the default power class MSD and the HPUE MSD requirements:

  - Option 2a: Equations/LUT are captured in the RAN4 TS,

  - Option 2b: Equations/LUT are not captured in the RAN4 TS and are provided to RAN5.

- Option 3:  Keep the existing way of working, i.e. RAN4 analyses case by case all HPUE MSD requirements and experts flag TP for TRs for which MSD needs corrections/re-evaluations.

- Other options are not precluded.

- Discussion

Before actual discussion, we note that this topic is strictly speaking broader than agenda 8.1.1.1.2 itself, but in the absence of better place we used this agenda item.

MSD for CA has been specified for >10 years starting from LTE CA. At the time every single CA combination had it’s own work item and usually several companies provided analysis, and then finally some kind of conclusion on the requirements was agreed. A lot of things haven happened since and extremely large amount of band combinations have been specified, not only for LTE CA but especially for EN-DC and NR CA. During past years, more and more HPUE band combinations have been introduced, of which everyone is already specified for PC3.

The current MSD framework which is to specify MSD for all UE power classes, and even for PC2 FDD accounting 1TX and 2TX separately, has become extremely heavy to maintain because the “dimensions” extend all the time. There are lots of inconsistencies and even errors in the specifications, unfortunately correcting them concurrently when specifying new HPUE combinations is very challenging from both technical aspects (doing the analysis) and from specification management aspects (making TP’s/draft CR’s and especially capturing everything in BigCR). At least we think that the problems are getting worse even companies and delegates are working hard to resolve issues and to specify new HPUE combinations.

During the past meetings more and more companies have started thinking whether the current framework is still best serving the interests of everyone in 3GPP, or whether it should be modified. This is basically the reason behind the WF [1] agreed in RAN4#112.

It is obvious the MSD framework would be tons lighter if only PC3 MSD is specified. New combinations requiring technical analysis would be rare, as even looking simple combinatory logic very vast majority of combinations are already specified.

It would be a big change to band combination work, so careful assessment together with all stakeholders is needed before conclusions.

We have been looking into this internally, and that work is still ongoing. It is obvious that PC3 MSD only would be much easier for 3GPP, but on the other hand it is not obvious if it simplifies or makes thing more complex for the cellular community. For instance, operators might still want to understand the impact to MSD due to UE power increase and so on. If/when so, the someone obviously must provide this information into operators. If/when that information comes from several vendors there is likely a lot of discrepancy in the numbers making thing more complicated to manage and so on.

In any case, the current MSD framework in our view is not fully sustainable because creating and maintaining error-free and consistent specifications is currently not happening at least in broad scale. For instance, there are probably several hundreds of PC2 FDD combinations coming in next meetings. Due to a limited number of companies/delegates doing purely technical work, not all of these can be handled in detail. Then the outcome, unfortunately, is that part of the combinations get specified without proper technical analysis, and some combinations which do get analysed may have quite a bit different requirements.

Observation 1: The current MSD framework is not fully sustainable. 3GPP should have error free and consistent specifications which currently is not happening at least in broad scale.

Observation 2: It is obvious that PC3 MSD only would be much easier for 3GPP, but it is not obvious if it simplifies or makes thing more complex for the cellular community. In general, having MSD numbers for all Power Classes in the specification is seen a good thing as it provides at least baseline information for all.

Observation 3: It would be a very big change to specify only PC3 MSD, so careful assessment together with all stakeholders is needed before conclusions.

If the conclusion is to specify only PC3 MSD, then it would be good to remove all PC2/PC1.5 MSD tables from the specification. This is because otherwise it would not be fair for the combinations not in the specification that PC2/PC1.5 MSD were specific for some combinations but not all. Also, if PC2/PC1.5 MSD is no longer verified, the respective specifications in RAN4 would be redundant. We recognize this is differing from Option1 in the sense that in Option1 already specified HPUE MSD’s were proposed to remain in TS, but after more thorough thinking that is not good for consistency.

Observation 4: If only PC3 MSD is specified, then existing PC2/PC1.5 MSD tables should be removed from 38.101-1 and from 38.101-3 to maintain consistency between new and old combinations.

If PC2 and PC1.5 MSD are continued to be specified, RAN4 should investigate how to simplify specification process. For instance, specifying only single set of requirements for FDD PC2 CA instead of 1TX and 2TX would help a lot. Even greater steps could be achieved if some kind of MSD derivation equations could be generated to extract HPUE MSD from PC3 MSD. Those are never ideal but could serve the purpose at least better than the current approach is.

Observation 5: There are potential ways to simplify PC2/PC1.5 MSD specification process, which could significantly help to improve the quality of the specifications.

Observation 6: Going to “PC3 MSD only” seems overly aggressive. Instead, 3GPP should consider ways how to simplify current PC2/PC1.5 MSD definition process.

Proposal 1: Instead of going directly into “PC3 MSD only” approach, companies should first consider if some simplifications e.g. by specifying just single set of requirements for HPUE FDD band combinations and/or by using some equations to derive PC2/PC1.5 MSD from respective PC3 MSD would help to bring the MSD specification framework back on track.

- Conclusion

HPUE MSD, specifically if current framework should be revisited was considered in this contribution. The following observations were done.

Observation 1: The current MSD framework is not fully sustainable. 3GPP should have error free and consistent specifications which currently is not happening at least in broad scale.

Observation 2: It is obvious that PC3 MSD only would be much easier for 3GPP, but it is not obvious if it simplifies or makes thing more complex for the cellular community. In general, having MSD numbers for all Power Classes in the specification is seen a good thing as it provides at least baseline information for all.

Observation 3: It would be a very big change to specify only PC3 MSD, so careful assessment together with all stakeholders is needed before conclusions.

Observation 4: If only PC3 MSD is specified, then existing PC2/PC1.5 MSD tables should be removed from 38.101-1 and from 38.101-3 to maintain consistency between new and old combinations.

Observation 5: There are potential ways to simplify PC2/PC1.5 MSD specification process, which could significantly help to improve the quality of the specifications.

Observation 6: Going to “PC3 MSD only” seems overly aggressive. Instead, 3GPP should consider ways how to simplify current PC2/PC1.5 MSD definition process.

Proposal 1: Instead of going directly into “PC3 MSD only” approach, companies should first consider if some simplifications e.g. by specifying just single set of requirements for HPUE FDD band combinations and/or by using some equations to derive PC2/PC1.5 MSD from respective PC3 MSD would help to bring the MSD specification framework back on track.

- Reference

[1] R4-2410605, “WF on HPUE MSD simplification”, Skyworks, Murata, Qualcomm, Samsung, Nokia