3GPP TSG-RAN WG4#112	R4-2412620

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda item:	5.2

Source: 	Qualcomm France

Title: 	About RX mixing clean-up

Document for:	Approval

- Introduction

Considerations on remaining issues for Harmonic mixing clean-up are provided.

- WF in RAN4#111

WF [1]:

<Way forward>

- RAN4 CR cover page to clarify that the changes to UL harmonic and Rx harmonic MSD tables are applicable only from Rel-18 and onward to subsequent Releases.

<Way forward on “RBstart=0”>

To ensure the harmonic centre frequency meets the direct hit collision with the DL carrier centre frequency, companies are invited to share their views on the following options at the next meeting. In all cases, the “Lcrb” requirements are kept, only “RBstart” may be changed.

- Option 1: RAN4 corrects all RBstart=0 values to ensure that the non-fully allocated RBs are centered in the middle of the UL channel. This means correcting several NR-CA, and EN-DC test points,

- Option 2: RAN4

  - Removes RBstart=0 from all UL Harmonic and Rx Harmonic mixing MSD tables, and

  - Adds an equation in the core requirement text that guarantees the UL RBs are centered in the middle of the UL channel,

- Option 3: RAN4

  - Removes RBstart=0 from all UL harmonic and Rx harmonic mixing MSD tables, and

  - Adds a sentence in the core requirement text that RAN4 MSD levels assume the UL RBs are centered in the middle of the UL channel,

  - Informs RAN5 that RBstart are no longer specified and that RAN5 needs to specify RBstart in the test requirement technical specifications.

- Other options are not precluded.

<Way forward on UL “Lcrb”>

- For UL Harmonic and Rx harmonic mixing MSD test points, RAN4 recommends that UL Lcrb are corrected as follows for SCS 15kHz / SCS 30kHz respectively:

  - For UL2/DLx test point, Lcrb=12RB / 6RB

  - For UL3/DLx test point, Lcrb=8RB / 4RB

  - For UL4/DLx test point, Lcrb=6RB / 3RB

  - For UL5/DLx test point, Lcrb=5RB / 2RB

- Additionally, for UL Rx harmonic mixing UL1/DLx, Lcrb=25RB / 12RB.

- Discussion

“RBstart=0”

There are no issues when the aggressor UL is fully loaded with RB’s, i.e. in this case the UL RB allocation is centered at UL Fc, and hence victim DL is naturally positioned at the middle of the aggressor UL range.

The issue is how to treat cases when the aggressor UL is not fully loaded, because of narrower victim DL or because of harmonic of UL aggressor hitting victim DL.

For instance, when 10MHz UL aggressor (15kHz SCS) has “UL1/DL3” harmonic mixing with 5MHz victim DL, the UL RB allocation is set to 25RB@RBstart=0. For example, let’s assume UL Fc is at 2550MHz. According to current Notes, the victim DL frequency is at 850MHz. Taking a closer look, UL RB allocation is confined within 2545.3125….2549.8125MHz so basically the center of UL RB allocation is at 2547.5625MHz. Using same note, the ideal victim DL frequency should be 849.1MHz instead of 850MHz.

There are cases with more issues, e.g. UL4/DL3 case. Assume 10MHz UL aggressor and 10MHz DL victim. UL RB (15Khz SCS) is set to 12RB@RBstart=0. UL Fc=2550MHz. According to current Notes, the victim DL frequency is at 3400MHz. Taking a closer look, UL RB allocation is confined within 2545.3125….2547.4725MHz so basically the center of UL RB allocation is at 2546.3925MHz. Using same note, the ideal victim DL frequency should be 3395MHz instead of 3400MHz.

“The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that  in MHz and  FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2  with  the carrier frequency in the victim (lower) band and  the channel bandwidth configured in the higher band”

“The requirements should be verified for DL NR-ARFCN of the victim (higher) band (superscript HB) such that fDLHB=fULLB/0.75  and FUL_lowLB+BWChannelLB/2≤fULLB≤FUL_highLB-BWChannelLB/2 with fULLB the UL carrier frequency and BWChannelLB the channel bandwidth configured in the lower band, both in MHz.”

RAN4 must have Core specifications, so in our view Option 3 is not desired. In Option 3, RAN5 would need to continue the work done by RAN4 by specifying the RB starts. In option 2, the spirit of specifying all Core requirements would be met, however still RAN5 would need to specify the exact UL RBstart positions

Option 1 is our preference. With that option, RAN4 would include complete core specification, and RAN5 would not need to work further with UL RBstarts, which would be needed with Option 2 or especially with Option 3. In our view, the work needed for changes is not prohibitively large.

Proposal 1: Use option 1 for “RBstart=0”

The UL RBstart position should be set such that the RB allocation is as close to UL Fc as possible. There are three possible UL BW’s for harmonics and harmonic mixing cases; 5MHz, 10MHz, and 20MHz. 5MHz and 10MHz are for licensed bands, and 20MHz is for unlicensed bands. The RB starts and Lcrb’s should be defined as follows.

-LCRB_UL=Floor(NRB_DL/UL harmonic order)

-RBstart=Floor((NRB_UL-LCRB_UL)/2)

Proposal 2: Use the following rules to specify UL RB allocation and RB start for UL harmonic and RX mixing cases.

- LCRB_UL=Floor(NRB_DL/UL harmonic order)

- RBstart=Floor((NRB_UL-LCRB_UL)/2)

To have specification as clean as possible and to ease the work of companies making TP’s for new combinations, we propose continuing to explicitly specify UL RB allocation in UL harmonic and RX mixing tables, and to have a note for RBstart.

Proposal 3: UL RB allocation is listed in MSD table, and RBstart is described in a note in equation format.

UL “Lcrb”

We have used the approach outlined in our Draft CR’s submitted in RAN4#111, so for this meeting we didn’t do changes into our CR’s as they were already aligned with WF.

- Conclusion

# WF on Harmonic MSD clean-up was considered with the following proposals. Our CR’s x,y are modified accordingly.

# Proposal 1: Use option 2 for “RBstart=0”

Proposal 2: Use the following rules to specify UL RB allocation and RB start for UL harmonic and RX mixing cases.

- LCRB_UL=Floor(NRB_DL/UL harmonic order)

- RBstart=Floor((NRB_UL-LCRB_UL)/2)

Proposal 3: UL RB allocation is listed in MSD table, and RBstart is described in a note in equation format.

- Reference

[1] R4-2410651, “WF on Harmonic MSD clean-up”, Skyworks, Qualcomm, Murata