3GPP TSG-RAN WG4 Meeting #112	R4-2412628

Maastricht, The Netherlands, 19 – 23 August, 2024

Title: 	(NR_MG_enh-Core) Discussion on remaining issues in Rel-17 MGE

Source: 	Huawei, HiSilicon

Agenda item:	4.5

Document for:	Discussion

Introduction

RRM core requirement maintenance for R17 MGE are discussed in RAN4#111. One of the issues discussed is the VIL of NCSG patterns, and one WF [1] is agreed which captures the issue and proposed options.

In this paper, we will provide our views on the VIL of NCSG patterns.

Discussion

In our view, the issue in [1]is related to the following two tables which define the VIL of NCSG in number of interrupted slots. From the titles of the tables, it can be seen that the VIL depends on whether the NCSG is per-UE, per-FR for FR1 or per-FR for FR2.

Table 9.1.9-1: Number of interrupted slots on all serving cells for per-UE NCSG or FR1 serving cells for FR1 NCSG during each VIL in NR standalone operation (with single carrier, NR CA)

| NR | Number of interrupted slots on serving cells |
| --- | --- |
| SCS | When MG timing advance of 0ms is applied |
| (kHz) | VIL=1ms |
| 15 | 1 |
| 30 | 2 |
| 60 | 4 |
| 120 | 8 |
| NOTE 1: NR SCS of 120 kHz is only applicable to the case with per-UE NCSG. NOTE 2: Void |  |

Table 9.1.9-2: Number of interrupted slots on FR2 serving cells for FR2 NCSG during each VIL in NR standalone operation (with single carrier, NR CA)

| NR | Number of interrupted slots on serving cells |  |
| --- | --- | --- |
| SCS | When MG timing advance of 0ms is applied | When MG timing advance of 0.75ms is applied |
| (kHz) | VIL=0.75ms | VIL=0.75ms |
| 60 | 3 | 3 |
| 120 | 6 | 6 |
| NOTE 1: Void |  |  |

On the other hand, in Table 9.1.9.3-2, there is one special case for per-UE NCSG, which both serving cells and MOs are in FR2.

Table 9.1.9.3-2: Applicability for NCSG pattern configurations supported by the UE for NR standalone operation with single carrier or NR CA configuration

| NCSG pattern configuration | Serving cell | Measurement Purpose NOTE 2 | Applicable NCSG Pattern Id |
| --- | --- | --- | --- |
|  | FR1, or FR1 + FR2 | E-UTRA | 0,1,2,3 |
|  |  | FR1 and/or FR2 | 0-11 |
|  |  | E-UTRA and FR1 and/or FR2 | 0, 1, 2, 3, 4, 6, 7, 8,10 |
| Per-UE NCSG | FR2 | E-UTRA only | 0,1,2,3 |
|  |  | FR1 only | 0-11 |
|  |  | FR1 and FR2 | 0-11 |
|  |  | E-UTRA and FR1 and/or FR2 | 0, 1, 2, 3, 4, 6, 7, 8,10 |
|  |  | FR2 only | 12-23 |

In our view, for this particular case, the VIL = 0.75ms should apply. This is based on the RAN4#100 agreement as copied below. Technically, the applicable VIL should depend on where the NCSG is used (the impacted serving cells and the MOs) but not the configured NCSG type (per-UE or per-FR).

## Issue 3-4: length of VIL - Agreements in the 1st round: - Translate [1ms] (FR1) and [0.75ms] (FR2) into the number of interrupted slots for defining the interruption requirements for the synchronous case and one more slot is added for asynchronous case.

Therefore, we support option 2 in [1] to update the applicability of VIL such that it depends on the NCSG pattern. Otherwise, if we use 1ms VIL for the highlighted case, it means the VIL for NCSG pattern 12-23 would be 1ms, and it will make NCSG pattern 12-23 exactly same as NCSG pattern 0-11.

Proposal: Update the applicability of VIL such that it depends on the NCSG pattern rather than NCSG type (per-UE or per-FR).

Conclusions

In this paper we provided our views on the VIL of NCSG patterns.

Proposal: Update the applicability of VIL such that it depends on the NCSG pattern rather than NCSG type (per-UE or per-FR).

Reference

- R4-2410326, WF on clarification of R17 NCSG pattern, OPPO