3GPP TSG-RAN WG4 Meeting #112	R4-2412634

Maastricht, The Netherlands, 19 – 23 August, 2024

Title: 	On remaining issues in core requirements for R18 MGE

Source: 	Huawei, HiSilicon

Agenda item:	5.15.1

Document for:	Approval

Introduction

Maintenance of RRM core requirements for R18 MG enhancement are discussed in RAN4#111, and the outcomes are captured in [1-2]. Further discussions are needed for the following issues.

- Joint operation of con-MG with pre-MG and NCSG

  - Case 1: joint operation of con-MG with pre-MG

  - Case 2: joint operation of con-MG with NCSG

- Measurement without MG

  - NFG

  - Inter-RAT measurement

  - UE capability

In this paper, we will provide our views on remaining issues in core requirements for R18 MGE.

Discussion

Joint operation of con-MG with pre-MG and NCSG

Case 1: joint operation of con-MG with pre-MG

Issue 2-1-7: [Case 1] - [New issue – Dynamic collision] What is the UE behaviour when the UE doesn’t support dynamic collision FG? Agreement from online session: For UE not supporting dynamic collision, the MG will be drop if overlapped with Pre-MG, regardless whether Pre-MG (with higher priority) is activated or deactivated, including the case when the MG overlaps with the Pre-MG activation/deactivation procedure.

UE behaviour when not supporting dynamic collision is agreed last meeting, but how to define requirements is still open. In our view, such UEs would handle the collision between MGs in the same way as R17 con-MG, ignoring the status of pre-MG(s). We do not see any additional requirement needed. In case of collision between pre-MG (de)activation procedure and MG, the pre-MG should be (de)activated following the normal time line (same as the case when there is no collision between pre-MG (de)activation procedure and MG), and the MG is always dropped following the collision handling rule for R17 con-MG.

Proposal 1: For UE not supporting dynamic collision,

- Clarify that the requirements for collision handling are same as R17 con-MG

- Do not define any requirement for collision between pre-MG (de)activation procedure and MG

Case 2: joint operation of con-MG with NCSG

Issue 3-2-1: [Case 2] When the UE is configured with Concurrent gaps with NCSG, what is the potential changes to UE behaviour for NCSG upon SCell activation (in Rel-18) Agreement from online session: - For UE configured with one NCSG and one Type 1/2 MG: All deactivated SCells are measured within NCSG, regardless of the reported UE capabilities [and gap association]. - Further details on the processing delay between NCSG and Type 1/2 MG can be further discussed. - For UE configured with 2 NCSG, deactivated SCells are measured with NCSG - If the association is provided, deactivated SCells are measured with NCSG according to gap association. - If the association is not provided, UE is not expected to cause interruption outside the VIL due to measurement on any of the deactivated SCells, and the existing measurement delay requirement does not apply to this case.

Basic UE requirements for deactivated SCell measurement in Case 2 are agreed last meeting. We support to remove the [] around “and gap association”. In our view, it could happen that the SCell MO is associated to a MG because when the SCell is activated the active BWP may not cover SSB. In this case, the SCell MO should also be measured within NCSG to avoid interruption outside NCSG, and it is in fact an important use case. We also do not see any technical issue because no matter UE measures the SCell MO outside MG or within NCSG, the gap association will be ignored when SCell is deactivated.

We also do not see the need to introduce extra processing delay for switching between NCSG and Type 1/2 MG. In R15, the switching between measurement outside MG and within MG could already happen following a BWP switching, and it is no different from switching between measurement outside MG and within NCSG (when SCell MO is measured outside MG when SCell is activated), or between measurement within MG and within NCSG (when SCell MO is measured within MG when SCell is activated).

Proposal 2: For UE configured with one NCSG and one Type 1/2 MG,

- All deactivated SCells are measured within NCSG, regardless of the reported UE capabilities and gap association, i.e. remove the [] in the agreement from last meeting

- No extra processing delay for switching between NCSG and Type 1/2 MG is needed

Measurement without MG

NFG

Issue 1-1-1: Misalignment between DRX-on duration and SMTC for NFG measurements - Proposals - Option 1: - Option 1a: Interruptions are always allowed outside DRX ON duration and it is according to Tcycle,i. - Option 1b: Interruptions are not allowed during DRX ON duration. - Option 1c: Interruptions are not allowed when DRX cycle is larger than 320ms. - Option 1d: Interruptions are not allowed in the DRX ON duration, excluding the time extended due to drx-inactivityTimer. - Option 2: Interruptions are allowed under the conditions of issues 1-1-2, 1-1-3, 1-1-4.

In general, it is reasonable to assume no interruption during DRX ON duration when there is no SMTC occasion within the ON duration. However, in practice UE may not perform RF retuning immediately before and after the SMTC occasion. Defining requirements based on such an assumption is also against the principle not to define interruption location. To allow some UE implementation flexibility, we suggest to allow UE RF retuning to happen in the time period from [4ms] before to [4ms] after the SMTC occasion.

Proposal 3: Interruption is not allowed during DRX ON duration, if there is no SMTC occasion within a time period starting [4ms] before the starting point of the DRX ON duration and ending [4ms] after the ending point of the DRX ON duration.

Issue 1-2-1: Interruption requirements in 8.2.2.2.19 apply also for NR-DC, EN-DC, and NE-DC - Background: - the NFG signalling is used in NR SA only, as shown below: From 38.331: #### – NeedForGapsInfoNR The IE NeedForGapsInfoNR indicates whether measurement gap is required for the UE to perform SSB based measurements on an NR target band while NR-DC or NE-DC is not configured.. - Proposals - Option 1: Yes. Interruption requirements in 8.2.2.2.19 shall also apply for NR-DC, EN-DC, and NE-DC considering that operations in one cell group do not impact operations on another cell group. - Option 1a: Same interruption requirement defined at 7.8.2.22 in 36.133 and 8.2.2.2.19 in 38.133 are applied for each cell group for UE perform NR measurement without gap with interruption. - 7.8.2.22 in 36.133 and 8.2.2.2.19 in 38.133 is applied for MCG and SCG, respectively in EN-DC, - 7.8.2.22 in 36.133 and 8.2.2.2.19 in 38.133 is applied for SCG and MCG, respectively in NE-DC, - 8.2.2.2.19 in 38.133 is applied for both MCG and SCG in NR-DC. - Option 2: NFG requirements are applicable for NR SA only. - Option 2a: deprioritize MR_DC for NFG in objective 2 of the WI.

We understand the signaling for NFG, both R16 and R18 signaling, is only defined for NR SA. Therefore, the requirements should also only apply for NR SA. Also, unlike option 1, we understand the measurement in one CG may cause interruption to another CG since the interruption has been defined per FR.

Proposal 4: NFG requirements are applicable for NR SA only.

Issue 1-3-2: NFG and NCSG capabilities - Previous Agreements - No need to establish the mapping between UE’s indication for NeedForGaps and NCSG. - Proposals - Option 1: NeedForGaps and NCSG are not expected to be enabled for the same UE at the same time. - Option 2: Allow to enable both R17 and R18 reporting.

As discussed in our earlier paper, NeedForGaps reporting and NeedforGapNCSG reporting are separate features with separate NW flags and separate UE capabilities. We understand they are not expected to be enabled for the same UE at the same time.

- If UE only supports one of them, NW can only configure UE to report with the supported signaling

- If UE supports both of them, it is up to NW to configure which signaling to use. If both are configured, there could be confusion in the UE behavior when UE reports ‘no-gap’ with NFG reporting and ‘ncsg’ with NFG reporting.

  - If UE reports ‘no-gap’ with NFG reporting, UE would expect no MG to be configured, and UE is required to meet the requirements either with or without interruption.

  - If UE reports ‘ncsg’ with NCSG reporting, UE would expect NCSG to be configured, otherwise UE is not required to meet any requirement.

In last meeting, some companies proposed to allow NW to enable both NFG and NCSG reporting for the same UE at same time, and whether UE should perform the measurement outside MG or within NCSG depends on whether NW configures NCSG. In our view, this will add complexity to both the spec and UE implementation, for a corner case and without clear benefit.

Proposal 5: NeedForGaps and NCSG are not expected to be enabled for the same UE at the same time.

A draft LS is provided in the Annex to ask RAN2 to make this clarification.

Inter-RAT measurement

Issue 2-1-1: Scheduling restriction for case b-1 - Proposals - Option 1: No scheduling restriction is applied for UE indicate nogap-noncsg for inter-RAT EUTRAN measurement without gap. Issue 2-1-2: Scheduling restrictions and UE capability reporting - Proposals - Proposal 1a: For Cases b-1 and/or b-2: When the UE require NO scheduling restriction for a specific carrier: - the UE shall report to the NW that the UE can measure a given/specific carrier frequency of inter-RAT EUTRAN without gap and without interruption; - the UE shall not report EMW (i.e. the UE can measure with at any reference signal occasion). - Proposal 1b: For Cases b-1 and/or b-2: When the UE require scheduling restriction for a specific carrier: - the UE reports to the NW that the UE can measure a given/specific carrier frequency of inter-RAT EUTRAN without gap and without interruption, - also, the UE shall report the support of effective measurement window (EMW) for inter-RAT EUTRAN capability (i.e. the UE can measure and cause scheduling restrictions only within the EMW occasions). - Proposal 1c: For Cases b-1 and/or b-2: When the UE reports for inter-RAT EUTRAN measurements without gap and without interruption and the UE doesn’t support EMW capability, the UE shall report NCSG, or gaps.

Issue 2-1-1 is already discussed last meeting for UE capability 32-4. The common understanding was that some UE implementation may use same baseband for NR serving cell and LTE measurement when the LTE carrier is overlapped with NR active BWP. As a result, 32-4 is kept as a pre-requisite for 32-7. We do not see the need to further discuss option 1.

Issue 2-1-2 is already addressed by the feature dependency between Case b-1/2 (32-4 and 32-5) and EMW (32-6), which states that “If UE supports 32-4 or 32-5 and UE requires scheduling restriction, UE should support this FG (32-6)”. We do not see the need to further discuss proposal 1a/b/c.

Proposal 6: Do not further discuss scheduling restriction due to mixed numerology for Case b-1/2.

Issue 2-2-1: Overlap between Effective measurement window and SMTC/SSB - Background - Previous Agreements - For case b-2, when EMW is configured overlapped with SMTC/SSB/CSI-RS measurement with scheduling restrictions, inter-RAT LTE measurement will be dropped. - For case b-1 and b-2, when EMW is partially overlapped with MG (EMW periodicity < MGRP), the EMW occasion colliding physically with MG will be dropped. - Proposals to: For case b-1 and b-2 inter-RAT LTE measurement causing scheduling restriction, when EMW periodicity is smaller than MGRP, - Option 1: RAN4 to update the legacy agreements as: after considering EMW dropping rule if EMW is colliding with SMTC/SSB/CSI-RS, when the remaining EMW is fully overlapping with MG, the inter-RAT meas will be performed within MG. - Option 2: RAN4 to update the legacy agreements as: after considering EMW dropping rule if EMW is colliding with SMTC/SSB/CSI-RS, when the remaining EMW is fully overlapping with MG, the inter-RAT measurements will be left to UE implementation (i.e. no requirements defined for this corner case).

We support option 1 for issue 2-2-1. Without the proposed clarification, the requirements may be understood as UE is not required to perform the inter-RAT LTE measurement when EMW collides with union of SMTC/SSB/CSI-RS and MG. One further clarification is our view is that for collision with SMTC/SSB/CSI-RS, we should only consider EMW outside MG.

Proposal 7: RAN4 to update the requirements for Case b-1 and b-2:

after considering EMW dropping rule if EMW outside MG is colliding with SMTC/SSB/CSI-RS, when the remaining EMW is fully overlapping with MG, the inter-RAT measurement will be performed within MG.

Issue 2-2-1a: Overlap between Effective measurement window and MG - Background - Agreements - For case b-1 and b-2, when EMW is partially overlapped with MG (EMW periodicity < MGRP), the EMW occasion colliding physically with MG will be dropped. - Note: The proximity rule in Rel-17 does not apply in this case. - Proposals to: For case b-1 and b-2 inter-RAT LTE measurement causing scheduling restriction, when EMW periodicity is larger than MGRP and all EMW are covered by measurement gaps, - Option 1: inter-RAT LTE measurement will be dropped. - Option 2: No UE behaviour is specified. - Option 3: apply legacy gap-based measurement requirements, i.e. RAN4 requirements should NOT be defined based on EMW. - Option 4: UE measurement requirements are based on EMW-RP.

For the case where EMW is configured and fully overlapped with MG, but the periodicity of MG is smaller than EMW, the inter-RAT LTE measurement causing scheduling restriction will be performed within MG, and the question is whether the measurement period is based on MGRP or the EMW-RP.

We support to define measurement period based on EMW-RP. The reason is that if NW wants UE to follow MGRP, it can simply does not configure EMW. When EMW is configured, it is more reasonable for UE to follow the EMW for inter-RAT measurement. In this sense, EMW can be regarded as SMTC for LTE.

Proposal 8: For Case b-1 and b-2 inter-RAT LTE measurement causing scheduling restriction, if EMW is fully overlapping with MG and EMW periodicity larger than MGRP, UE measurement requirements are based on EMW-RP.

Issue 2-2-3: Scaling factor for case a-1: Nfreq definition - Background - The principles are different between NR MO outside gap and LTE inter-frequency without MG, where all inter-frequency MOs, regardless if they are measured with or without MG, are counted in the same Nfreq. - Proposals - Option 1: number of NR MOs that are measured outside MG (same principle as NR SA). - Option 2: total number of LTE and NR MOs (same principle as LTE SA). - Option 2a: total number of inter-frequency LTE and NR MOs (same principle as LTE SA).

In last meeting, some companies commented that the scaling factor for Case a-1 has not been agreed. In the endorsed draftCR, Nfreq accounts for all NR MOs that are measured outside MG. It follows the principle of NR SA where MOs measured outside and within MG are counted separately.

Specifically, NR MOs that are measured outside MG are listed below.

- When MG is configured, Nfreq equals to the total number NR MOa that satisfy one the following conditions.

  - UE indicates [nogap-interruption] for the band which the NR MO belongs to, and none of the SMTCs of the NRMO is overlapped by the MG,

  - UE indicates [nogap-nointerruption] for the band which the NRMO belongs to, and part or none of the SMTCs of the NR MO are overlapped by the MG.

- When MG is not configured, Nfreq equals to the total number NR MOs that satisfy one the following conditions.

  - UE indicates [nogap-interruption] for the band which the NR MO belongs to,

  - UE indicates [nogap-nointerruption] for the band which the NR MO belongs to.

It is noted that the principle is different from LTE inter-frequency without MG, where all inter-frequency MOs, regardless if they are measured with or without MG, are counted in the same Nfreq. We suggest RAN4 to discuss which principle to go with.

Proposal 9: For Case a-1, RAN4 to discuss the calculation of Nfreq

- Option 1: number of NR MOs that are measured outside MG (same principle as NR SA)

- Option 2: total number of LTE and NR MOs (same principle as LTE SA)

UE capability

Issue 1-4-1: Relations between nr-NeedForGap-Reporting-r16 and nr-NeedForInterruptionReport-r18 and UE behaviours - Previous agreements Issue 1-1-2: Scenario 2, NR measurements without gaps Tentative agreements - “no-gap” as part of NeedForGapsInfoNR-r16=FALSE means that the UE support measurement without gaps - The UE may or may not cause interruption. - if UE causes interruptions when performing measurements without gaps: - Support early implementation of Rel-18 NeedForInterruption: - Optional since R17 - FFS the UE behavior if the Rel-18 UE does not support NeedForInterruptionNR-r18. - Proposals - Option 1: A Rel-18 UE indicating support of nr-NeedForGap-Reporting-r16 shall also indicate support of nr-NeedForInterruptionReport-r18. - Option 2: When a Rel-18 UE only supports Rel-16 NFG capability but not supports Rel-18 NFI capability, the UE’s behaviour is the same as Rel-16 UE. - Option 2a: When a Rel-18 UE supports both Rel-16 NFG and Rel-18 NFI capabilities, but NW doesn’t configure Rel-18 needForInterruptionConfigNR, the UE’s behaviour is the same as Rel-16 UE. - Option 3: A Rel-18 UE indicating support of interRAT-NeedForGapsNR-r16 shall also indicate if interruptions are needed. - No new RRM test cases are defined for a Rel-18 UE supporting signalling above.

Issue 2-3-1a: Relations between interRAT-NeedForGaps-r16 and interRAT-NeedForIntrNR-r18 and UE behaviours - Previous agreements Issue 1-1-1: Scenario 1, LTE – NR inter-RAT measurement Tentative agreements - interRAT-NeedForGaps-r16=FALSE means that the UE support measurement without gaps - The UE may or may not cause interruption. - if UE causes interruptions when performing measurements without gaps: - Support early implementation of Rel-18 NeedForInterruption: - Optional since R17 - FFS mandatory report of the UE capability R18 interRAT-NeedForIntrNR-r18 from Rel-18 UE if the UE reports interRAT-NeedForGaps-r16=FALSE - FFS the UE behavior if the Rel-18 UE does not support interRAT-NeedForIntrNR-r18. - Proposals - Option 1: A Rel-18 UE indicating support of interRAT-NeedForGapsNR-r16 shall also indicate support of interRAT-NeedForInterruptionNR-r18. - Option 2: When a Rel-18 UE only supports Rel-16 NFG capability but not supports Rel-18 NFI capability, the UE’s behaviour is the same as Rel-16 UE. - Option 2a: When a Rel-18 UE supports both Rel-16 NFG and Rel-18 NFI capabilities, but NW doesn’t configure Rel-18 needForInterruptionConfigNR, the UE’s behaviour is the same as Rel-16 UE. - Option 3: A Rel-18 UE indicating support of interRAT-NeedForGapsNR-r16 shall also indicate if interruptions are needed. - No new RRM test cases are defined for a Rel-18 UE supporting signalling above.

The NFG requirements for R16 UE are discussed last meeting as R16 maintenance. Some remaining issue from [2] are related to requirements for R18 UE, namely whether R18 UE should mandatorily support R18 NFG if it reports ‘no-gap’ for R16 NFG, and what is the UE behavior if the R18 UE does support R18 NFG.

Based on discussion last meeting, R18 NFG refers to

- Reporting of interRAT-NeedForInterruptionNR and meeting the R18 requirements in 36.133, for LTE – NR inter-RAT measurement

- Reporting of nr-NeedForInterruptionReport-r18 and meeting the R18 requirements in 38.133, for NR intra-and inter-frequency measurements

We do not support to make it mandatory for R18 UE to support R18 NFG when it indicates ‘no-gap’ via R16 NFG signaling. In our view, supporting a specific feature (NFG) in a new release (R18) should be always optional. Making it mandatory means a R18 UE has to support R18 NFG if it indicates support of R16 NFG. Having such a binding among R16 and R18 features may exclude the possibility for UE vendors to use same implementation for NFG in R16/17 and R18, and a possible outcome is that a R18 UE just does not indicate support of R16 NFG. This is clearly not desirable for either NW or UE, as usage of the feature is limited.

Therefore, we suggest it is optional for R18 UE to support R18 NFG when it indicates ‘no-gap’ via R16 NFG signalling. When a R18 UE supports R16 NFG but does not support R18 NFG, we understand the R18 NFG requirements do not apply for this UE. The UE behaviour can be same as a R16 UE supporting R16 NFG, and NW can treat this UE in the same way as it treats a R16 UE supporting R16 NFG.

Proposal 10: It is optional for R18 UE to support R18 NFG when it indicates ‘no-gap’ via R16 NFG signaling. R18 NFG requirements do not apply for R18 UE that does not support R18 NFG.

Conclusions

In this paper we provided our views on remaining issues in core requirements for R18 MGE.

Proposal 1: For UE not supporting dynamic collision,

- Clarify that the requirements for collision handling are same as R17 con-MG

- Do not define any requirement for collision between pre-MG (de)activation procedure and MG

Proposal 2: For UE configured with one NCSG and one Type 1/2 MG,

- All deactivated SCells are measured within NCSG, regardless of the reported UE capabilities and gap association, i.e. remove the [] in the agreement from last meeting

- No extra processing delay for switching between NCSG and Type 1/2 MG is needed

Proposal 3: Interruption is not allowed during DRX ON duration, if there is no SMTC occasion within a time period starting [4ms] before the starting point of the DRX ON duration and ending [4ms] after the ending point of the DRX ON duration.

Proposal 4: NFG requirements are applicable for NR SA only.

Proposal 5: NeedForGaps and NCSG are not expected to be enabled for the same UE at the same time.

Proposal 6: Do not further discuss scheduling restriction due to mixed numerology for Case b-1/2.

Proposal 7: RAN4 to update the requirements for Case b-1 and b-2:

after considering EMW dropping rule if EMW outside MG is colliding with SMTC/SSB/CSI-RS, when the remaining EMW is fully overlapping with MG, the inter-RAT measurement will be performed within MG.

Proposal 8: For Case b-1 and b-2 inter-RAT LTE measurement causing scheduling restriction, if EMW is fully overlapping with MG and EMW periodicity larger than MGRP, UE measurement requirements are based on EMW-RP.

Proposal 9: For Case a-1, RAN4 to discuss the calculation of Nfreq

- Option 1: number of NR MOs that are measured outside MG (same principle as NR SA)

- Option 2: total number of LTE and NR MOs (same principle as LTE SA)

Proposal 10: It is optional for R18 UE to support R18 NFG when it indicates ‘no-gap’ via R16 NFG signaling. R18 NFG requirements do not apply for R18 UE that does not support R18 NFG.

Reference

- R4-2410376, WF on NR_MG_enh2_part1, MediaTek inc.

- R4-2410355, WF on NR_MG_enh2_part2, Intel Corporation

Annex: draft LS on configuration of needForGapsConfigNR and needForGapNCSG-ConfigNR

3GPP TSG-RAN WG4 Meeting #110 R4-240xxxx Athens, Greece, 26 February – 1 March, 2024 Title: LS on configuration of needForGapsConfigNR and needForGapNCSG-ConfigNR Response to: Release: Release 18 Work Item: NR_MG_enh2-Core Source: RAN4 To: RAN2 Cc: Contact Person: #### Name: Li Zhang - E-mail Address: zhangli164@huawei.com Attachments: - 1. Overall Description: RAN4 has discussed the UE behaviour when both needForGapsConfigNR and needForGapNCSG-ConfigNR are configured to the same UE at the same time. RAN4 concludes that needForGapsConfigNR and needForGapNCSG-ConfigNR are not expected to be enabled for the same UE at the same time. RAN4 believes it is better to capture the restriction in RAN2 spec, and from Rel-17. RAN4 respectfully asks RAN2 to take the above information into account and capture the restriction in RAN2 spec. 2. Actions: To RAN2: RAN4 respectfully asks RAN2 to take the above information into account and capture the restriction in RAN2 spec. 3. Date of Next TSG-RAN4 Meetings: RAN WG4 Meeting #111 May 20 – May 24, 2024 Fukuoka, Japan