3GPP TSG-RAN4 Meeting #112	R4-2412635

# Maastricht, The Netherlands, 19 – 23 August, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 4851 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on RRM requirements for con-MG + pre-MG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are some issues in pre-MG collision handling requirements: - The requirements for UE not supporting dynamic collision are TBD. - In 9.1.12.4, the requirements for Scenario 3 are not needed because the collision between the MG and pre-MG (de)activation procedure does not impact UE behaviour. - UE scheduling requirements has nothing to do with collision between Pre-MG (de)activation procedure and MG, so they should be defined in a new clause. Also, slots in the dropped MG occasion may not be used for scheudling because they may be subject to other interruptions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Define requirements for UE not supporting dynamic collision: - Remove the definition of dynamic collision scenario since it is not used anywhere in the requirements. - Clarify that for UE not supporting dynamic collision, the collision condition is same as in R17 con-MG. - Add requirements for collision handling by referring to R17 con-MG requirements since the basic collision handling rule for Case 1 is missing. - Clarify that requirements in 9.1.12.4 are only applicable for UE supporting dynamic collision. - Remove the requirements for Scenario 3 in 9.1.12.4. - Add a new clause for UE scheduling requirements. Update the wording of the requirements to define when a slot is interrupted. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements for pre-MG collision handling are incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.12.3, 9.1.12.4, 9.1.12.5 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

#### 9.1.12.3	Collisions involving Pre-MG(s)

For a UE that supports [dynamic collision capability]:

-	Collisions between a Pre-MG and a measurement gap may occur only when the Pre-MG is activated and the conditions for collision defined in clause 9.1.8.3 are satisfied. No collisions can occur between a per-FR Pre-MG and a per-FR measurement gap when they are configured in different FRs.

-	Collisions between two Pre-MGs may occur only when both Pre-MGs are activated and the conditions for collision defined in clause 9.1.8.3 are satisfied. No collisions can occur between per-FR Pre-MGs when they are configured in different FRs.

For a UE that does not support [dynamic collision capability]:

-	Collisions between a Pre-MG and a measurement gap may occur when the conditions for collision defined in clause 9.1.8.3 are satisfied. No collisions can occur between a per-FR Pre-MG and a per-FR measurement gap when they are configured in different FRs.

-	Collisions between two Pre-MGs may occur when the conditions for collision defined in clause 9.1.8.3 are satisfied. No collisions can occur between per-FR Pre-MGs when they are configured in different FRs.

When collisions between a Pre-MG and a measurement gap occurs, or when collisions between two Pre-MGs occurs, the requirements in clause 9.1.8.3 apply. The requirements for [concurrent measurement gaps with Pre-MG] apply provided that the two measurement gaps (at least one of the gaps is Pre-MG) colliding with each other are configured with different priorities.

#### 9.1.12.4	Collision between Pre-MG activation/deactivation and measurement gap

The requirements in this clause apply for UE that supports [dynamic collision capability].

A measurement gap occasion and a Pre-MG activation/deactivation procedure collide when the ending point of the Pre-MG activation/deactivation procedure occurs anywhere within a time period starting 4ms before the starting point of the gap occasion and ending 4ms after the ending point of the gap occasion. The ending point of the Pre-MG activation/deactivation procedure is defined in clause 8.19.5.3.

When a collision occurs between a measurement gap occasion and a Pre-MG activation procedure, and the Pre-MG is configured with higher priority, the UE shall perform measurements during the measurement gap occasion and the activation of the Pre-MG is delayed until 5ms after the ending point of the measurement gap occasion.

When a collision occurs between a measurement gap occasion and a Pre-MG deactivation procedure, and the Pre-MG is configured with higher priority, the measurement gap occasion shall be (or remain) dropped if the measurement gap occasion collides with an occasion of the Pre-MG.

#### 9.1.12.5	Pre-MG related requirements

A slot is interrupted by a Pre-MG if it overlaps with any occasion of the configured pre-MG, if the Pre-MG is activated and the Pre-MG occasion is not dropped.

<End of Change 1>