3GPP TSG-RAN WG4 Meeting #112	R4-2412639

Maastricht, The Netherlands, 19 – 23 August, 2024

Title: 	Discussion on remaining issues in R18 less than 5MHz

Source: 	Huawei, HiSilicon

Agenda item:	5.11.3

Document for:	Discussion

Introduction

RRM core and performance requirements for R18 less than 5MHz are discussed in RAN4#111, and two CRs [1-2] are agreed. In our view, the following issues need to be further discussed in maintenance phase.

- Cell reselection test

- Qin/Qout levels for RLM/BFD test

In this paper, we will provide our views on remaining issues in R18 less than 5MHz.

Discussion

Cell reselection test

In RAN4#110-bis, the following was agreed.

Issue 1-2: For the following different scenarios, should the time to identify a target cell be extended to allow additional SSB samples for SBI and MIB reading due to PBCH puncturing: Issue 1-2-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell (Table 6.2.1.2.1-1)? Issue 1-2-2: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell (Table 6.2.1.2.1-2)? Issue 1-2-3: Time to identify an unknown target NR inter-frequency cell for RRC connection release with re-direction, (Table 6.2.3.2.1-1)? Issue 1-2-4: Time for RRC connection re-establishment shall be extended by 2xTSMTC for the unknown inter-frequency cell and by 3xTSMTC for the unknown intra-frequency cell, based on the target cell side condition of Es/Iot≥-4 dB for inter-frequency target cell and Es/Iot≥-6 dB for intra-frequency target cell? Issue 1-2-5: Time for RRC connection release with re-direction shall be extended by 2xTSMTC for the unknown inter-frequency cell and by 3xTSMTC for the unknown intra-frequency cell, based on the target cell side condition of Es/Iot≥-4 dB for inter-frequency target cell and Es/Iot≥-6 dB for intra-frequency target cell? Issue 1-3: Intra-frequency and inter-frequency cell reselection delays for a target cell with 12 PRB SSB by 2 DRX cycles? Agreement: - For Issue 1-2-1/2/3/4/5 and Issue 1-3, RAN4 to not extend the cell identification delays for 12 PRB SSB but extend TSI-NR in the test-cases. Exact extension is FFS.

The consideration behind the agreement was that in RRC connection re-establishment and RRC connection release with re-direction, there is a term TSI-NR which is the time required for receiving all the relevant system information. It can be used to accommodate the additional time for UE to read PBCH/MIB due to punctured PBCH. The exact value of TSI-NR is not defined as part of core requirements but only in the test case, so RAN4 agreed not to change the core requirements but to extend the TSI-NR in test case.

For cell reselection, the consideration was similar. The core requirements are defined for UE evaluating whether a neighbour cell meets the reselection criteria, and do not involve PBCH/MIB reading. In the test case, UE is supposed to execute the reselection, so it needs to read PBCH/MIB of the target cell, and a term TSI-NR is added in the test requirements. Again, this TSI-NR can be used to accommodate the additional time for UE to read PBCH/MIB, and it needs to be extended as agreed in RAN4#110-bis.

However, in the cell reselection test case, the additional time for PBCH/MIB reading is not added to TSI-NR.

##### A.6.1.1.9.3 Test Requirements The cell reselection delay to a newly detectable cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration update on Cell 2. The cell re-selection delay to a newly detectable cell shall be less than 34 s + 2 measurement samples for UE operating on a cell with less than 5MHz BW. The cell reselection delay to an already detected cell is defined as the time from the beginning of time period T3, to the moment when the UE camps on cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration update on cell 1. The cell re-selection delay to an already detected cell shall be less than 8 s + 2 measurement samples for UE operating on a cell with less than 5MHz BW. The rate of correct cell reselections observed during repeated tests shall be at least 90%. NOTE: The cell re-selection delay to a newly detectable cell can be expressed as: Tdetect, NR_Intra + TSI-NR, and to an already detected cell can be expressed as: Tevaluate, NR_ intra + TSI-NR, Where: Tdetect, NR_Intra + 40ms See Table 4.2.2.3-1 in clause 4.2.2.3 Tevaluate, NR_ intra See Table 4.2.2.3-1 in clause 4.2.2.3 TSI-NR Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280ms is assumed in this test case. This gives a total of 33.32 s, allow 34 s for the cell re-selection delay to a newly detectable cell and 7.72 s for the cell re-selection delay to an already detected cell in the test case, which we allow 8 s.

Although the test requirements are same in absolute allowed time, it will cause confusion that the additional time for PBCH/MIB reading is not included as part of TSI-NR. Following this logic, the core requirements for RRC connection re-establishment and release with re-direction, as well as paging interruption as copied below, where TSI-NR does appear, need to be updated.

#### 4.2.2.6 Maximum interruption in paging reception At intra-frequency and inter-frequency cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels of the target intra-frequency and inter-frequency cell for paging reception. The interruption time shall not exceed TSI-NR + 2*Ttarget_cell_SMTC_period ms. Ttarget_cell_SMTC_period is the periodicity of the SMTC occasions configured for the target NR cell. If the target cell is in the PCI list of smtc2-LP, the SMTC periodicity follows smtc2-LP; otherwise, the SMTC periodicity follows smtc. TSI-NR is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 38.331 [2] for an NR cell.

There are two options to make the core requirements and test cases consistent:

- Option 1: Update the test requirements in A.6.1.1.9.3 to include the additional time for PBCH/MIB reading, i.e. the 2 SMTC samples in TSI-NR.

- Option 2: Update the core requirements for RRC connection re-establishment, release with re-direction, and paging interruption to include the additional time for PBCH/MIB reading, i.e. the 2 SMTC samples

It is clear that option 1 is simpler and aligned with previous agreement. Therefore, we suggest to update the test requirements in A.6.1.1.9.3 to include the additional time for PBCH/MIB reading, i.e. the 2 SMTC samples, in TSI-NR.

Proposal 1: Update the test requirements in A.6.1.1.9.3 to include the additional time for PBCH/MIB reading, i.e. the 2 SMTC samples, in TSI-NR.

Qin/Qout levels for RLM/BFD test

RAN4 has agreed on the PDCCH parameters for RLM and BFD for UE operating in a cell with less than 5MHz. To define the SNR levels RAN4 needs to agree on the Qin/Qout levels from the new PDCCH setup. In Table 1, we provide our simulation results. It is noted that the simulations are done with interleaving size R=2, and the power boosting is not considered.

Table 1: Simulation results for Qin and Qout levels with PDCCH for less than 5MHz operation

|  | SNR (dB) @ 10% BLER | SNR (dB) @ 2% BLER |
| --- | --- | --- |
| OOS parameters with 12 RB | -4.00 | N/A |
| IS parameters with 12 RB | N/A | 2.14 |
| OOS parameters with 15 RB | -6.60 | N/A |
| IS parameters with 15 RB | N/A | -1.82 |

Proposal 2: RAN4 to take simulation results in Table 1 into account for defining SNR levels for RLM/BFD test cases.

Conclusions

In this paper we provided our views on remaining issues in R18 less than 5MHz.

Proposal 1: Update the test requirements in A.6.1.1.9.3 to include the additional time for PBCH/MIB reading, i.e. the 2 SMTC samples, in TSI-NR.

Proposal 2: RAN4 to take simulation results in Table 1 into account for defining SNR levels for RLM/BFD test cases.

Table 1: Simulation results for Qin and Qout levels with PDCCH for less than 5MHz operation

|  | SNR (dB) @ 10% BLER | SNR (dB) @ 2% BLER |
| --- | --- | --- |
| OOS parameters with 12 RB | -4.00 | N/A |
| IS parameters with 12 RB | N/A | 2.14 |
| OOS parameters with 15 RB | -6.60 | N/A |
| IS parameters with 15 RB | N/A | -1.82 |

Reference

- R4-2410380, WF on RRM requirements for NR_NTN_enh, Moderator (Qualcomm Incorporated)

- R4-2410395, BigCR for NR_FR1_lessthan_5MHz_BW, Nokia

- R4-2410431, Big CR for Introducing agreed test cases and common parameters for NR_FR1_lessthan_5MHz_BW, Nokia