3GPP TSG-RAN4 Meeting #112	R4-2412641

# Maastricht, The Netherlands, 19 – 23 August, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 4856 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on TCs for less than 5MHz operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR1_lessthan_5MHz_BW-Perf |  |  |  |  |  | Date: |  |  | 2024-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are some issues in cell reselection TC for less than 5MHz: - For general test parameter, PRACH configuration is not re-used, but there is no new PRACH RMC defined for less than 5MHz. Technically, there is no issue to re-use the existing configuration. - For cell specific test parameters, the RMCs for PDSCH, RMSI CORESET and dedicated CORESET cannot be re-used from existing configuration. Also, the Io levels should be re-calculated based on 2.7MHz BW. - It is agreed in RAN4#110-bis to include the additional time for PBCH/MIB decoding is in TSI-NR. The agreement is not captured correctly in currrent test requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update cell reselection TC for less than 5MHz for above reasons. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cell reselection TC for less than 5MHz are incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.1.1.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

#### A.6.1.1.9	Cell reselection to FR1 intra-frequency NR case for UE operating on a cell with less than 5MHz BW

##### A.6.1.1.9.1	Test Purpose and Environment

This test is to verify the requirement for the intra frequency NR cell reselection requirements specified in clause 4.2.2.3. for UE capable of operating on a cell with less than 5MHz BW.

Supported test configurations are specified in Table A.6.1.1.9.2-1. General test parameters as specified in Table A.6.1.1.1.2-2 with config 1 apply except those specified in Table A.6.1.1.9.2-2. Cell specific test parameters as specified in Table A.6.1.1.1.2-3 with config 1 apply except those specified in Table A.6.1.1.9.2-3. The test procedure specified in A.6.1.1.1.2 applies to this test.

Table A.6.1.1.9.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | FDD duplex mode, 15 kHz SSB SCS, 3 MHz bandwidth |

Table A.6.1.1.9.2-2: General test parameters for intra frequency NR cell re-selection test case

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 |  |
| BWchannel | MHz | Config 1 | 3: NRB,c = 12 |  |
| SSB Configuration |  | Config 1 | SSB.13 |  |

Table A.6.1.1.1.9-3: Cell specific test parameters for intra-frequency NR cell re-selection test case

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| PDSCH RMC |  | 1 | SR.1.2 FDD |  |  | SR.1.2 FDD |  |  |
| RMSI CORESET |  | 1 | CR.1.2 FDD |  |  | CR.1.2 FDD |  |  |
| Dedicated CORESET |  | 1 | CCR.1.6 FDD |  |  | CCR.1.6 FDD |  |  |
| Io | dBm/2.7 MHz | 1 | -60.31 | -58.58 | -58.58 | Same as parameters specified in Cell 1 columns- |  |  |

##### A.6.1.1.9.3	Test Requirements

The cell reselection delay to a newly detectable cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration update on Cell 2.

The cell re-selection delay to a newly detectable cell shall be less than 34 s for UE operating on a cell with less than 5MHz BW.

The cell reselection delay to an already detected cell is defined as the time from the beginning of time period T3, to the moment when the UE camps on cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration update on cell 1.

The cell re-selection delay to an already detected cell shall be less than 8 s for UE operating on a cell with less than 5MHz BW.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a newly detectable cell can be expressed as: Tdetect, NR_Intra + TSI-NR, and to an already detected cell can be expressed as: Tevaluate, NR_ intra + TSI-NR,

Where:

Tdetect, NR_Intra 	+ 40ms See Table 4.2.2.3-1 in clause 4.2.2.3

Tevaluate, NR_ intra See Table 4.2.2.3-1 in clause 4.2.2.3

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1320 ms is assumed in this test case.

This gives a total of 33.36 s, allow 34 s for the cell re-selection delay to a newly detectable cell and 7.76 s for the cell re-selection delay to an already detected cell in the test case, which we allow 8 s.

<End of Change 1>